https://github.com/Embetronicx/Tutorials/tree/master/Linux/Device_Driver

# Device Driver 1 - Introduction

- Linux Architecture Overview

- User Space vs Kernel Space

+-----------------------------+

|         User Space          |

|-----------------------------|

| Applications (C, Python)    |

| Shells (bash, zsh)          |

| Libraries (libc)            |

+-----------------------------+

|

|  System Calls

v

+-----------------------------+

|        Kernel Space         |

|-----------------------------|

| System Call Interface       |

| Kernel Core (Scheduler,     |

| Memory Mgmt, IPC, VFS)      |

| Device Drivers              |

| Loadable Kernel Modules     |

+-----------------------------+

|

v

+-----------------------------+

|          Hardware           |

+-----------------------------+

- Key Points

- User programs can’t directly access hardware → they must go through the kernel.

- Kernel space has full hardware access and must remain stable and secure.

- Device drivers are a key part of kernel space.

kernel is the core part of an operating system that sits between your hardware and your software. The Linux kernel is the specific kernel used by Linux-based systems.

It’s responsible for things like:

- Managing hardware (CPU, memory, disks, USB devices, etc.)

- Running processes (starting, stopping, scheduling programs)

- Handling memory (who gets RAM and when)

- File systems (reading/writing files)

- Security & permissions

- Networking

Custom codes can be added to Linux kernels via two methods.

- The basic way is to add the code to the kernel source tree and recompile the kernel.

- A more efficient way to do this is by adding code to the kernel while it is running. This process is called loading the module, where the module refers to the code that we want to add to the kernel.

What is a Loadable Kernel Module (LKM)?

A piece of code that can be loaded into (or removed from) the Linux kernel at runtime.

- Why LKMs?

- Avoid recompiling the entire kernel

- Add/remove hardware support dynamically

- Make development/testing easier

- Useful for device drivers, filesystems, system call extensions

- How LKMs behave

- They register themselves with the kernel when inserted

- They respond to kernel-level events (I/O requests, interrupts)

- They don’t “run and exit” like normal programs

- What is a Device Driver?

A driver is software that makes hardware usable by providing a standard interface to the kernel.

- Roles of a driver

- Abstract complicated hardware details

- Allow user programs to interact with devices as files

- Translate OS requests into hardware-specific operations

- Why Linux Needs Drivers

- Different hardware → different protocols

- Kernel needs a uniform interface

- Without drivers, hardware cannot be used by Linux

- Device Types in Linux

Linux follows the philosophy: “Everything is a file.”

Devices are represented as special files in /dev/.

- A. Character Devices

- Data processed one byte at a time

- Sequential access

- Cannot be mounted

- Examples:

  - Keyboard

  - Serial ports

  - Sensors

- B. Block Devices

- Data processed in blocks (512B, 4KB, etc.)

- Can be mounted( mounted - devices being attached to the filesystem so their stored data becomes accessible.)

- Efficient for large data

- Examples:

  - Hard disks

  - SSDs

  - USB drives

- C. Network Devices

- Operate with packets

- Examples:

  - Ethernet cards

  - WiFi interfaces

  - Virtual interfaces (loopback)

- 🛠️ 6. Kernel Modules vs User Programs

| Feature | User Program | Kernel Module |
| --- | --- | --- |
| Runs In | User Space | Kernel Space |
| Privilege | Limited | Full |
| Libraries | Uses libc; normal APIs | Uses kernel APIs only |
| Crash Impact | Only app crashes | System can crash! |
| Execution | Linear (start → end) | Event-driven, persistent |
| Compiled As | ELF executable | .ko module |

- Important:

A driver is conceptually the hardware interface code.
A kernel module is the mechanism used to load that driver into the kernel.

A driver can be:

- Built into the kernel, OR

- Loaded dynamically as a module.

- 7. How a Driver Gets Accessed by User Programs

+------------------------+

|      User Program      |

+------------------------+

|

| open(), read(), write()

v

+------------------------+

|  System Call Interface |

+------------------------+

|

v

+------------------------+

|    Device Driver       |

+------------------------+

|

v

+------------------------+

|       Hardware         |

+------------------------+

Programs interact with drivers through file operations (open, read, write, ioctl, etc.).

- 📚 8. Glossary

| Term | Meaning |
| --- | --- |
| Kernel | Core of OS that interacts with hardware |
| User Space | Area where normal applications run |
| Kernel Space | Privileged area where the kernel runs |
| System Call | Interface between user programs and kernel |
| LKM | Loadable Kernel Module — kernel code inserted at runtime |
| Device File | Special file in /dev representing a device |
| Character Device | One-byte-at-a-time device |
| Block Device | Block-based device used for storage |
| Network Device | Device that transfers packets |
| Driver | Software that controls hardware |
| VFS | Virtual File System layer that abstracts different filesystem types |
| IRQ | Hardware interrupt signal |

- 📝 9. Clean, Exam-Style Notes

- Linux Device Driver – Part 1 Notes

- Linux divided into user space and kernel space

- Device drivers run in kernel space

- Linux uses loadable kernel modules (LKMs) to extend kernel functionality at runtime

- Drivers abstract hardware details and expose devices as files

- Devices fall into three categories:

  - Character devices (byte-by-byte)

  - Block devices (block-based storage)

  - Network devices (packet-based)

- Modules differ from user programs in privilege, execution model, and libraries

- Kernel must remain stable; buggy drivers can crash the entire OS

- Drivers typically register operations (open/read/write) with the kernel

- Understanding kernel architecture is essential for writing drivers

# Device Driver 2 - First Driver

This tutorial teaches you how to write a very simple Linux kernel device driver (a kernel module) from scratch. It introduces the basic structure, core macros, init/exit functions, compilation, and loading/unloading of the first Linux device driver. Embetronicx

- Prerequisites

- A Linux development environment such as Ubuntu, Raspberry Pi, or Beaglebone must be set up before starting.

Module Information Macros

Linux device drivers require metadata declarations using macros from linux/module.h :

- MODULE_LICENSE(): Specifies licensing (e.g., "GPL", "GPL v2", "Dual BSD/GPL") - helps users verify free software and allows community/vendor bug report policies

- MODULE_AUTHOR(): Documents the author's name/email (format: "Name <email>")

- MODULE_DESCRIPTION(): Provides a brief description of the driver functionality

- MODULE_VERSION(): Specifies version in format [<epoch>:]<version>[-<extra-version>]

These help tools like modinfo show information about the module.

Core Driver Structure

Every kernel module requires two essential functions :

Init Function: Executes when driver is loaded (insmod)

static int __init hello_world_init(void) {

return 0;

}

module_init(hello_world_init);

Exit Function: Executes when driver is unloaded (rmmod)

static void __exit hello_world_exit(void) {

}

module_exit(hello_world_exit);

Kernel Logging with printk()

Unlike user-space printf(), kernel modules use printk() which supports log level classification :

| Log Level | Purpose |
| --- | --- |
| KERN_EMERG | Emergency messages before crash |
| KERN_ALERT | Requires immediate action |
| KERN_CRIT | Critical hardware/software failures |
| KERN_ERR | Error conditions and hardware difficulties |
| KERN_WARNING | Problematic but non-critical situations |
| KERN_INFO | Informational messages |
| KERN_DEBUG | Debugging output |

- Printk() is a kernel-level function, which has the ability to print out to different log levels as defined in. We can see the prints using dmesg command.

- printf() will always print to a file descriptor – STD_OUT. We can see the prints in the STD_OUT console.

Note: In the newer Linux kernels, you can use the APIs below instead of this printk.

- pr_info – Print an info-level message. (ex. pr_info("test info message\n")).

- pr_cont – Continues a previous log message in the same line.

- pr_debug – Print a debug-level message conditionally.

- pr_err – Print an error-level message. (ex. pr_err(“test error message\n”)).

- pr_warn – Print a warning-level message.

Compilation and Makefile

Basic Makefile structure for compiling kernel modules :

makefile

obj-m += hello_world.o

KDIR = /lib/modules/$(shell uname -r)/build

all:

make -C $(KDIR) M=$(shell pwd) modules

clean:

make -C $(KDIR) M=$(shell pwd) clean

Compile with sudo make to generate the .ko (kernel object) file .

Loading and Managing Drivers

- Load module: sudo insmod hello_world_module.ko

- List loaded modules: lsmod

- Unload module: sudo rmmod hello_world_module

- View module info: modinfo hello_world_module.ko

- View kernel log: dmesg

/*

****************************************************************************/

#include<linux/kernel.h>

#include<linux/init.h>

#include<linux/module.h>

/*Module Init function*/

static int __init hello_world_init(void)

{

printk(KERN_INFO "Welcome to EmbeTronicX\n");

printk(KERN_INFO "This is the Simple Module\n");

printk(KERN_INFO "Kernel Module Inserted Successfully...\n");

return 0;

}

/* Module Exit function*/

static void __exit hello_world_exit(void)

{

printk(KERN_INFO "Kernel Module Removed Successfully...\n");

}

module_init(hello_world_init);

module_exit(hello_world_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("EmbeTronicX <embetronicx@gmail.com>");

MODULE_DESCRIPTION("A simple hello world driver");

MODULE_VERSION("2:1.0");

# Device Driver 3 - Passing Arguments

- Overview

Module parameters allow passing arguments to kernel modules during loading time, similar to command-line arguments in C programs .

- Just like programs accept command-line arguments (argc, argv), Linux kernel modules can accept parameters when loaded.

- This allows customizing module behavior without changing code

- Three Module Parameter Macros

| Macro | Purpose | Syntax |
| --- | --- | --- |
| module_param() | Single value | module_param(name, type, perm); |
| module_param_array() | Array of values | module_param_array(name, type, num, perm); |
| module_param_cb() | Value + callback on change | module_param_cb(name, &ops, var, perm); |

- 1. module_param()

Purpose: Initialize single parameters

Syntax: module_param(name, type, perm);

Creates sysfs entry: /sys/module/<module_name>/parameters/<param_name>

Supported types:

- int, long, short, uint, ulong, ushort - Integer types

- bool - Boolean (0 or 1)

- invbool - Inverted boolean

- charp - Character pointer (string)

Example:

int valueETX;

char *nameETX;

module_param(valueETX, int, S_IWUSR|S_IRUSR);

module_param(nameETX, charp, S_IWUSR|S_IRUSR);

- 2. module_param_array()

Purpose: Pass arrays as arguments

Syntax:  module_param_array(name, type, num, perm);

Parameters:

- name - Array name

- type - Element type

- num - Integer variable to store count (optional, use NULL)

- perm - Permissions

Example:

int arr_valueETX[4];

module_param_array(arr_valueETX, int, NULL, S_IRUSR|S_IWUSR);

Load with: insmod module.ko arr_valueETX=100,102,104,106

- 3. module_param_cb()

Purpose: Register callback function triggered when parameter value changes via sysfs

Why needed: Normal parameters can be changed via sysfs, but driver won't be notified . Useful for writing to hardware registers when value changes .

module_param_cb() = module parameter + custom read/write behavior

Setup:

int notify_param(const char *val, const struct kernel_param *kp) {

int res = param_set_int(val, kp);

if(res==0) {

printk("Callback called, new value = %d\n", cb_valueETX);

return 0;

}

return -1;

}

const struct kernel_param_ops my_param_ops = {

.set = &notify_param,       // Custom setter

.get = &param_get_int,      // Standard getter

};

int cb_valueETX = 0;

module_param_cb(cb_valueETX, &my_param_ops, &cb_valueETX, S_IRUGO|S_IWUSR);

How it works (Concept)

When user runs:

echo 10 > /sys/module/module_name/parameters/my_param

👉 Kernel calls:

set_function("10", ...)

When user runs:

cat /sys/module/module_name/parameters/my_param

👉 Kernel calls:

get_function(buffer, ...)

- Permission Flags

Format: S_I[R|W|X][USR|GRP]

- R = Read, W = Write, X = Execute

- USR = User, GRP = Group

- Combine with OR: S_IWUSR|S_IRUSR

- Loading Driver with Arguments

bash

sudo insmod hello_world_module.ko valueETX=14 nameETX="EmbeTronicX" arr_valueETX=100,102,104,106

- Changing Parameters at Runtime

For regular parameters:

bash

sudo sh -c "echo 13 > /sys/module/hello_world_module/parameters/cb_valueETX"

For callback parameters: Same method, but callback function executes and prints notification

For normal module_param() variables also we can change that variable after loading based on the permission, if the permission is read-only then we cannot able to change it after loading the module.

- Key Takeaways

- Module parameters enable runtime configuration without recompilation

- Sysfs entries provide interface to modify parameters dynamically

- Callback mechanism allows immediate response to parameter changes for hardware control

- Essential for flexible driver configuration in embedded Linux systems

# Device Driver 4 - Major & Minor Number

- What Are They?

- Linux represents character and block devices using a pair of numbers: major number and minor number.

- These numbers together uniquely identify a device driver within the system.

Major Number

- Identifies the device driver

- The kernel looks at the major number of a device file to decide which driver to call.

- You can see assigned major numbers in the file:

- cat /proc/devices

which lists all character and block device major numbers.

Minor Number

- Identifies a specific device or sub-device handled by that driver

- Multiple devices using the same driver share the major number but have different minor numbers.

dev_t Data Type: Holds both major and minor numbers in a single variable.

Example:

- A driver managing multiple UART ports might use:

  - Major number: driver ID

  - Minor numbers: 0, 1, 2… for each port instance

Allocating Device Numbers

- 1. Static Allocation

- You pick a specific major number for your driver.

- register_chrdev_region(dev_t first, unsigned int count, char *name);

  - first — first device number (contains major and minor)

  - count — Number of consecutive device numbers needed

  - name — driver name shown in system lists (appears in /proc/devices)

- If range is unavailable, registration fails.

Example:

dev_t dev = MKDEV(235, 0);  // Major=235, Minor=0

register_chrdev_region(dev, 1, "Embetronicx_Dev");

Helpful macros:

- MKDEV(major, minor) — create a device number

- MAJOR(dev_t) — extract major number

- MINOR(dev_t) — extract minor number

- 2. Dynamic Allocation

- Kernel assigns a free major number automatically.

- alloc_chrdev_region(dev_t *dev, unsigned int firstminor, unsigned int count, char *name);

- Parameters:

- dev - Output parameter that receives allocated device number

- firstminor - Starting minor number (usually 0)

- count - Number of consecutive minor numbers needed

- name - Device name

Returns: 0 on success, negative error code on failure

Example – c:

dev_t dev;

alloc_chrdev_region(&dev, 0, 1, "Embetronicx_Dev");

// Extract allocated numbers

int major = MAJOR(dev);

int minor = MINOR(dev);

When to Use

- Preferred method for most drivers​

- Avoids conflicts with other drivers​

- Kernel automatically finds available major number

Pros vs Cons:

| Method | Pros | Cons |
| --- | --- | --- |
| Static | Known number ahead of time | Conflict risk |
| Dynamic | Avoids conflicts | Device number changes each load |

- Unregistering Numbers

- Always free allocated device numbers when the driver unloads:

- unregister_chrdev_region(dev_t first, unsigned int count);

- Commonly done in the module cleanup function.

- Sample Code Overview

- Static Major Allocation

dev_t dev = MKDEV(235, 0);

static int __init hello_world_init(void) {

register_chrdev_region(dev, 1, "Embetronicx_Dev");

printk("Major = %d Minor = %d \n", MAJOR(dev), MINOR(dev));

return 0;

}

static void __exit hello_world_exit(void) {

unregister_chrdev_region(dev, 1);

}

✔ Shows setting a fixed major number (e.g., 235).
✔ Prints assigned major/minor values.

- 📌 Dynamic Major Allocation

dev_t dev = 0;

static int __init hello_world_init(void) {

alloc_chrdev_region(&dev, 0, 1, "Embetronicx_Dev");

printk("Major = %d Minor = %d \n", MAJOR(dev), MINOR(dev));

return 0;

}

static void __exit hello_world_exit(void) {

unregister_chrdev_region(dev, 1);

}

✔ Lets kernel pick a free major number automatically.

- Extra Notes

- Device files exist physically under /dev but the driver logic looks up devices by number, not name.

- Linux traditionally uses <major>:<minor> displayed by listing device files (e.g., ls -l /dev).

## Older / legacy approach - register_chrdev()

This is an older, higher-level API that both allocates/registers a major number and associates file operations with it.

Prototype:

int register_chrdev(unsigned int major,

const char *name,

const struct file_operations *fops);

Example:

Major = register_chrdev(240, "mydriver", &fops);

or dynamically allocate a major:

int major;

major = register_chrdev(0, "mydriver", &fops);

If major is 0, the kernel chooses a free major number and returns it.

Remove it in __exit using:

unregister_chrdev(major, “mydriver”);

- Summary Points

- Major number: tells kernel which driver to use.

- Minor number: tells driver which specific hardware instance.

- Two allocation ways:

  - Static – fixed number you choose.

  - Dynamic – kernel assigns automatically.

- Always unregister numbers on module exit.

Refer for code: https://embetronicx.com/tutorials/linux/device-drivers/character-device-driver-major-number-and-minor-number/

# Device Driver 5 - Creating Device File

## What are device files in Linux?

In Linux, device files are special files that represent hardware devices (or virtual devices). They live mostly in the /dev directory.

Linux follows a big philosophy:

“Everything is a file.”

So instead of talking to hardware directly, programs read from and write to files that act as gateways to devices.

## Where they live /dev

Examples:

- /dev/sda → a disk

- /dev/tty → a terminal

- /dev/null → a “black hole” device

- /dev/random → random number generator

We can create a device file in two ways.

- Manually

- Automatically

### Manual creation

mknod -m <permissions> <name> <device type> <major> <minor>

eg: sudo mknod -m 666 /dev/etx_device c 246 0

### Automatically Creating Device File

The automatic creation of device files can be handled with udev. Udev is the device manager for the Linux kernel that creates/removes device nodes in the /dev directory dynamically. Just follow the below steps.

  - Include the header file linux/device.h and linux/kdev_t.h

  - Create the struct Class

  - Create Device with the class which is created by the above step

#### Create the class

struct class * class_create(struct module *owner, const char *name);

owner – pointer to the module that is to “own” this struct class

name – pointer to a string for the name of this class

void class_destroy (struct class * cls);

#### Create Device

struct device *device_create(struct *class, struct device *parent, dev_t dev, void * drvdata, const char *fmt, ...);

class – pointer to the struct class that this device should be registered to

parent – pointer to the parent struct device of this new device, if any

devt– the dev_t for the char device to be added

drvdata – the data to be added to the device for callbacks

fmt – string for the device’s name

... – variable arguments

# Device Driver 6 - File Operations

# cdev Structure & File Operations (Linux Driver Tutorial Part 6)

## Why This Is Needed

To support operations like open, read, write, and close on a device file, the driver must register two things: a cdev structure and a file_operations structure.

## 1. inode vs file

- struct inode — kernel's internal representation of a file (one per file on disk). Contains struct cdev inside it.

- struct file — represents an open file descriptor (many can exist for one inode).

## 2. struct cdev — Character Device Structure

This is the kernel's internal structure that represents a character device.

struct cdev {

struct kobject kobj;

struct module *owner;           // set to THIS_MODULE

const struct file_operations *ops;  // pointer to file operations

struct list_head list;

dev_t dev;

unsigned int count;

};

You only need to fill two fields: ops and owner.

### Two ways to allocate cdev:

Method 1 — Runtime allocation (standalone):

struct cdev *my_cdev = cdev_alloc();

my_cdev->ops = &my_fops;

Method 2 — Embed inside your own structure:

Syntax:

void cdev_init(struct cdev *cdev, struct file_operations *fops);

int cdev_add(struct cdev *dev, dev_t num, unsigned int count);

cdev_init(&etx_cdev, &fops);

### Registering and Removing cdev:

// Add cdev to the system (device becomes alive after this)

cdev_add(&etx_cdev, dev, 1);

// Remove cdev from the system, Syntax: void cdev_del(struct cdev *dev);

cdev_del(&etx_cdev);

cdev_add() arguments: cdev pointer, starting device number, count of device numbers. Always check the return value — negative means failure.

## 3. struct file_operations — fops

Defined in <linux/fs.h>. It's a collection of function pointers that connect system calls to your driver functions. Conventionally named fops.

Unsupported operations are left as NULL.

### Key fields you use most:

| Field | Purpose | Return on NULL |
| --- | --- | --- |
| owner | Set to THIS_MODULE — prevents unload while in use | — |
| read | Called when user reads from device | -EINVAL |
| write | Called when user writes to device | -EINVAL |
| open | Called when device file is opened | succeeds silently |
| release | Called when device file is closed | — |
| unlocked_ioctl | Device-specific commands | -ENOTTY |

### Defining fops:

static struct file_operations fops = {

.owner   = THIS_MODULE,

.read    = etx_read,

.write   = etx_write,

.open    = etx_open,

.release = etx_release,

};

## 4. Function Signatures

int     etx_open   (struct inode *inode, struct file *file);

int     etx_release(struct inode *inode, struct file *file);

ssize_t etx_read   (struct file *filp, char __user *buf, size_t len, loff_t *off);

ssize_t etx_write  (struct file *filp, const char __user *buf, size_t len, loff_t *off);

- __user — indicates the buffer is in user space (not kernel space)

- read/write return the number of bytes transferred, or negative on error

## 5. Complete Init Flow (with cdev added)

alloc_chrdev_region()   → get Major:Minor

cdev_init()             → link fops to cdev

cdev_add()              → register with kernel (device goes live)

class_create()          → create /sys/class/etx_class/

device_create()         → udev creates /dev/etx_device

Cleanup (reverse order):

device_destroy()

class_destroy()

cdev_del()

unregister_chrdev_region()

## 6. Testing the Driver

# Load driver

sudo insmod driver.ko

# Write to device (triggers open → write → release)

echo 1 > /dev/etx_device (or) sudo echo 1 | sudo tee /dev/etx_device

# Read from device (triggers open → read → release)

cat /dev/etx_device

# Check kernel logs

dmesg

# Unload driver

sudo rmmod driver

## Quick Summary

cdev_init()  →  links your fops to the cdev structure

cdev_add()   →  registers cdev with the kernel (device is now alive)

cdev_del()   →  removes cdev on cleanup

file_operations  →  table of your driver's function pointers

(open, read, write, release, ioctl...)

Every file operation your driver supports needs a function implemented and pointed to in file_operations. Unsupported ones are just left as NULL.

https://github.com/Embetronicx/Tutorials/tree/master/Linux/Device_Driver/File_Operations

# Device Driver 7 - Real Device Driver

Previous tutorials used dummy drivers — open/read/write just printed logs, nothing actually happened. This tutorial builds a real driver that:

- Allocates kernel memory (kmalloc)

- Accepts data written from userspace and stores it in kernel memory

- Sends that stored data back when userspace reads it

- Comes with a userspace C application to test it

## Concept — Why You Can't Share Memory Directly

Kernel space and user space have separate, protected memory address spaces. You cannot directly access a user pointer in kernel code or vice versa.

User Space App          Kernel / Driver

─────────────           ───────────────

char buf[]      <──────  copy_to_user()    (kernel → user)

char buf[]      ──────>  copy_from_user()  (user → kernel)

You must always use these two functions to safely transfer data across the boundary.

### 1. kmalloc() — function is used to allocate the memory in kernel space. This is like a malloc() function in userspace.

#include <linux/slab.h>

void *kmalloc(size_t size, gfp_t flags);

- size– how many bytes of memory are required.

- flags– the type of memory to allocate.(GFP_USER, GFP_KERNEL, GFP_ATOMIC etc)

### 2. kfree() — This is like a free() function in the userspace. This is used to free the previously allocated memory

void kfree(const void *objp)

- *objp – pointer returned by kmalloc

### 3. copy_to_user() — Kernel -> User space

unsigned long copy_to_user(void __user *to, const void *from, unsigned long n);

to – Destination address, in the user space

from – The source address in the kernel space

n – Number of bytes to copy

- Copies n bytes FROM kernel buffer TO user buffer

- Returns 0 on success, non-zero = number of bytes that failed to copy

### 4. copy_from_user() — User → Kernel space

unsigned long copy_from_user(void *to, const void __user *from, unsigned long n);

to – Destination address, in the kernel space

from – The source address in the user space

n – Number of bytes to copy

- Copies n bytes FROM user buffer TO kernel buffer

- Returns 0 on success, non-zero = number of bytes that failed to copy

## Two New Headers Needed

#include <linux/slab.h>      // for kmalloc(), kfree()

#include <linux/uaccess.h>   // for copy_to_user(), copy_from_user()

## Kernel Memory — kmalloc / kfree

#define mem_size 1024          // buffer size = 1024 bytes

uint8_t *kernel_buffer;        // pointer to kernel buffer

| Function | Purpose |
| --- | --- |
| kmalloc(size, GFP_KERNEL) | Allocate size bytes in kernel memory |
| kfree(ptr) | Free the allocated kernel memory |

GFP_KERNEL — normal allocation flag, can sleep if needed (safe to use in open/init).

## How Each Function Is Implemented

### etx_open() — Allocate kernel buffer when device is opened

static int etx_open(struct inode *inode, struct file *file)

{

kernel_buffer = kmalloc(mem_size, GFP_KERNEL);

pr_info("Device File Opened...!!!\n");

return 0;

}

Memory is created fresh every time the device is opened.

### etx_release() — Free kernel buffer when device is closed

static int etx_release(struct inode *inode, struct file *file)

{

kfree(kernel_buffer);

pr_info("Device File Closed...!!!\n");

return 0;

}

Always free what you allocated — prevents memory leaks.

### etx_write() — Receive data from userspace into kernel buffer

static ssize_t etx_write(struct file *filp, const char __user *buf, size_t len, loff_t *off)

{

copy_from_user(kernel_buffer, buf, len);

pr_info("Data Write : Done!\n");

return len;

}

Userspace writes → copy_from_user() safely copies it into kernel_buffer.

### etx_read() — Send data from kernel buffer back to userspace

static ssize_t etx_read(struct file *filp, char __user *buf, size_t len, loff_t *off)

{

copy_to_user(buf, kernel_buffer, mem_size);

pr_info("Data Read : Done!\n");

return mem_size;

}

Userspace reads → copy_to_user() safely copies kernel_buffer back to user.

## The Userspace Application

A separate C program (userapp.c) that talks to the driver via the device file:

int fd = open("/dev/etx_device", O_RDWR);   // open device

write(fd, write_buf, strlen(write_buf));    // send data to driver

read(fd, read_buf, mem_size);               // get data back from driver

close(fd);          // close device → kfree() called

Test flow:

- User types a string → write() → copy_from_user() → stored in kernel_buffer

- User requests read → read() → copy_to_user() → gets the same string back

Example output:

Enter the string to write: embetronicx

Data written to driver ✓

Data read from driver: embetronicx ✓

## Important Note — echo/cat vs Userspace App

|  | echo / cat | Userspace App |
| --- | --- | --- |
| open() | called | called |
| close() | called immediately after | called only when app is done |
| Memory lifetime | buffer created & destroyed per command | buffer lives for the whole session |

If you use echo and cat, allocate kernel_buffer in init instead of open() — because echo opens, writes, closes (buffer freed), then cat opens a new buffer and tries to read from it (which is empty). Using init-time allocation avoids this issue.

## Complete Flow Summary

insmod driver.ko

│

▼

open("/dev/etx_device")  ->  etx_open()   -> kmalloc(1024)

│

▼

write(fd, "hello", 5)    ->  etx_write()  -> copy_from_user() -> kernel_buffer = "hello"

│

▼

read(fd, buf, 1024)      ->  etx_read()   -> copy_to_user()   -> user gets "hello"

│

▼

close(fd)                ->  etx_release() -> kfree(kernel_buffer)

## Quick Reference

| API | Direction | Header |
| --- | --- | --- |
| copy_to_user() | Kernel -> User | <linux/uaccess.h> |
| copy_from_user() | User -> Kernel | <linux/uaccess.h> |
| kmalloc() | Allocate kernel memory | <linux/slab.h> |
| kfree() | Free kernel memory | <linux/slab.h> |

| Function | Purpose |
| --- | --- |
| put_user() | Copy one value to user space |
| get_user() | Read one value from user space |
| copy_to_user() | Copy buffer/block to user space |
| copy_from_user() | Copy buffer/block from user space |

Rule: Never dereference a __user pointer directly in kernel code. Always use copy_from_user() / copy_to_user(). Doing otherwise can cause kernel crashes or security vulnerabilities.

# Device Driver 8 - IOCTL Tutorial

- There are many ways to Communicate between the Userspace and Kernel Space, they are:

- IOCTL

- Procfs

- Sysfs

- Configfs

- Debugfs

- Sysctl

- UDP Sockets

- Netlink Sockets

- In this tutorial, we are going to focus on IOCTL.

- What is IOCTL?

- IOCTL stands for Input/Output Control. A special system call used for device-specific operations that cannot be done using regular read() or write() calls.

- Common uses: adjusting device settings, controlling hardware features (e.g., ejecting CD, changing baud rate, Adjustimg volume, Reading/writing hardware registers).

- It lets a program send control commands directly to a device driver.

💡 IOCTL is sent from user space to kernel space via the ioctl() system call. (Wikipedia)

- Why Use IOCTL?

- Standard read()/write() manage data transfer.

- IOCTL handles device control commands that regular APIs don’t support. (embetronicx.com)

Alternative communication methods include:

- procfs, sysfs, debugfs, netlink sockets, etc. (embetronicx.com)

Steps involved in IOCTL

- Define IOCTL Command in the driver

- Write the IOCTL function in the driver

- Create IOCTL command in a Userspace application

- Use the IOCTL system call in a Userspace

- 🛠️ Steps to Implement IOCTL

1. Define IOCTL Command in the driver - Use macros to define commands:

#include <linux/ioctl.h>

#define "ioctl name" _IOX("magic number","command number","argument type")

- where _IOX can be:

  - _IO – no argument

  - _IOR – read from driver

  - _IOW – write to driver

  - _IOWR – read/write (docs.kernel.org)

- The Magic Number is a unique number or character that will differentiate our set of ioctl calls from the other ioctl calls. some times the major number for the device is used here.

- Command Number is the number that is assigned to the commands in ioctl. This is used to differentiate the commands from one another.  (e.g., 'a', 'b')

- The last is the Data Type — type of data being passed (e.g., int32_t*)

Example:

#include <linux/ioctl.h>

#define WR_VALUE _IOW('a', 'a', int32_t*)

#define RD_VALUE _IOR('a', 'b', int32_t*)

‘a’ is a magic number that identifies this device’s IOCTLs.

2. Write the IOCTL function in the driver
Add an ioctl function in the driver to handle commands:

Syntax:   int  ioctl(struct inode *inode, struct file *file, unsigned int cmd, unsigned long arg)

Old vs New ioctl prototypes

Old (deprecated) - > int ioctl(struct inode *inode, struct file *file, unsigned int cmd, unsigned long arg);

New (current) -> long unlocked_ioctl(struct file *file, unsigned int cmd, unsigned long arg);

- inode parameter is “missing” because modern Linux kernels no longer pass it to ioctl handlers.

Example -> static long etx_ioctl(struct file *file, unsigned int cmd, unsigned long arg)

Inside the function:

- Use switch(cmd) to handle each command.

- Use copy_from_user() and copy_to_user() to transfer data to/from user space. (embetronicx.com)

Connect it to the file operations:

.unlocked_ioctl = etx_ioctl,

unlocked_ioctl is the modern replacement for the old ioctl. (linux-kernel-labs.github.io)

3. Define same IOCTL command in User Space (Create IOCTL command in a Userspace application)
The same macros must be included in the user application so it uses the same definitions.

Copy the exact same #define lines from the driver into your user app. Both must match — same magic, same command number, same type.

#define WR_VALUE  _IOW('a', 'a', int32_t*)

#define RD_VALUE  _IOR('a', 'b', int32_t*)

Just define the ioctl command like how we defined it in the driver. (embetronicx.com)

4. Call IOCTL from user Application (from userspace)
Use the standard system call:

Syntax:

#include <sys/ioctl.h>

long ioctl( "file descriptor","ioctl command","Arguments");

ioctl(fd, WR_VALUE, (int32_t*)&number);

ioctl(fd, RD_VALUE, (int32_t*)&value);

Where:

- fd = file descriptor of the device

- WR_VALUE or RD_VALUE = ioctl command which is implemented to achieve the desired functionality

- argument = pointer to data to send or receive (embetronicx.com)

## Passing Multiple Arguments

IOCTL takes only one arg. To pass multiple values — pack them in a struct and pass the struct's address:

struct my_data { int x; int y; int z; };

struct my_data d = {1, 2, 3};

ioctl(fd, MY_CMD, (struct my_data*) &d);

Build & Test Commands

sudo make                    		# build driver

gcc -o test_app test_app.c   	# build user app

sudo insmod driver.ko     		# load driver

sudo ./test_app              		# run app

sudo dmesg                   		# check kernel logs

sudo rmmod driver            	# unload driver

Best Practices

✔ Use proper macros (_IO, _IOR, _IOW, _IOWR). (docs.kernel.org)
✔ Always validate and safely copy data between spaces. (kernel.org)
✔ Keep matching definitions in both driver and user space. (embetronicx.com)

# Device Driver 9 - Procfs Tutorial

# What Is Procfs?

- Procfs (Process Filesystem) is a virtual filesystem mounted at /proc. It does not exist on disk, it lives entirely in RAM/memory. The kernel populates it dynamically.

- It acts as a bridge between user space and kernel space — you can read kernel info or write/control kernel behaviour just using standard file operations like cat and echo. There by enabling user programs to read kernel and process information.

- 2. Why It’s Useful

- Allows userspace tools to obtain system information without special syscalls.

- Common /proc Entries (Already Exist in Linux)

| Entry | What It Shows |
| --- | --- |
| /proc/meminfo | RAM usage details |
| /proc/modules | Loaded kernel modules (same as lsmod) |
| /proc/devices | Registered char/block major numbers |
| /proc/interrupts | Registered IRQ numbers |
| /proc/cpuinfo | CPU details |
| /proc/kallsyms | All kernel symbols |
| /proc/partitions | Connected block devices |
| /proc/filesystems | Active filesystem drivers |

- Can also be used to modify kernel parameters at runtime (e.g., via writable attributes under /proc/sys). (embetronicx.com)

- The proc entry can also be used to pass data to the kernel by writing into the kernel,

Two Types of Proc Entries

- Read-only — expose kernel data to user space (cat /proc/something)

- Read + Write — user can also push data into kernel (echo "val" > /proc/something)

Example of writing to existing proc: echo 1 > /proc/sys/net/ipv4/ip_forward   # enables IP forwarding

- 3. How Procfs Works in a Device Driver (High-Level)

Procfs entries can be created within a kernel driver so that the driver exports custom data to userspace.

Key steps:

- a. Create a Procfs Directory

struct proc_dir_entry *proc_mkdir(const char *name, struct proc_dir_entry *parent);

- name: directory name under /proc

- parent: pointer to parent entry (NULL for root) (embetronicx.com)

- b. Create a Procfs File Entry

Kernel >= v3.10 and <= v5.5:

struct proc_dir_entry *proc_create ( const char *name, umode_t mode,

struct proc_dir_entry *parent,

const struct file_operations *proc_fops )

Kernel >= v5.6 (API CHANGED):

proc_create(const char *name, umode_t mode,

struct proc_dir_entry *parent,

const struct proc_ops *proc_ops);  //proc_ops instead of file_operations

Example: proc_create("etx_proc", 0666, NULL, &proc_fops);

- Creates a proc file named etx_proc under /proc

- Permissions: 0666 (read/write)

- The ops structure defines read/write callbacks. (embetronicx.com)

Note: In Linux 5.6+, API changed — use proc_ops instead of file_operations. (embetronicx.com)

### Proc File Operations Structure

Kernel v3.10 to v5.5 — uses struct file_operations:

static struct file_operations proc_fops = {

.open    = open_proc,

.read    = read_proc,

.write   = write_proc,

.release = release_proc

};

Kernel v5.6+ — uses struct proc_ops (field names changed too):

The driver must provide functions for:

static struct proc_ops proc_fops = {

.proc_open = open_proc,

.proc_read = read_proc,

.proc_write = write_proc,

.proc_release = release_proc

};

- . proc_open – called when proc file is opened

- . proc_read – output data to userspace

- . proc_write – receive data from userspace

- . proc_release – cleanup on close (embetronicx.com)

Simplified functions look like:

Open and Release function
	static int open_proc(struct inode *inode, struct file *file)

{

printk(KERN_INFO "proc file opend.....\t");

return 0;

}

static int release_proc(struct inode *inode, struct file *file)

{

printk(KERN_INFO "proc file released.....\n");

return 0;

}

Write Function

static ssize_t write_proc(struct file *filp, const char *buff, size_t len, loff_t * off)

{

printk(KERN_INFO "proc file write.....\t");

copy_from_user(etx_array,buff,len);

return len;

}

Read Function

static ssize_t read_proc(struct file *filp, char __user *buffer, size_t length,loff_t * offset)

{

printk(KERN_INFO "proc file read.....\n");

if(len)

len=0;

else{

len=1;

return 0;

}

copy_to_user(buffer,etx_array,20);

return length;;

}

These let userspace write/read /from the proc file. (embetronicx.com)

### Proc Functions

| Function | When Called | Data Direction |
| --- | --- | --- |
| open_proc | On cat or echo (optional) | — |
| read_proc | On cat /proc/etx/etx_proc | Kernel → User (copy_to_user) |
| write_proc | On echo "data" > /proc/etx/etx_proc | User → Kernel (copy_from_user) |
| release_proc | When file is closed (optional) | — |

- d. Remove the Proc Entry

Proc entry should be removed in the Driver exit function using the below function

#### Remove a Single Proc Entry
void remove_proc_entry(const char *name, struct proc_dir_entry *parent);

Example: remove_proc_entry("etx_proc",NULL););

And you can remove the complete parent directory using (Remove Entire Proc Directory (all entries inside too))
proc_remove(struct proc_dir_entry *parent); 	 // cleanest way — removes dir + all children

- 4. Build and test commands

- sudo make                                    # build

- sudo insmod driver.ko                        # load

- ls /proc/                                    # see 'etx' folder

- cat /proc/etx/etx_proc                       # read → "try_proc_array"

- echo "device driver proc" > /proc/etx/etx_proc  # write

- cat /proc/etx/etx_proc                       # read again → "device driver proc"

- sudo dmesg                                   # view kernel logs

- sudo rmmod driver                            # unload

- Users can then do:

  - cat /proc/etx/etx_proc — read string from the driver

  - echo "text" > /proc/etx/etx_proc — send string to the driver (embetronicx.com)

- 5. Typical Use Cases

✔ Debugging kernel modules by exposing internal variables
✔ Exporting live system or driver statistics
✔ Passing simple control parameters from userspace into driver logic (embetronicx.com)

- Quick Concept Recap

| Concept | What It Means |
| --- | --- |
| Virtual filesystem | Only in RAM, not stored on disk |
| Bridge kernel ↔ user space | Allows interaction with kernel data safely |
| Proc file entries | Virtual files representing live system state |
| Driver exports | Kernel modules can expose custom data/control |

# Device Driver 10 - Waitqueue Tutorial

### What is a Waitqueue?

A waitqueue is a kernel mechanism that allows a process/thread to sleep (stop executing) until a specific condition becomes true or an event occurs. When sleeping, the CPU is freed for other tasks.

Required header: #include <linux/wait.h>

Use case example: A driver may sleep until there is data to read or an interrupt signals completion.

⚠️ Old functions sleep_on() and interruptible_sleep_on() are deprecated — they cause race conditions. Always use the wait_event_* macros instead.

- 3 Main Steps in Using Waitqueues

- Initialize Waitqueue (static method or dynamic method)

- Put the process to sleep until the event comes (Queuing Macros)

- Wake up the process

- 1) Initializing Waitqueue

You must include the header:   #include <linux/wait.h>

There are two ways to initialize a waitqueue:

Static Method (at compile time):
DECLARE_WAIT_QUEUE_HEAD(wq);

Dynamic Method (at runtime, inside a function):
wait_queue_head_t wq;
init_waitqueue_head(&wq);

Both methods work identically after initialization.

2) Putting a Process to Sleep

Linux provides several macros that put a process to sleep until a condition becomes true:

| Macro | Sleep Type | Woken By | Returns |
| --- | --- | --- | --- |
| wait_event(wq, condition) | TASK_UNINTERRUPTIBLE | condition only | nothing |
| wait_event_timeout(wq, condition, timeout) | TASK_UNINTERRUPTIBLE | condition or timeout | 0=timeout expired, 1=condition true after timeout, remaining jiffies=condition true before timeout |
| wait_event_cmd(wq, condition, cmd1, cmd2) | TASK_UNINTERRUPTIBLE | condition only | nothing — runs cmd1 before sleep, cmd2 after |
| wait_event_interruptible(wq, condition) | TASK_INTERRUPTIBLE | condition or signal | 0=success, -ERESTARTSYS=signal received |
| wait_event_interruptible_timeout(wq, condition, timeout) | TASK_INTERRUPTIBLE | condition, signal, or timeout | 0, 1, remaining jiffies, or -ERESTARTSYS |
| wait_event_killable(wq, condition) | TASK_KILLABLE | condition or fatal signal | 0=success, -ERESTARTSYS=signal |

- TASK_UNINTERRUPTIBLE → cannot be interrupted by signals (even Ctrl+C). Process only wakes when condition is true.

- TASK_INTERRUPTIBLE → can be woken by signals too. Most commonly used.

- TASK_KILLABLE → only woken by fatal (kill) signals, not regular signals.

| Macro | Woken by | Behavior |
| --- | --- | --- |
| wait_event(wq, cond) | Condition only | Sleep until cond is true (uninterruptible) |
| wait_event_timeout(wq, cond, timeout) | condition or timeout | Sleep until cond is true or timeout |
| wait_event_interruptible(wq, cond) | condition only | Sleep until cond true or signal received |
| wait_event_interruptible_timeout(wq, cond, timeout) | condition or signal | Sleep until cond, timeout, or signal |
| wait_event_killable(wq, cond) |  | Sleep (killable by fatal signal) |

Every time the waitqueue is woken up, the condition is checked again.

Return values vary depending on whether the condition became true, timeout occurred, or signal interrupted.

- 3) Waking Up Tasks

There are functions to wake processes waiting on a queue: When some Tasks are in sleep mode because of the waitqueue, then we can use the below function to wake up those task

| Function | Who It Wakes |
| --- | --- |
| wake_up(&wq) | ONE process in non-interruptible sleep |
| wake_up_all(&wq) | ALL processes on the wait queue |
| wake_up_interruptible(&wq) | ONE process in interruptible sleep |
| wake_up_sync(&wq) | Wakes but does NOT reschedule immediately |
| wake_up_interruptible_sync(&wq) | Same as above but for interruptible sleep |

- Driver Example (Key Idea) - In the provided driver code example:

- A kernel thread runs in a loop waiting on the waitqueue.
- The read() function triggers a wakeup with a flag to show read event.
- The thread wakes up, checks the flag, performs action, and sleeps again.
- On driver exit, another wakeup is triggered to exit the thread.

This demonstrates how a waitqueue can synchronize actions like read events and cleanup operations in a device driver.

Build and Test commands

sudo make                        # build

sudo insmod driver.ko            # load → thread starts, sleeps immediately

sudo cat /dev/etx_device         # triggers read → wakes thread (flag=1)

sudo dmesg                       # see: "Event Came From Read Function - 1"

sudo rmmod driver                # triggers exit → wakes thread (flag=2)

sudo dmesg                       # see: "Event Came From Exit Function"

- Key Points to Remember
- Waitqueues let a process sleep efficiently until a specific condition or event happens.
- They are suited for drivers where you must wait for hardware or data.
- Proper initialization and wakeup calls are essential to avoid race conditions.
- Choosing the right waiting macro depends on whether interrupts/signals should break sleep.

- Refer kthread at : Device Driver 19 - Kernel Thread

# Device Driver 11 - SysFS Tutorial

What is Sysfs?

- Sysfs is a virtual filesystem mounted at /sys. Like procfs, it lives in RAM (not on disk). It exports kernel data structures, their attributes, and relationships to user space as files and directories.

- Many files are readable, and some are writable to support runtime configuration.

Purpose of Sysfs - Sysfs is mainly used for:

- Exposing device and driver information to userspace

- Allowing userspace to interact with kernel components via simple file reads/writes

- Providing a standardized interface for devices and kernel objects
(Sysfs is tied to the kernel’s device model — procfs is mainly for process info.)

### Sysfs vs Procfs

|  | Procfs (/proc) | Sysfs (/sys) |
| --- | --- | --- |
| Purpose | Process & kernel info | Device/driver attributes |
| Structure | Less organized | Strictly hierarchical |
| Rule | No strict format | One value per file |
| Used for | General kernel info | Hardware attributes |

### Required Headers

#include <linux/sysfs.h>		/* sysfs_create_file, sysfs_remove_file   */

#include <linux/kobject.h> 	/* kobject, kobject_create_and_add, etc.  */

- 🧩 Core Concept — Kernel Objects (kobjects)

- kobject is the core data structure linking kernel components to sysfs.

- Kobject is the glue between the kernel and sysfs. Every directory you see in /sys is backed by a kobject.

- Every kobject gets a directory in /sys with a name and potentially attributes underneath.

struct kobject {

char  		*name;    /* directory name in /sys            */

struct kobject 	*parent;  /* parent directory's kobject        */

struct kref       	 kref;    /* reference counter (auto free)     */

struct kset       	*kset;    /* group of related kobjects         */

struct kobj_type  	*ktype;   /* type: holds sysfs ops             */

struct sysfs_dirent	*sd;    /* sysfs internal representation     */

};

- Without diving deep, kobject is essential for creating sysfs entries.

- Step 1. Create a Directory in /sys

Use:

struct kobject *kobject_create_and_add(const char *name, struct kobject *parent);

- name — directory name under /sys.

- parent — where to place it (e.g., kernel_kobj puts it under /sys/kernel/).

| Parent Argument | Creates directory under |
| --- | --- |
| kernel_kobj | /sys/kernel/ |
| mm_kobj | /sys/kernel/mm/ |
| power_kobj | /sys/power/ |
| NULL | /sys/ (top level) |

- After use, call kobject_put() to free it safely.

Example

struct kobject *kobj_ref;

/*Creating a directory in /sys/kernel/ */

kobj_ref = kobject_create_and_add("etx_sysfs",kernel_kobj);	//sys/kernel/etx_sysfs

/*Freeing Kobj*/

kobject_put(kobj_ref);

### Step 2 — Define the Attribute (sysfs file)

An attribute = one sysfs file. Use struct kobj_attribute with the __ATTR macro.

struct kobj_attribute {

struct attribute attr;          /* name + permissions              */

ssize_t (*show)(struct kobject *, struct kobj_attribute *, char *);

ssize_t (*store)(struct kobject *, struct kobj_attribute *, const char *, size_t);

};

| Function | Called When | Direction |
| --- | --- | --- |
| show() | User reads the file (cat) | Kernel → User |
| store() | User writes the file (echo) | User → Kernel |

Define using __ATTR macro:

/* __ATTR(name, permissions, show_fn, store_fn) */

static struct kobj_attribute etx_attr = __ATTR(etx_value, 0660, sysfs_show, sysfs_store);

### Step 3 — Create the Sysfs File

/* Links the attribute (file) to the kobject (directory) */

int sysfs_create_file(struct kobject *kobj, const struct attribute *attr);

/* Usage: */

sysfs_create_file(kobj_ref, &etx_attr.attr);

/* → creates /sys/kernel/etx_sysfs/etx_value */

Returns 0 on success, negative on failure.

Cleanup:

sysfs_remove_file(kobj_ref, &etx_attr.attr);

### show() and store() Functions

/* Called on: cat /sys/kernel/etx_sysfs/etx_value */

static ssize_t sysfs_show(struct kobject *kobj, struct kobj_attribute *attr, char *buf)

{

/* sprintf writes value into buf — sysfs sends buf to user */

return sprintf(buf, "%d\n", etx_value);

}

/* Called on: echo 10 > /sys/kernel/etx_sysfs/etx_value */

static ssize_t sysfs_store(struct kobject *kobj, struct kobj_attribute *attr,

const char *buf, size_t count)

{

/* sscanf reads value from buf — buf contains what user wrote */

sscanf(buf, "%d", &etx_value);

return count;   /* MUST return count or shell retries */

}

### Init & Exit — What's New vs Previous Tutorials

Init adds:

kobj_ref = kobject_create_and_add("etx_sysfs", kernel_kobj); 	/* make /sys/kernel/etx_sysfs/ */

sysfs_create_file(kobj_ref, &etx_attr.attr);                 /* make etx_value file */

Exit adds (before device cleanup):

sysfs_remove_file(kernel_kobj, &etx_attr.attr); 	/* remove file */

kobject_put(kobj_ref);                            /* free kobject */

### Test Commands

sudo insmod driver.ko

ls /sys/kernel/etx_sysfs/           		# see etx_value file

cat /sys/kernel/etx_sysfs/etx_value 	# read  → calls sysfs_show()

echo 100 > /sys/kernel/etx_sysfs/etx_value  # write → calls sysfs_store()

cat /sys/kernel/etx_sysfs/etx_value 	# read again → prints 100

sudo dmesg   # view kernel logs

sudo rmmod driver

### Complete Flow Summary

insmod driver.ko

├── /dev/etx_device              → created (via udev)

├── /sys/kernel/etx_sysfs/       → created (via kobject_create_and_add)

└── /sys/kernel/etx_sysfs/etx_value → created (via sysfs_create_file)

cat /sys/kernel/etx_sysfs/etx_value

└── sysfs_show() → sprintf(buf, "%d\n", etx_value) → prints: 0

echo 100 > /sys/kernel/etx_sysfs/etx_value

└── sysfs_store() → sscanf(buf, "%d", &etx_value) → etx_value = 100

cat /sys/kernel/etx_sysfs/etx_value

└── sysfs_show() → prints: 100

rmmod driver

├── sysfs_remove_file() → removes etx_value file

├── kobject_put()       → removes etx_sysfs/ dir

└── /dev/etx_device     → removed

- Example Use Case

In the tutorial example:

- A kernel module creates a directory at: /sys/kernel/etx_sysfs

- Inside this directory is a file: /sys/kernel/etx_sysfs/etx_value

- Reading this file shows an integer value. Writing updates that value.

- This allows userspace to read and control a kernel module variable simply using standard I/O commands (cat, echo).

- How Userspace Interacts with Sysfs

- Applications open, read, and write sysfs files like normal files.

- Sysfs forwards these operations to kernel functions (show and store).

- Attributes are usually text-based ASCII for simplicity and tool compatibility.

- Important Takeaways

✔ Sysfs is a kernel → userspace interface for device and kernel data. (kernel.org)
✔ It uses virtual filesystem semantics — no physical storage is used. (kernel.org)
✔ Each item in sysfs (directory/file) is backed by kernel objects and callbacks. (EmbeTronicX)
✔ Useful for debugging, configuration, device control and monitoring. (EmbeTronicX)

# Device Driver 12 - Interrupts Tutorial

## 1️. What Is an Interrupt?

- An interrupt is a signal from hardware that tells the CPU to stop its current work and attend to an event. The function that handles it is called an Interrupt Handler or ISR (Interrupt Service Routine).

- Without interrupts the CPU would have to poll devices continuously — wasting time checking devices for events. (EmbeTronicX)

Polling vs Interrupts

| Polling | Interrupts |
| --- | --- |
| CPU repeatedly checks status | CPU responds only when device signals it |
| Less efficient | More efficient and responsive |

→ Interrupts help the system respond quickly without constant checking.

## 2️. How Interrupts Work

- Hardware asserts an interrupt line to the interrupt controller.

- The controller signals the CPU.

- CPU stops its current execution and notifies the OS.

- The kernel handles the interrupt with the interrupt handler (ISR).

Hardware device → signal → Interrupt Controller

│

▼

CPU detects signal

│

▼

CPU pauses current task

│

▼

OS notified → runs ISR for that device

│

▼

Resumes previous task

## 3️. Interrupts vs Exceptions

|  | Interrupts | Exceptions |
| --- | --- | --- |
| Triggered by | Hardware (async) | Processor (sync) |
| When | Anytime | During instruction execution |
| Example | Network card, keyboard | Divide by zero, page fault |

#### Types of Interrupts

- Maskable — can be disabled/ignored temporarily  by the cpu (e.g., normal I/O device IRQs)

- Non-maskable — always processed, never ignored (e.g., hardware failures)

#### Types of Exceptions

- Faults — can be corrected and retried (e.g., page fault, divide by zero, segfault)

- Traps — reported after instruction executes (e.g., breakpoints used in debugging)

- Aborts — severe unrecoverable errors (e.g., hardware failure, corrupt system tables)

## 4️. Interrupt Handler (ISR)

- Every device that generates interrupts must register an ISR

- ISR is a normal C function with a specific prototype

- Runs in interrupt context (also called atomic context)

#### Rules of Interrupt Context — Cannot do:

❌ Sleep or block

❌ Acquire a mutex

❌ Perform time-consuming work

❌ Access user space memory

Why? Because interrupt context runs to completion — it is NOT preemptible. Sleeping here would freeze the system.

#### ISR must be:

✅ Fast (complete in microseconds)

✅ Acknowledge the hardware immediately

✅ Do only time-critical work

## 5️. Execution Contexts

### Process Context

- Runs user-space system calls.

- Preemptible (can be interrupted). Can sleep, can be tied to a process or block

### Interrupt Context

- Runs ISR code. (Handling hardware interrupts)

- Not preemptible.

- Restrictions in interrupt context:

  - Cannot sleep or block.

  - Cannot acquire blocking locks.

  - Cannot access user-space memory. (EmbeTronicX)

## 6️⃣ Top Half vs Bottom Half

To handle heavy interrupt processing efficiently, the work is split:

#### Top Half

- Runs immediately when interrupt arrives

- Does only time-critical work:

  - Acknowledge the hardware

  - Copy data from device to memory (e.g., network packet)

- Fast — in and out as quickly as possible

#### Bottom Half

- Runs later when convenient

- Does the heavy processing (e.g., processing the network packet)

- Interrupts are enabled while it runs

- NOT time-critical

Bottom Half Mechanisms in Linux

| Mechanism | Notes |
| --- | --- |
| Workqueue | Runs in process context — can sleep. Most flexible. |
| Threaded IRQ | ISR runs in a dedicated kernel thread |
| Softirq | Runs in interrupt context — cannot sleep. Very fast. |
| Tasklet | Built on top of softirq. Easier to use. Cannot sleep. |

## Flow summery

Hardware event

│

▼

IRQ line → Interrupt Controller → CPU

│

▼

Kernel saves current state

│

▼

Runs ISR (Top Half) — FAST, atomic context

│

▼

Schedules Bottom Half (Workqueue / Tasklet / Softirq / Threaded IRQ)

│

▼

Restores previous state → resumes normal execution

│

▼

Bottom Half runs later — can do heavy work, interrupts enabled

## Linux Generic IRQ Handling (Advanced)

This subsystem:

- Abstracts interrupt controller details.

- Lets drivers use common APIs like:

  - request_irq()

  - free_irq()

  - enable_irq()

  - disable_irq() (kernel.org)

This makes drivers portable across architectures.

## Glossary

- ISR (Interrupt Service Routine): Function to handle interrupts.

- IRQ: Number assigned to an interrupt source.

- Top Half: Immediate handling part of interrupt.

- Bottom Half: Deferred work part.

- Softirq/Tasklet: Mechanisms for deferred processing.

# Device Driver 13 - Interrupt Programming

### 1. What Is an Interrupt Handler?

A Interrrupt handler or ISR – Interrupt Service Routine is a function that runs when a specific interrupt occurs. It's part of the kernel and must be fast and simple.

## 2. Rules for Interrupt Handlers

Before writing an interrupt driver:

- No sleeping — never call functions that sleep inside an ISR

- Use spinlocks not mutexes — mutex will put ISR to sleep if unavailable

- No user space access — cannot exchange data with user space

- Be fast — split heavy work into top half (ISR) + bottom half (workqueue/tasklet/softirq)

- Not re-entrant — ISR won't be called again until current one finishes (its own IRQ is disabled)

- Higher priority can interrupt — unless marked as fast handler (IRQF_DISABLED), but overusing this degrades system performance

## 3. Core Interrupt Functions

#### request_irq() — Register an ISR

int request_irq(unsigned int irq,

irq_handler_t handler,

unsigned long flags,

const char *name,

void *dev_id);

| Parameter | Meaning |
| --- | --- |
| irq | IRQ number to register (e.g., 11) |
| handler | Your ISR function |
| flags | Behavior flags (see below) |
| name | Device name shown in /proc/interrupts |
| dev_id | Unique cookie — required for shared IRQs, used to identify handler on free (“cookie” is just a small piece of data used as an identifier or tag.) |

- Returns 0 on success, negative on failure

- ⚠️ Cannot be called from interrupt context — it can block

#### free_irq() — Unregister an ISR

free_irq(unsigned int irq, void *dev_id);

irq – IRQ number to the register
dev_id -  must match what was passed to request_irq()

- Must be called from process context (not interrupt context)

- If IRQ is shared → only removes THIS handler; line stays active until last handler removed

Other Useful Functions

| Function | Purpose |
| --- | --- |
| enable_irq(irq) | Re-enable a disabled IRQ |
| disable_irq(irq) | Disable an IRQ (waits for running handler to finish) |
| disable_irq_nosync(irq) | Disable immediately, doesn't wait for handler |
| in_irq() | Returns true if currently inside an interrupt handler |
| in_interrupt() | Returns true if in handler OR bottom half |

## Important Flags for Interrupts

These go into the flags argument of request_irq():

| Flag | Meaning |
| --- | --- |
| IRQF_DISABLED | Disable ALL interrupts while this ISR runs (fast handler — use sparingly) |
| IRQF_SHARED | IRQ line is shared among multiple handlers — each handler must set this |
| IRQF_TIMER | Handler processes system timer interrupts |
| IRQF_SAMPLE_RANDOM | Interrupt timing fed to kernel entropy pool (good for random hardware) |

## Registering & Releasing an Interrupt

### Register

#define IRQ_NO 11

if (request_irq(IRQ_NO, irq_handler, IRQF_SHARED, "etx_device",

(void *)(irq_handler))) {

printk(KERN_INFO "my_device: cannot register IRQ ");

goto irq;

}

This tells Linux: for interrupt line 11, call irq_handler() when interrupt happens.

### Free/Release

free_irq(IRQ_NO, (void *)(irq_handler));

Removes the registered handler when driver exits.

### Interrupt Handler Prototype

static irqreturn_t irq_handler(int irq, void *dev_id)

{

/* do minimal fast work here */

return IRQ_HANDLED;   /* success */

/* return IRQ_NONE if this wasn't our interrupt */

}

| Return Value | Meaning |
| --- | --- |
| IRQ_HANDLED | ISR handled the interrupt successfully |
| IRQ_NONE | This interrupt wasn't from our device |

### Triggering Interrupt via Software (No Hardware Needed)

On x86, the IDT (Interrupt Descriptor Table) has 256 entries — each maps a vector number to an interrupt handler. The CPU uses int <vector> instruction to trigger interrupts programmatically.

IRQ to Vector mapping (x86):

Vector = FIRST_EXTERNAL_VECTOR + 0x10 + IRQ_number

= 0x20 + 0x10 + 11

= 0x3B  (59 in decimal)

So to trigger IRQ 11 from software:  asm("int $0x3B");   /* fires IRQ 11 */

This is placed inside etx_read() — so every cat /dev/etx_device fires the interrupt.

## Test Commands

sudo make                        	# build

sudo insmod driver.ko	# load — registers IRQ 11 handler

cat /proc/interrupts | grep etx  	# confirm IRQ 11 registered

sudo cat /dev/etx_device         	# triggers read → fires software interrupt

dmesg                            		# see: "Shared IRQ: Interrupt Occurred"

sudo rmmod driver                # unload — frees IRQ 11

### New Kernel Issue & Fix

On newer kernels you may see:

do_IRQ: 1.59 No irq handler for vector

Root cause: vector_irq is not exported in newer kernels — so the software interrupt trick doesn't work directly.

Fix for kernel 5.4.47:

- Download kernel source

- Add EXPORT_SYMBOL(vector_irq); in arch/x86/kernel/irq.c

- Recompile and install the custom kernel

In the new driver, etx_read() adds extra setup:

struct irq_desc *desc;

desc = irq_to_desc(11);          /* get IRQ 11 descriptor */

if (!desc) return -EINVAL;

__this_cpu_write(vector_irq[59], desc);  /* map vector 59 → IRQ 11 */

asm("int $0x3B");                /* now fire it */

## Summary – Key Takeaways

- Interrupts register with request_irq()
- Handlers must be simple, non-sleeping functions
- Flags control interrupt behavior
- Interrupts can be simulated for test without hardware
- Bottom halves (workqueues/tasklets) handle deferred work
- Always free interrupts when done

# Device Driver 14 - Workqueue (Static Method)

### Why Workqueue?

ISR (Top Half) must be fast. If there's heavy work to do after an interrupt, we push it to a Bottom Half that runs later. Workqueue is one of the 4 bottom-half mechanisms.

### 4 Bottom Half Mechanisms

| Mechanism | Runs In | Can Sleep? |
| --- | --- | --- |
| Workqueue | Process context | ✅ Yes |
| Threaded IRQ | Kernel thread | ✅ Yes |
| Softirq | Atomic context | ❌ No |
| Tasklet | Atomic context | ❌ No |

Simple rule: Need to sleep in bottom half? → Use Workqueue. No sleep needed? → Use Softirq or Tasklet.

### What is a Workqueue?

- Introduced in kernel 2.6

- Defers work(postpones execution)| to a kernel thread[Runs inside the kernel (not user space)] | (runs like a process context [process context means code runs like a normal process])

- Can sleep, use mutexes, and do time-consuming work

- Most flexible bottom-half mechanism

### 2 Ways to Implement Workqueue

1. Global Workqueue (use kernel's built-in worker thread)

├── Static Method  ← This tutorial

└── Dynamic Method ← Next tutorial

2. Own Workqueue (create your own worker thread)

### Global Workqueue — Static Method

No need to create your own workqueue or thread — use the kernel's global one.

#### Required Header -> #include <linux/workqueue.h>

#### Step 1 — Initialize Work (Static)

DECLARE_WORK(name, func);

name = name of work_struct variable created(static struct work_struct 											 workqueue;)

func = function to run when work is scheduled

/* Example: */

DECLARE_WORK(workqueue, workqueue_fn);

#### Step 2 — Write the Work Function

void workqueue_fn(struct work_struct *work)

{

/* heavy processing goes here — CAN sleep */

printk(KERN_INFO "Executing Workqueue Function\n");

}

#### Step 3 — Schedule Work from ISR

int schedule_work( struct work_struct *work );   /* adds work to global workqueue */

int scheduled_delayed_work( struct delayed_work *dwork, unsigned long delay );

int schedule_work_on( int cpu, struct work_struct *work );

int scheduled_delayed_work_on(int cpu, struct delayed_work *dwork, unsigned long delay );

All Scheduling Functions

| Function | Purpose |
| --- | --- |
| schedule_work(&work) | Add work to global workqueue immediately |
| scheduled_delayed_work(&dwork, delay) | Add work after delay jiffies |
| schedule_work_on(cpu, &work) | Add work on a specific CPU |
| scheduled_delayed_work_on(cpu, &dwork, delay) | Delayed + specific CPU |

- All return non-zero if work was newly queued, zero if already queued

- delay = 0 means run immediately (same as schedule_work)

### Flush Functions (Wait for work to complete)

int flush_work( struct work_struct *work );	/* wait for specific work item to finish  */

void flush_scheduled_work( void );	/* wait for ALL global workqueue work     */

- Caller blocks until the work completes

- Use when you need to ensure work is done before proceeding

### Cancel Functions (Stop work before it runs)

int cancel_work_sync( struct work_struct *work );		/* cancel or wait if already running  */

int cancel_delayed_work_sync( struct delayed_work *dwork );	/*same but for delayed work          */

- If work hasn't started → cancelled immediately

- If work is already running → waits for it to finish then returns

### Check Functions (Is work pending?)

work_pending(&work);           /* returns true if work not yet executed */

delayed_work_pending(&dwork);  /* same for delayed work                 */

### Test Commands

sudo make                        # build

sudo insmod driver.ko            # load driver

sudo cat /dev/etx_device         # triggers read → fires IRQ → schedules work

dmesg                            # see both messages:

# "Shared IRQ: Interrupt Occurred"

# "Executing Workqueue Function"

sudo rmmod driver                # unload

Expected dmesg output:

Shared IRQ: Interrupt Occurred       ← ISR (Top Half) ran

Executing Workqueue Function          ← Workqueue (Bottom Half) ran

# Device Driver 15 - Workqueue (Dynamic Method)

### Only One New Concept — INIT_WORK()

/* Step 1:   */

static struct work_struct workqueue;

/* Step 2: Initialize it at runtime inside init() */

INIT_WORK(&workqueue, workqueue_fn);

/*

* &workqueue = pointer to the work_struct to initialize

* workqueue_fn = the function to run when work is scheduled

*

* MUST be called BEFORE request_irq() in a real driver

* to avoid the interrupt firing before the work is initialized.

*/

# Device Driver 16 - Own Workqueue

Workqueues are created through a macro called create_workqueue, which returns a workqueue_struct reference. You can remove this workqueue later (if needed) through a call to the destroy_workqueue function.

### Global Workqueue vs Own Workqueue

|  | Global Workqueue (Part 14/15) | Own Workqueue (Part 16) |
| --- | --- | --- |
| Created by | Kernel (always exists) | You (create_workqueue()) |
| Schedule work | schedule_work(&work) | queue_work(own_wq, &work) |
| Destroy | Not needed | destroy_workqueue(own_wq) |
| Visible in ps(process status) | No | Yes — [own_wq] appears |
| Control | Limited | Full (priority, CPU binding, etc.) |

### Required Header : #include <linux/workqueue.h>

### Create & Destroy Your Own Workqueue

#### Create (multi-thread — one worker per CPU)

struct workqueue_struct *create_workqueue(name);

/* name = name of your workqueue (shows in ps -aef as [name]) */

/* Example: */

own_workqueue = create_workqueue("own_wq");

#### Create (single-thread — one worker for ALL CPUs)

struct workqueue_struct *create_singlethread_workqueue(name);

#### Under the Hood — Both use alloc_workqueue()Since create_workqueue and create_singlethread_workqueue() are macros. Both are using the alloc_workqueue function in the background.

/* create_workqueue internally calls: */

alloc_workqueue("%s", WQ_MEM_RECLAIM, 1, namcd ldd	e);

/* create_singlethread_workqueue internally calls: */

alloc_workqueue("%s", WQ_UNBOUND | WQ_MEM_RECLAIM, 1, name);

#### alloc_workqueue() — Direct API (for advanced control)

alloc_workqueue(fmt, flags, max_active);

/*

* fmt        = printf format for workqueue name

* flags      = WQ_* flags (see below)

* max_active = max parallel work items, 0 = default

* Returns: workqueue pointer on success, NULL on failure

*/

#### Destroy

destroy_workqueue(own_workqueue);

/* Waits for all pending work to finish, then destroys the workqueue.

* MUST be called in exit() to prevent memory leaks. */

### WQ_* Flags (for alloc_workqueue)

| Flag | Meaning |
| --- | --- |
| WQ_UNBOUND | Workers NOT bound to any specific CPU. Useful for wide concurrency swings or long CPU-intensive tasks |
| WQ_FREEZABLE | Participates in system suspend — drains work during freeze, resumes after thawed(unfreezed) |
| WQ_MEM_RECLAIM | Guarantees at least one execution context even under memory pressure. MUST set if used in memory reclaim paths |
| WQ_HIGHPRI | Work runs in high-priority worker threads (elevated nice levels). Normal and highpri pools are completely separate |
| WQ_CPU_INTENSIVE | CPU-heavy work — doesn't count toward concurrency level, lets scheduler manage execution timing |

### Queue Work to Your Own Workqueue

Instead of schedule_work(), use queue_work():

| Function | Purpose |
| --- | --- |
| int queue_work( struct workqueue_struct *wq, struct work_struct *work ); wq – workqueue to use work – work to queue It returns false if work was already on a queue, true otherwise. | Add work to your workqueue on current CPU |
| int queue_work_on( int cpu, struct workqueue_struct *wq, struct work_struct *work ); cpu– cpu to put the work task on wq – workqueue to use work– job to be done | Add work on a specific CPU |
| int queue_delayed_work( struct workqueue_struct *wq,struct delayed_work *dwork, unsigned long delay ); wq – workqueue to use dwork – work to queue delay– number of jiffies to wait before queueing or 0 for immediate execution | Add work after delay jiffies |
| int queue_delayed_work_on( int cpu, struct workqueue_struct *wq,struct delayed_work *dwork, unsigned long delay ); cpu– CPU to put the work task on wq – workqueue to use dwork – work to queue delay– number of jiffies to wait before queueing or 0 for immediate execution | Delayed + specific CPU |

All return true if work was newly queued, false if already queued.

### schedule_work vs queue_work — Key Difference

schedule_work(&work)          → uses GLOBAL kernel workqueue

queue_work(own_wq, &work)     → uses YOUR OWN workqueue

Use schedule_work when you don't need a dedicated workqueue. Use queue_work when you need full control (priority, CPU affinity, visibility, concurrency limits).

### Test Commands

sudo make

sudo insmod driver.ko

sudo cat /dev/etx_device    # fires IRQ → queues work to own_wq

dmesg                       # see both messages

ps -aef | grep own_wq       # see your workqueue thread: [own_wq]

sudo rmmod driver           # destroy_workqueue() runs → thread gone

Expected dmesg:

Shared IRQ: Interrupt Occurred       ← ISR (Top Half)

Executing Workqueue Function          ← own_wq (Bottom Half)

Visible in ps:

root  3516  2  0  21:35  ?  00:00:00  [own_wq]

All 3 Workqueue Methods Side-by-Side

/* ── Part 14: Global Workqueue — Static ── */

DECLARE_WORK(work, workqueue_fn);      	/* compile-time init              */

schedule_work(&work);                  		/* submit to GLOBAL workqueue     */

/* No create/destroy needed — kernel owns the global workqueue */

/* ── Part 15: Global Workqueue — Dynamic ── */

struct work_struct work;               /* declare manually               */

INIT_WORK(&work, workqueue_fn);        /* runtime init in init()         */

schedule_work(&work);                  /* submit to GLOBAL workqueue     */

/* No create/destroy needed */

/* ── Part 16: Own Workqueue ── */

struct workqueue_struct *own_wq;       /* pointer to our workqueue       */

DECLARE_WORK(work, workqueue_fn);      /* work init (static or dynamic)  */

own_wq = create_workqueue("own_wq");   /* CREATE our workqueue in init() */

queue_work(own_wq, &work);            /* submit to OUR workqueue        */

destroy_workqueue(own_wq);            /* DESTROY in exit() — mandatory! */

# Device Driver 17 - Linked List 1

## What is a Linked List?

A sequence of nodes where each node holds data and a pointer to the next node. A head pointer always tracks the first element.

HEAD → [data|next] → [data|next] → [data|next] → NULL

Nodes do not need contiguous memory — they can be anywhere in RAM.

## Advantages & Disadvantages of linked list

| Advantages | Disadvantages |
| --- | --- |
| Dynamic size — grows/shrinks at runtime | Extra memory needed for pointers |
| Easy insert and delete anywhere | No random access — must traverse from head |
| Used to implement stacks, queues, graphs | Reverse traversal is harder |
| No need to know size in advance |  |

## Types of Linked Lists

- Singly — each node points only forward (next)

- Doubly — each node points forward (next) AND backward (prev)

- Circular — last node points back to head

## Linux Kernel's Built-in Linked List

The kernel provides a ready-made doubly linked list — no need to implement your own and it is Defined in:

/lib/modules/$(uname -r)/build/include/linux/list.h

### How it's Different from Normal Linked List

Normal way (you manage prev/next in YOUR struct):

struct my_list {

int data;

struct my_list *prev;   /* you manage this */

struct my_list *next;   /* you manage this */

};

Linux kernel way (embed list_head INSIDE your struct):

struct my_list {

struct list_head list; //kernel manages prev/next inside here

int data;

};

### struct list_head (the kernel's node type)

struct list_head {

struct list_head *next;		//holds the address of next struct instance(node)

struct list_head *prev;		//holds the address of prev struct instance(node)

};

The key idea: you embed list_head inside your struct. The kernel's macros then use container_of() to get back to your full struct from any list_head pointer.

## Step-by-Step Usage

### Step 1 — Create the Head Node

LIST_HEAD(etx_linked_list);

/* Creates + initializes head node.

* Internally:

*   struct list_head etx_linked_list = { &etx_linked_list, &etx_linked_list };

* Both prev and next point to ITSELF → means list is EMPTY.

*/

### Step 2 — Create a Node -- INIT_LIST_HEAD(struct list_head *list);

struct my_list {

struct list_head list;  /* MUST be first or at least present */

int data;

};

struct my_list new_node;

INIT_LIST_HEAD(&new_node.list);  /* initialize the list_head inside */

new_node.data = 10;

### Step 3 — Add Node to List

list_add(&new_node.list, &etx_linked_list);

/* Adds AFTER head → good for STACK (LIFO) */

list_add_tail(&new_node.list, &etx_linked_list);

/* Adds BEFORE head → good for QUEUE (FIFO) */

## All API Functions — Quick Reference

### Add / Delete / Replace

| Function | Purpose |
| --- | --- |
| list_add(new, head) inline void list_add(struct list_head *new, struct list_head *head); | Insert after head → stack behavior |
| list_add_tail(new, head) inline void list_add_tail(struct list_head *new, struct list_head *head); | Insert before head → queue behavior |
| list_del(entry) inline void list_del(struct list_head *entry); | Remove node (does NOT free memory) |
| list_del_init(entry) inline void list_del_init(struct list_head *entry); | Remove node + reinitialize it (does NOT free memory) |
| list_replace(old, new) inline void list_replace(struct list_head *old, struct list_head *new); | Replace old node with new node |
| list_replace_init(old, new) inline void list_replace_init(struct list_head *old, struct list_head *new); | Replace + reinitialize old node |

⚠️ list_del() and list_del_init() only disconnect the node. You still need to call kfree() yourself to free the memory!

### Move / Rotate

| Function | Purpose |
| --- | --- |
| list_move(list, head) inline void list_move(struct list_head *list, struct list_head *head); | Remove + re-add AFTER head |
| list_move_tail(list, head) inline void list_move_tail(struct list_head *list, struct list_head *head); | Remove + re-add BEFORE head |
| list_rotate_left(head) inline void list_rotate_left(struct list_head *head); | Rotate list to the left |

### Test / Check

| Function | Returns |
| --- | --- |
| list_empty(head) inline int list_empty(const struct list_head *head); | 1 if list is empty, 0 otherwise |
| list_is_last(list, head) inline int list_is_last(const struct list_head *list, const struct list_head *head); | 1 if node is the last entry |
| list_is_singular(head) inline int list_is_singular(const struct list_head *head); | 1 if list has exactly one entry |

### Split / Join

| Function | Purpose |
| --- | --- |
| list_cut_position(list, head, entry) inline void list_cut_position(struct list_head *list, struct list_head *head, struct list_head *entry); list – a new list to add all removed entries head– a list with entries entry– an entry within the head could be the head itself and if so we won’t cut the list | Split list into two at entry point |
| list_splice(list, head) inline void list_splice(const struct list_head *list, struct list_head *head); list – the new list to add. head – the place to add it in the first list. | Join two lists together |

## Traversal Macros

### list_entry() — Get your struct back from a list_head pointer

list_entry(ptr, type, member);

/* ptr    = pointer to struct list_head

type   = your struct type (e.g., struct my_list)

member = name of list_head field inside your struct (e.g., list)

Returns: pointer to YOUR struct */

### list_for_each() — Iterate over list (forward)

list_for_each(pos, head);

/* pos  = loop cursor (struct list_head *)

head = your list head */

### list_for_each_entry() — Iterate + get struct in one step

list_for_each_entry(pos, head, member);

/* pos    = loop cursor (your struct type *)

head   = list head

member = name of list_head inside your struct */

### list_for_each_entry_safe() — Safe iteration (allows deletion during loop)

list_for_each_entry_safe(pos, n, head, member);

/* Same as list_for_each_entry BUT safe to delete 'pos' inside the loop.

n = temporary storage pointer (same type as pos) */

⚠️ Always use _safe version when deleting nodes inside a loop!

### Reverse Traversal

list_for_each_prev(pos, head);              /* traverse backward            */

list_for_each_entry_reverse(pos, head, member); /* backward + get struct    */

## Complete Usage Example

/* Your node structure */

struct my_list {

struct list_head list;   /* kernel list node — embed this */

int data;

};

/* Create head */

LIST_HEAD(etx_linked_list);

/* Create + add nodes */

struct my_list *node = kmalloc(sizeof(*node), GFP_KERNEL);

INIT_LIST_HEAD(&node->list);

node->data = 42;

list_add_tail(&node->list, &etx_linked_list);  /* add to tail */

/* Traverse */

struct my_list *tmp;

list_for_each_entry(tmp, &etx_linked_list, list) {

printk(KERN_INFO "data = %d\n", tmp->data);

}

/* Delete node safely during traversal */

struct my_list *safe_tmp;

list_for_each_entry_safe(tmp, safe_tmp, &etx_linked_list, list) {

list_del(&tmp->list);  /* remove from list */

kfree(tmp);            /* free memory — list_del doesn't do this! */

}

## Quick Summary Table

LIST_HEAD(name)              → create + init the head node

INIT_LIST_HEAD(&node.list)   → init a node's list_head

list_add()                   → add after head (STACK)

list_add_tail()              → add before head (QUEUE)

list_del()                   → remove node (you still free memory!)

list_del_init()              → remove + reinit node

list_for_each_entry()        → traverse forward + get your struct

list_for_each_entry_safe()   → traverse forward (safe to delete nodes)

list_for_each_entry_reverse()→ traverse backward + get your struct

list_entry()                 → get your struct from a list_head pointer

list_empty()                 → check if list has no nodes

list_is_last()               → check if node is last

list_is_singular()           → check if list has exactly 1 node

Key takeaway: The Linux kernel's linked list works backwards from what you're used to — instead of your struct containing next/prev pointers, the list_head is embedded inside your struct, and kernel macros use container_of() to go from the list_head back to your struct. This design lets the same list machinery work with ANY struct type.

# Device Driver 18 - Linked List 2

a practical working example combining everything learned so far — char device + workqueue + IRQ + linked list. Each time you write to the device, a new node is added to the linked list. Reading the device prints all nodes.

### How It All Connects

echo 10 > /dev/etx_device

│

├── etx_write()

│     ├── sscanf(buf, "%d", &etx_value)   ← read value from user

│     └── asm("int $0x3B")                 ← fire IRQ 11

│               └── irq_handler()          [TOP HALF]

│                     └── queue_work(own_workqueue, &work)

│                               └── workqueue_fn()    [BOTTOM HALF]

│                                     └── kmalloc new node

│                                     └── node->data = etx_value (10)

│                                     └── list_add_tail() → adds to list

cat /dev/etx_device

└── etx_read()

└── list_for_each_entry() → prints all nodes and their data

rmmod driver

└── list_for_each_entry_safe() → delete all nodes + kfree() each

### Key Difference from Previous Tutorials

Write now does TWO things (unlike before):

- Reads the integer value from user (sscanf)

- Fires the interrupt (asm("int $0x3B"))

Previous tutorials used echo only to trigger the interrupt — here it also carries data that gets stored into the linked list.

### 4 Key Operations

#### 1 — Create Head (global, at compile time)

LIST_HEAD(Head_Node);

/* Creates + initializes the head/anchor of the list.

Both prev and next point to itself = empty list. */

#### 2 — Create Node and Add (in workqueue_fn)

temp_node = kmalloc(sizeof(struct my_list), GFP_KERNEL);  /* allocate node */

temp_node->data = etx_value;                              /* store data     */

INIT_LIST_HEAD(&temp_node->list);                         /* init list_head */

list_add_tail(&temp_node->list, &Head_Node);              /* add to tail    */

#### 3 — Traverse (in etx_read)

list_for_each_entry(temp, &Head_Node, list) {

printk("Node %d data = %d\n", count++, temp->data);

}

#### 4 — Delete All (in exit)

list_for_each_entry_safe(cursor, temp, &Head_Node, list) {

list_del(&cursor->list);  /* remove from list           */

kfree(cursor);            /* free memory — MUST do this */

}

### Test Commands & Expected Output

sudo insmod driver.ko

# Read before writing (empty list)

cat /dev/etx_device

# → Total Nodes = 0

# Write value 10

echo 10 > /dev/etx_device

# → workqueue adds node with data=10

# Read after writing

cat /dev/etx_device

# → Node 0 data = 10

# → Total Nodes = 1

# Write more values

echo 20 > /dev/etx_device

echo 30 > /dev/etx_device

cat /dev/etx_device

# → Node 0 data = 10

# → Node 1 data = 20

# → Node 2 data = 30

# → Total Nodes = 3

sudo rmmod driver   # → all nodes freed automatically

# Device Driver 19 - Kernel Thread

### Process vs Thread

| Process | Thread |
| --- | --- |
| Independent executing program | Independent flow of control WITHIN a process |
| Heavyweight — own address space | Lightweight — shares address space with other threads |
| Context switch is expensive(time-consuming) | Context switch is fast |
| Inter-process communication is slow | Inter-thread communication is fast (shared memory) |

One process can have multiple threads — each runs different code concurrently while sharing the same data.

Disadvantage of threads: Shared memory causes concurrency issues → needs synchronization (mutex, spinlock, etc.)

### 2 Types of Threads

| User Level Thread | Kernel Level Thread |
| --- | --- |
| Kernel has NO idea about them | Managed directly by the OS/kernel |
| Managed entirely by user-space thread library | Thread operations implemented in kernel code |
| All in user space | No thread management code needed in app |

### Required Headers

#include <linux/kthread.h>   /* all kthread functions */

#include <linux/sched.h>     /* task_struct           */

#include <linux/delay.h>     /* msleep()              */

### Kernel Thread API — All Functions

#### 1. Create (only creates, does NOT start)

struct task_struct *kthread_create(int (*threadfn)(void *data),

void *data,

const char namefmt[], ...);

/*

* threadfn  = function the thread will run

* data      = argument passed to threadfn (can be NULL)

* namefmt   = name of thread (shown in ps/top as [name])

*

* Returns: task_struct* on success, ERR_PTR(-ENOMEM) on failure

* Thread is created but NOT running yet — call wake_up_process() to start

*/

#### 2. Start (wake up after create)

int wake_up_process(struct task_struct *p);

/*

* p – The process to be woken up.

* Moves thread from created state to TASK_RUNNING.

* Returns: 1 if thread was woken, 0 if already running

*/

#### 3. Create + Start in One Call (recommended)

struct task_struct *kthread_run(threadfn, data, namefmt, ...);

/*

* = kthread_create() + wake_up_process() combined

* Returns: task_struct* on success, ERR_PTR(-ENOMEM) on failure

* Thread starts running immediately after this call

*/

#### 4. Stop

int kthread_stop(struct task_struct *k);

/*

* k – thread created by kthread_create.

* Sets kthread_should_stop() to true for thread k.

* Wakes the thread if sleeping, then WAITS for it to return.

* Returns: thread's return value, or -EINTR if thread was never woken.

* ⚠️ Thread function must NOT call do_exit() if you use kthread_stop()!

*/

#### 5. Check if Stop was Requested (inside thread function)

int kthread_should_stop(void);

/*

* Returns true when kthread_stop() has been called on this thread.

* Use this in your thread loop to know when to exit.

* Works per-thread — knows which thread is calling it via `current`.

*/

#### 6. Bind Thread to Specific CPU

void kthread_bind(struct task_struct *k, unsigned int cpu);

/*

* k – thread created by kthread_create.

* cpu – CPU (might not be online, must be possible) for k to run on.

* Pins the thread to run only on the specified CPU.

* Must be called before wake_up_process().

*/

### Thread Function Rules

#### For Long-Running Threads (loop forever):

int thread_function(void *pv)

{

while(!kthread_should_stop()) {   /* check stop flag every iteration */

/* do your work here */

msleep(1000);                 /* sleep — won't miss stop signal  */

}

return 0;   /* return value passed back to kthread_stop() caller     */

}

#### For Short One-Shot Threads (runs once and done):

int thread_function(void *pv)

{

/* do task once */

do_exit(0);   /* kill itself — only if kthread_stop() won't be called */

return 0;

}

Rule: If you will call kthread_stop() → use kthread_should_stop() loop and return normally. If nobody calls kthread_stop() → use do_exit().

### Passing Data to Thread Function

int32_t my_value = 42;

/* pass address as void* */

kthread_create(thread_function, (void *) &my_value, "eTx Thread");

/* inside thread_function, cast back */

int thread_function(void *pv)

{

printk("Value = %d\n", *(int32_t*)pv);   /* cast void* back to int32_t* */

return 0;

}

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

# → Device Driver Insert...Done!!!

# → In EmbeTronicX Thread Function 0    ← prints every second

# → In EmbeTronicX Thread Function 1

# → In EmbeTronicX Thread Function 2

# → In EmbeTronicX Thread Function 3

sudo rmmod driver   # → kthread_stop() called → thread exits

# Device Driver 20 - Tasklet (Static Method)

### What is a Tasklet?

A tasklet is a bottom-half mechanism — a small, fast, one-shot function scheduled to run later (after an interrupt). Think of it as a tiny lightweight thread with no stack and no context of its own.

Tasklets are built on top of softirqs — so they run in atomic/interrupt context.

### Tasklet vs Workqueue — Key Difference

|  | Tasklet | Workqueue |
| --- | --- | --- |
| Context | Atomic (interrupt context) | Process context |
| Can sleep? | ❌ NO | ✅ YES |
| Can use mutex? | ❌ NO — use spinlock | ✅ YES |
| Speed | Faster | Slightly slower |
| Use when | Short, fast deferred work | Long or sleeping work |

### Important Rules (Points to Remember)

- ❌ Cannot use sleep(), mutex, semaphore — use spinlock instead

- ✅ Can run in parallel with OTHER tasklets

- ❌ Same tasklet cannot run concurrently with itself (runs on one CPU only)

- ✅ A tasklet always runs on the same CPU core that scheduled it (cache optimization)

- Scheduling uses non-preemptive execution — one by one in turn

- 2 priorities: Normal and High

### Tasklet Structure

struct tasklet_struct {

struct tasklet_struct *next;  	/* next tasklet in queue                   */

unsigned long state;          		/* TASKLET_STATE_SCHED or _RUN             */

atomic_t count;             		/* 0 = enabled, non-zero = disabled        */

void (*func)(unsigned long);  	/* your tasklet function                   */

unsigned long data;           		/* argument passed to func                 */

};

### Create Tasklet — Static Method

#### DECLARE_TASKLET — Creates and ENABLES tasklet at compile time

DECLARE_TASKLET(name, func, data);

/*

* name = variable name of tasklet_struct

* func = function to run (bottom half)

* data = unsigned long argument passed to func

*

* Tasklet starts in ENABLED state — ready to run when scheduled.

*/

/* Example: */

DECLARE_TASKLET(tasklet, tasklet_fn, 1);

/* Internally creates: */

struct tasklet_struct tasklet = { NULL, 0, 0, tasklet_fn, 1 };

#### DECLARE_TASKLET_DISABLED — Creates but DISABLES tasklet

DECLARE_TASKLET_DISABLED(name, func, data);

/* Tasklet can be scheduled but will NOT run until tasklet_enable() is called */

⚠️ Kernel Note: On newer kernels (6.x+), DECLARE_TASKLET(name, func, data) was changed to DECLARE_TASKLET_OLD(name, func) — the data argument was removed from the newer API.

### Enable & Disable Tasklet

tasklet_enable(&tasklet);         /* re-enable a disabled tasklet            */

tasklet_disable(&tasklet);        /* disable + WAIT for running tasklet to finish */

tasklet_disable_nosync(&tasklet); 	/* disable immediately, no waiting         */

Important: If disabled multiple times → must be enabled the same number of times. The count field in tasklet_struct tracks this.

### Schedule (Queue) the Tasklet

tasklet_schedule(&tasklet);

/* Add to NORMAL priority queue.

* If already scheduled (not yet run) → silently ignored (no duplicate).

* Runs on the SAME CPU that called this. */

tasklet_hi_schedule(&tasklet);

/* Add to HIGH priority queue — runs before normal priority tasklets.

* Also silently discards if already scheduled. */

tasklet_hi_schedule_first(&tasklet);

/* High priority + avoids touching other tasklets.

* Use only for special cases like kmemcheck. */

### Kill (Destroy) the Tasklet

tasklet_kill(&tasklet);

/* Waits for tasklet to finish if currently running, then kills it.

* MUST be called in exit() to clean up properly.

* ⚠️ Cannot be called from interrupt context — it may sleep! */

tasklet_kill_immediate(&tasklet, cpu);

/* Only used when a CPU is in the dead state. Rare use case. */

### Tasklet Function Prototype

void tasklet_fn(unsigned long arg)

{

/* Bottom half work here — NO sleeping, NO mutex */

printk(KERN_INFO "Tasklet executing: arg = %ld\n", arg);

}

### Test Commands & Expected Output

sudo insmod driver.ko

sudo cat /dev/etx_device     # triggers read → fires IRQ → schedules tasklet

dmesg

Device File Opened...!!!

Read function

Shared IRQ: Interrupt Occurred     ← ISR (Top Half) ran

Executing Tasklet Function : arg = 1   ← Tasklet (Bottom Half) ran

Device File Closed...!!!

sudo rmmod driver

### All 3 Bottom Half Mechanisms Compared

ISR fires → schedule bottom half:

ISR fires → schedule bottom half:

WORKQUEUE  → queue_work(wq, &work)      	→ process context → CAN  sleep

TASKLET    	→ tasklet_schedule(&tasklet) 	→ atomic context  → CANNOT sleep

SOFTIRQ    	→ raise_softirq(nr)         		 → atomic context  → CANNOT sleep

(lowest level)

Use WORKQUEUE when:  need to sleep, mutex, heavy work, or I/O

Use TASKLET when:    	fast work, no sleep, simpler than softirq

Use SOFTIRQ when:    	absolute maximum performance (network stack uses this)

Key takeaway: Tasklet = fast bottom half in atomic context. Static method uses DECLARE_TASKLET() at compile time. Always call tasklet_kill() in exit before free_irq(). Never sleep inside a tasklet.

# Device Driver 21 - Tasklet (Dynamic Method)

### What's Different from Static Method?

This tutorial covers the same tasklet behavior as Part 20, but uses tasklet_init() at runtime instead of DECLARE_TASKLET() at compile time.

|  | Static Method (Part 20) | Dynamic Method (Part 21) |
| --- | --- | --- |
| How | DECLARE_TASKLET(name, func,data) | tasklet_init(&tasklet, func, data) |
| When | Compile time | Runtime (inside init()) |
| Memory | Stack/global — auto | Heap — use kmalloc() + kfree() |
| Pointer | struct tasklet_struct tasklet | struct tasklet_struct *tasklet |
| Behavior | Identical | Identical |

Everything else (scheduling, enable/disable, kill) is 100% identical to the Static Method. Refer to Part 20 notes for those.

### Only One New Concept — tasklet_init()

void tasklet_init(struct tasklet_struct *t,

void (*func)(unsigned long),

unsigned long data);

/*

* t    = pointer to tasklet_struct to initialize

* func = bottom half function to run

* data = unsigned long argument passed to func

*

* Sets up the tasklet struct fields internally:

*   tasklet->func  = func

*   tasklet->data  = data

*   tasklet->state = TASKLET_STATE_SCHED

*   tasklet->count = 0  (enabled)

*/

### Dynamic Method — 3 Steps

#### Step 1 — Declare a POINTER (not a variable)

struct tasklet_struct *tasklet = NULL;

/* Just a pointer — no memory yet, no initialization */

#### Step 2 — Allocate + Initialize in init()

/* Allocate memory for the tasklet_struct on the heap */

tasklet = kmalloc(sizeof(struct tasklet_struct), GFP_KERNEL);

if(tasklet == NULL) {

printk(KERN_INFO "Cannot allocate Memory");

goto irq;   /* cleanup on failure */

}

/* Initialize the tasklet at runtime */

tasklet_init(tasklet, tasklet_fn, 0);

/*            ↑pointer  ↑function  ↑data arg (0 here) */

#### Step 3 — Kill + Free in exit()

tasklet_kill(tasklet);      /* stop tasklet — wait if running */

if(tasklet != NULL) {

kfree(tasklet);         /* free kmalloc'd memory — MANDATORY */

}

⚠️ tasklet_kill() alone is NOT enough for dynamic method — you MUST also call kfree() to free the heap memory. Static method has no kfree() because the struct was not kmalloc'd.

### Scheduling — Same as Static (pointer syntax difference only)

/* Static method: uses address of global variable (&)*/

tasklet_schedule(&tasklet);   /* & because tasklet is struct, not pointer */

/* Dynamic method: already a pointer */

tasklet_schedule(tasklet);    /* no & needed — tasklet IS a pointer */

### Test Commands & Expected Output

sudo insmod driver.ko

sudo cat /dev/etx_device     # triggers IRQ → schedules tasklet

dmesg

Device File Opened...!!!

Read function

Shared IRQ: Interrupt Occurred     ← ISR (Top Half)

Executing Tasklet Function : arg = 0   ← Tasklet (Bottom Half), data=0

Device File Closed...!!!

Note: arg = 0 here because tasklet_init(tasklet, tasklet_fn, 0) — data was 0. In static method it was 1 from DECLARE_TASKLET(tasklet, tasklet_fn, 1).

Static vs Dynamic — Full Side-by-Side

/* ── STATIC METHOD (Part 20) ── */

/* Declare + init at compile time — global struct on stack/BSS */

DECLARE_TASKLET(tasklet, tasklet_fn, 1);   /* data=1 */

/* Schedule — pass address */

tasklet_schedule(&tasklet);               /* & needed */

/* Kill — pass address, no kfree */

tasklet_kill(&tasklet);

/* ── DYNAMIC METHOD (Part 21) ── */

/* Step 1: Declare pointer globally */

struct tasklet_struct *tasklet = NULL;

/* Step 2: Allocate + init at runtime in init() */

tasklet = kmalloc(sizeof(struct tasklet_struct), GFP_KERNEL);

tasklet_init(tasklet, tasklet_fn, 0);     /* data=0 */

/* Schedule — pass pointer directly */

tasklet_schedule(tasklet);               /* no & needed */

/* Kill — pass pointer, then kfree */

tasklet_kill(tasklet);

kfree(tasklet);                          /* MANDATORY for dynamic only */

Key takeaway: Dynamic method = kmalloc + tasklet_init at runtime. Must kfree in exit — this is the only real difference from the static method. Use dynamic when the tasklet struct lives inside a runtime-allocated device structure.

# Device Driver 22 - Mutex Tutorial

### What is a Race Condition?

When two or more threads access shared data at the same time and try to change it simultaneously, results become unpredictable — this is a race condition.

Real examples:

- Two threads reading/writing the same SPI device → wrong data or crash

- Two threads writing to an LED display → garbled output

- Two threads modifying the same linked list → corruption or crash

### What is a Mutex?

Mutex = Mutual Exclusion Lock — allows only ONE thread to hold the lock at any time.

Key rules:

- Only the thread that locked the mutex can unlock it

- Recursive locking is NOT allowed — same thread cannot lock it twice

- Thread trying to lock a held mutex → sleeps until it's available

- ❌ Cannot be used in interrupt context (it can sleep)

- ❌ Cannot use memset to zero-initialize a mutex

Analogy: A car with only one key — whoever has the key can enter. Others wait until the key is returned.

### Required Header -> #include <linux/mutex.h>

### Mutex Structure

struct mutex {

atomic_t        count;      	/* 1=unlocked, 0=locked, negative=waiters */

spinlock_t      wait_lock;  	/* protects the wait_list                  */

struct list_head wait_list; 	/* threads waiting for this mutex          */

};

### Initialize Mutex — 2 Ways

#### Static Method (for global mutex — compile time)

DEFINE_MUTEX(etx_mutex);

/* Creates AND initializes mutex in one line.

* Use for global/module-level mutexes. */

#### Dynamic Method (for per-object mutex — runtime)

struct mutex etx_mutex;       /* just declare the struct — not initialized */

mutex_init(&etx_mutex);       /* initialize at runtime — must call before use */

/*

* Use when mutex is a field inside a kmalloc'd struct.

* Must be called before any lock/unlock operations.

* Do NOT initialize an already locked mutex.

*/

### Lock the Mutex — 3 Ways

#### mutex_lock() — Standard lock (sleeps until available)

void mutex_lock(struct mutex *lock);

/*

* Acquires mutex exclusively.

* If mutex is held by another thread → current thread SLEEPS until free.

* Cannot be interrupted by signals.

* ❌ Do NOT use in interrupt context — it can sleep!

*/

#### mutex_lock_interruptible() — Lock but allow signal interruption

int mutex_lock_interruptible(struct mutex *lock);

/*

* Same as mutex_lock() but CAN be interrupted by signals.

* Returns: 0 = mutex acquired successfully

*         -EINTR = interrupted by a signal (mutex NOT acquired)

* Check return value — if -EINTR, the mutex was NOT locked!

*/

#### mutex_trylock() — Try to lock without sleeping

int mutex_trylock(struct mutex *lock);

/*

* Tries to acquire mutex WITHOUT waiting/sleeping.

* Returns: 1 = mutex acquired successfully

*          0 = mutex is held by someone else (not acquired)

* ❌ Do NOT use in interrupt context.

*/

### Unlock the Mutex

void mutex_unlock(struct mutex *lock);

/*

* Releases the mutex — MUST be called by the SAME task that locked it.

* Wakes up one waiting thread (if any).

* ❌ Do NOT use in interrupt context.

* ❌ Do NOT unlock a mutex that isn't locked.

*/

### Check Mutex Status

int mutex_is_locked(struct mutex *lock);

/*

* Returns: 1 = mutex is currently locked

*          0 = mutex is unlocked (free)

* Useful for debugging or conditional checks.

*/

### Mutex vs Spinlock — Quick Comparison

|  | Mutex | Spinlock |
| --- | --- | --- |
| Waiting behavior | Sleeps (process yields CPU) | Busy-waits (burns CPU) |
| Can use in ISR? | ❌ NO | ✅ YES |
| Can sleep? | ✅ YES | ❌ NO |
| Use when | Long waits, process context | Very short critical sections |

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Kthread1 Created Successfully...

Kthread2 Created Successfully...

Device Driver Insert...Done!!!

In EmbeTronicX Thread Function1 1    ← Thread1 got mutex first

In EmbeTronicX Thread Function2 2    ← Thread2 got mutex after Thread1 released

In EmbeTronicX Thread Function1 3

In EmbeTronicX Thread Function2 4

...  (alternates every second)

sudo rmmod driver   # → both threads stop

### 3 Lock Functions — When to Use Which?

mutex_lock()              	→ standard, sleeps until available, uninterruptible

Use: most cases

mutex_lock_interruptible()→ sleeps but can be interrupted by signals

Use: when you want Ctrl+C to work

Must check return value for -EINTR!

mutex_trylock()           	→ no sleeping, immediate return

Returns 1=got it, 0=someone else has it

Use: when you have an alternative if lock unavailable

Key takeaway: Always pair mutex_lock() with mutex_unlock() in the same function. Never lock in interrupt context. Always initialize the mutex before threads start. In Linux kernel, there is no mutex_destroy() — just ensure it's unlocked before module unloads.

# Device Driver 23 - Spinlock Tutorial 1

### What is a Spinlock?

A spinlock is a lock where a thread that cannot acquire it keeps trying (spinning) in a loop until it becomes available — instead of sleeping like a mutex.

Two states: Locked or Unlocked — same as mutex.

Analogy: Mutex = you go sit and wait (sleep). Spinlock = you stand at the door knocking repeatedly until it opens.

### Spinlock vs Mutex

|  | Mutex | Spinlock |
| --- | --- | --- |
| Waiting behavior | Sleeps — yields CPU | Spins — burns CPU |
| Use in ISR? | ❌ NO (can sleep) | ✅ YES |
| Best for | Long waits, process context | Very short critical sections |
| Context switch overhead | Yes — goes to sleep | No — stays on CPU |

### When is Spinlock NOT Available?

If the kernel is compiled without CONFIG_SMP and CONFIG_PREEMPT disabled → spinlock is NOT available (no need for it on single-CPU non-preemptible systems).

If CONFIG_SMP disabled but CONFIG_PREEMPT enabled → spinlock simply disables preemption (sufficient to prevent races).

### Required Header

#include <linux/spinlock.h>   /* included via linux/kernel.h usually */

### Initialize Spinlock — 2 Ways

#### Static Method (compile time — most common)

DEFINE_SPINLOCK(etx_spinlock);

/* Creates spinlock_t variable named etx_spinlock, initialized to UNLOCKED.

* Expands to: spinlock_t etx_spinlock = __SPIN_LOCK_UNLOCKED(etx_spinlock) */

#### Dynamic Method (runtime)

spinlock_t etx_spinlock;

spin_lock_init(&etx_spinlock);

/* Use when spinlock is inside a kmalloc'd struct */

### 6 Approaches — Which Lock to Use Where

This is the most important part of this tutorial. Use the right approach based on WHERE your shared data is accessed.

#### Approach 1 — User Context only (Kernel Threads)

spin_lock(&etx_spinlock);      /* lock — spins if unavailable         */

/* critical section */

spin_unlock(&etx_spinlock);    /* unlock                              */

spin_trylock(&etx_spinlock);   /* try lock — returns non-zero if got it,

zero if not — does NOT spin          */

spin_is_locked(&etx_spinlock); /* returns non-zero if locked, 0 if free */

Use when: Shared between two or more kernel threads only.

#### Approach 2 — Between Bottom Halves

Use the same as Approach 1 (spin_lock / spin_unlock).

Use when: Two tasklets or two workqueues share data.

#### Approach 3 — User Context AND Bottom Halves

spin_lock_bh(&etx_spinlock);   /* lock + disable softirqs/tasklets on this CPU */

/* critical section */

spin_unlock_bh(&etx_spinlock); /* unlock + re-enable softirqs */

Use when: A kernel thread AND a tasklet/softirq share data. The _bh suffix = "Bottom Half" — prevents softirqs from preempting while locked.

#### Approach 4 — Hard IRQ and Bottom Halves

spin_lock_irq(&etx_spinlock);   /* lock + disable ALL interrupts on this CPU */

/* critical section */

spin_unlock_irq(&etx_spinlock); /* unlock + re-enable interrupts */

Use when: An ISR (hard IRQ) and a bottom half (tasklet/workqueue) share data. Disabling interrupts prevents the ISR from preempting the bottom half.

#### Approach 5 — Alternative to Approach 4 (Saves IRQ state)

unsigned long flags;

spin_lock_irqsave(&etx_spinlock, flags);   /* lock + save irq state in flags */

/* critical section */

spin_unlock_irqrestore(&etx_spinlock, flags); /* unlock + restore irq state */

Use when: You need to preserve the previous interrupt state (whether interrupts were enabled or disabled before locking). Safer than Approach 4 in nested contexts.

Approach 4 vs Approach 5: Approach 4 always re-enables interrupts on unlock. Approach 5 restores whatever state interrupts were in before locking — safer for nested calls.

#### Approach 6 — Between Hard IRQs (Two ISRs)

Use Approach 5 (spin_lock_irqsave / spin_unlock_irqrestore).

Use when: Two different hardware ISRs share data.

### All 6 Approaches — Quick Reference Table

| Scenario | Lock Function |
| --- | --- |
| Thread ↔ Thread | spin_lock / spin_unlock |
| BH ↔ BH | spin_lock / spin_unlock |
| Thread ↔ BH | spin_lock_bh / spin_unlock_bh |
| IRQ ↔ BH | spin_lock_irq / spin_unlock_irq |
| IRQ ↔ BH (save state) | spin_lock_irqsave / spin_unlock_irqrestore |
| IRQ ↔ IRQ | spin_lock_irqsave / spin_unlock_irqrestore |

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Kthread1 Created Successfully...

Kthread2 Created Successfully...

Spinlock is not locked in Thread Function1   ← before lock

Spinlock is locked in Thread Function1       ← after lock

In EmbeTronicX Thread Function1 1

In EmbeTronicX Thread Function2 2

In EmbeTronicX Thread Function1 3

...

sudo rmmod driver

### Mutex vs Spinlock — Decision Guide

Shared between threads only?

├── Critical section is SHORT (< few microseconds)?  → SPINLOCK

└── Critical section is LONG or needs sleep?         → MUTEX

Shared between ISR and anything?

└── MUST use SPINLOCK (ISR cannot sleep)

├── ISR ↔ BH:        spin_lock_irq

├── ISR ↔ Thread:    spin_lock_irqsave

└── ISR ↔ ISR:       spin_lock_irqsave

Key takeaway: Use spinlock when you need to protect shared data in interrupt context OR when the critical section is extremely short. Use mutex for longer critical sections in process context only. Always pick the right APPROACH (1-6) based on where your lock is used.

# Device Driver 24 - Spinlock Part 2 (Read/Write Spinlock)

### SpinLock

The spinlock is a very simple single-holder lock. If a process attempts to acquire a spinlock and it is unavailable, the process will keep trying (spinning) until it can acquire the lock. Read Write spinlock also does the same but it has separate locks for the read and the write operation.

### Why Read-Write Spinlock?

Problem with regular spinlock: Even if 4 threads all want to read the same variable (no writing), only one can access it at a time — others spin waiting. This wastes performance since reading the same data from multiple threads simultaneously is perfectly safe.

Solution: Use Read-Write Spinlock — allows multiple readers simultaneously, but only one writer at a time, and writing blocks all readers.

### How it Works

State: no thread in critical section

→ Any reader OR writer can enter

State: Reader(s) in critical section

→ New readers CAN enter freely (no blocking)

→ Writers MUST WAIT until all readers leave

State: Writer inside a critical section

→ No readers AND no other writers can enter

→ Everyone else spins and waits

Key insight: Reader gives importance to readers over writers. If you want to give priority to the writer instead, use seqlock (Part 25).

### When to Use

- Data is read far more often than it is written

- Architecture clearly divided into readers and writers

- Multiple reader threads need parallel access to the same data

### Required Header

#include <linux/rwlock.h>   /* usually included via linux/spinlock.h */

### Initialize — 2 Ways

#### Static Method

DEFINE_RWLOCK(etx_rwlock);

/* Creates rwlock_t variable 'etx_rwlock', initialized to UNLOCKED.

Use for global/module-level locks. */

#### Dynamic Method

rwlock_t etx_rwlock;

rwlock_init(&etx_rwlock);

/* Use when rwlock is inside a kmalloc'd struct — call in init() before threads. */

### 6 Approaches — Same Structure as Part 23

#### Approach 1 — User Context only (Kernel Threads)

/* READER */

read_lock(&etx_rwlock);     /* multiple readers can hold this simultaneously */

/* read data */

read_unlock(&etx_rwlock);

/* WRITER */

write_lock(&etx_rwlock);    /* exclusive — blocks all readers and writers     */

/* write data */

write_unlock(&etx_rwlock);

#### Approach 2 — Between Bottom Halves

Use same as Approach 1 (read_lock / write_lock).

#### Approach 3 — User Context AND Bottom Halves

/* Disables softirqs + grabs lock */

read_lock_bh(&etx_rwlock);

read_unlock_bh(&etx_rwlock);

write_lock_bh(&etx_rwlock);

write_unlock_bh(&etx_rwlock);

#### Approach 4 — Hard IRQ and Bottom Halves

/* Disables ALL interrupts + grabs lock */

read_lock_irq(&etx_rwlock);

read_unlock_irq(&etx_rwlock);

write_lock_irq(&etx_rwlock);

write_unlock_irq(&etx_rwlock);

#### Approach 5 — Alternative to Approach 4 (Saves IRQ state)

unsigned long flags;

/* Read */

read_lock_irqsave(&etx_rwlock, flags);

read_unlock_irqrestore(&etx_rwlock, flags);

/* Write */

write_lock_irqsave(&etx_rwlock, flags);

write_unlock_irqrestore(&etx_rwlock, flags);

#### Approach 6 — Between Hard IRQs

Use Approach 5 (irqsave / irqrestore variants).

### All Functions Quick Reference

| Context | Read Lock | Read Unlock | Write Lock | Write Unlock |
| --- | --- | --- | --- | --- |
| Thread ↔ Thread | read_lock | read_unlock | write_lock | write_unlock |
| BH ↔ BH | read_lock | read_unlock | write_lock | write_unlock |
| Thread ↔ BH | read_lock_bh | read_unlock_bh | write_lock_bh | write_unlock_bh |
| IRQ ↔ BH | read_lock_irq | read_unlock_irq | write_lock_irq | write_unlock_irq |
| IRQ ↔ BH (save) | read_lock_irqsave | read_unlock_irqrestore | write_lock_irqsave | write_unlock_irqrestore |
| IRQ ↔ IRQ | read_lock_irqsave | read_unlock_irqrestore | write_lock_irqsave | write_unlock_irqrestore |

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Kthread1 Created Successfully...     ← Writer thread started

Kthread2 Created Successfully...     ← Reader thread started

In EmbeTronicX Thread Function2 : Read value 1   ← reader reads

In EmbeTronicX Thread Function2 : Read value 2   ← reader reads again

...

sudo rmmod driver

### Spinlock vs Read-Write Spinlock — Decision Guide

All accessors do READ + WRITE (mixed)?

→ Use regular SPINLOCK (simpler)

Mostly READERS, rarely one WRITER?

→ Use READ-WRITE SPINLOCK (better performance)

→ Readers don't block each other

→ Writer gets exclusive access

Writer must have priority over readers?

→ Use SEQLOCK (Part 25) — readers never block writers

Key takeaway: Read-Write Spinlock allows unlimited concurrent readers, but only one exclusive writer. Use it when reads significantly outnumber writes. For writer-priority scenarios, use seqlock instead.

# Device Driver 25 - Sending Signals

### What is a Signal?

A signal is a simple asynchronous notification sent to a process to inform it that something happened. Similar to a real-world alert (SMS, doorbell, microwave beep).

Linux examples: SIGINT (Ctrl+C), SIGKILL, SIGSEGV (segfault), etc.

This tutorial: Send a signal FROM kernel driver TO user space application when an interrupt fires.

### Why Send Signals from Driver?

Instead of user app constantly polling the driver (busy-waiting), the driver can notify the app asynchronously when an event happens (interrupt, data ready, hardware event, etc.).

User app waits (blocking) → IRQ fires in kernel → driver sends signal → app wakes and handles it

### 4 Steps to Send Signal from Driver to User Space

#### Step 1 — Define the Signal Number

#define SIGETX  44

/* Use signals 32-64 (real-time signals) for custom driver signals.

* Signals 1-31 are standard (SIGINT=2, SIGKILL=9, etc.) — avoid those.

* 44 is commonly used for driver-to-app signaling. */

#### Step 2 — Register User App with Driver (via IOCTL)

The driver needs to know which process to send the signal to. The app sends its PID implicitly via get_current() during the IOCTL call.

/* IOCTL command to let app register itself with the driver */

#define REG_CURRENT_TASK  _IOW('a', 'a', int32_t*)

/* In driver's ioctl handler: */

static long etx_ioctl(struct file *file, unsigned int cmd, unsigned long arg)

{

if (cmd == REG_CURRENT_TASK) {

task = get_current();   /* get the calling process's task_struct */

signum = SIGETX;        /* remember which signal to send         */

}

return 0;

}

- get_current() returns a pointer to the task_struct of the calling process

- The driver stores this pointer to know which process to signal later

#### Step 3 — Send Signal from ISR (or wherever event occurs)

struct kernel_siginfo info;

memset(&info, 0, sizeof(struct siginfo));

info.si_signo = SIGETX;    /* signal number to send  */

info.si_code  = SI_QUEUE;  /* sent via sigqueue() mechanism */

info.si_int   = 1;         /* data value passed with signal */

send_sig_info(SIGETX, &info, task);   /* send signal to registered process */

| Argument | Meaning |
| --- | --- |
| SIGETX | Signal number to send (44) |
| &info | Signal info struct with data |
| task | task_struct* of target process |

Returns < 0 on failure — always check!

#### Step 4 — Unregister App on Close

static int etx_release(struct inode *inode, struct file *file)

{

if (get_current() == task) {

task = NULL;   /* clear stored task — prevent signal to dead process */

}

return 0;

}

### Key Kernel Functions

| Function | Purpose |
| --- | --- |
| get_current() | Returns task_struct* of the currently running process |
| send_sig_info(sig, info, task) | Sends signal with info to the target task |
| memset(&info, 0, sizeof(...)) | Zero out kernel_siginfo before filling it |

### struct siginfo Fields

struct kernel_siginfo {

int si_signo;  	/* signal number (e.g., 44)     */

int si_code;   	/* SI_QUEUE = queued signal       */

int si_int;    	/* integer data sent with signal  */

/* other fields... */

};

### User Space App — How to Receive the Signal

#### Install Signal Handler

#include <signal.h>

struct sigaction act;

/* Handler function */

void sig_event_handler(int n, siginfo_t *info, void *unused)

{

if (n == SIGETX) {

int val = info->si_int;   /* data sent by driver */

printf("Received signal from kernel, value = %d\n", val);

}

}

/* Install handler for our custom signal  */

sigemptyset(&act.sa_mask);

act.sa_flags = (SA_SIGINFO | SA_RESTART);

act.sa_sigaction = sig_event_handler;

sigaction(SIGETX, &act, NULL);

| sa_flags | Meaning |
| --- | --- |
| SA_SIGINFO | Use sa_sigaction (not sa_handler) — allows extra info |
| SA_RESTART | Restart interrupted system calls automatically |
| SA_RESETHAND | Reset handler to default after first signal (one-shot) |

#### Register with Driver via IOCTL

ioctl(fd, REG_CURRENT_TASK, (int32_t*) &number);

/* Sends the app's PID/task to the driver via IOCTL.

* Driver calls get_current() internally to store task_struct. */

### Build & Test Commands

# Build driver

sudo make

# Build user app

gcc -o test_app test_app.c

# Load driver

sudo insmod driver.ko

# Run app (in terminal 1 — waits for signal)

sudo ./test_app

# In another terminal:

sudo cat /dev/etx_device # triggers etx_read → sends signal

# Check kernel log

dmesg

Expected output — test_app terminal:

Installed signal handler for SIGETX = 44

Opening Driver

Registering application...Done!!!

Waiting for signal...

Received signal from kernel : Value = 1    ← after cat command

Expected dmesg:

Device Driver Insert...Done!!!

REG_CURRENT_TASK        ← app registered

Read Function           ← cat triggered read

Shared IRQ: Interrupt Occurred

Sending signal to app   ← signal sent

Key takeaway: The driver uses get_current() in IOCTL to capture the app's task_struct. The tex_read( ) uses send_sig_info() to deliver the signal. Always set task = NULL on release to prevent signaling a dead process. Use real-time signals (32–64) for custom driver signals — never use standard signals (1–31).

# Device Driver 26 - Kernel Timer Tutorial

### What is a Kernel Timer?

A kernel timer schedules a function to run at a specific time in the future, based on the system clock tick (jiffies). It is not periodic by default — you must re-arm it manually inside the callback if you want it to repeat.

Timer callbacks run in interrupt context (softirq) — same rules as ISR apply.

## Uses of Kernel Timers

- Polling a device at regular intervals when hardware can't generate interrupts.

- Sending messages to another device at regular intervals.

- Detecting timeouts — trigger an error if an expected action doesn't happen in time.

### Key Concepts

#### Jiffies – ticks since boot

A jiffy represents one tick of the kernel timer interrupt.

jiffies≈uptime_seconds×HZ

uptime in seconds - cat /proc/uptime

HZ - getconf CLK_TCK

jiffies = 1234 seconds x 100 hz

The kernel internal clock counter incremented by every timer interrupt (tick). HZ = ticks per second (e.g., 250 or 1000 depending on config).

#### Convert milliseconds to jiffies

msecs_to_jiffies(ms)    /* converts milliseconds → jiffies */

jiffies + msecs_to_jiffies(5000)  /* = "5 seconds from now" */

# From /proc/timer_list (closest to raw kernel jiffies)

On many systems:

> cat /proc/timer_list | grep jiffies

Example output: jiffies: 4298372821

This is the actual kernel jiffies counter.

Usually requires root:

sudo cat /proc/timer_list

### Required Headers

#include <linux/timer.h>    /* all timer APIs       */

#include <linux/jiffies.h>  /* jiffies, msecs_to_jiffies */

### Timer Structure

struct timer_list {

unsigned long  expires;              /* expiry time in jiffies            */

void (*function)(struct timer_list *); /* callback function (newer kernel) */

unsigned long  data;                 /* argument to callback (older kernel)*/

};

### Initialize — 4 Ways

#### 1. init_timer() — Low level (manual setup)

struct timer_list etx_timer;

init_timer(&etx_timer);

etx_timer.function = timer_callback;   /* set callback manually */

etx_timer.data     = 0;                /* set data manually     */

etx_timer.expires  = jiffies + msecs_to_jiffies(5000);

#### 2. setup_timer() — Older kernel (< 4.15)

setup_timer(&etx_timer, timer_callback, data);

/* callback function arg type: unsigned long */

void timer_callback(unsigned long data) { }

#### 3. timer_setup() — Newer kernel (>= 4.15) ✅ Recommended

timer_setup(&etx_timer, timer_callback, 0);

/* callback function arg type: struct timer_list *   */

void timer_callback(struct timer_list *data) { }

#### 4. DEFINE_TIMER() — Compile time (all-in-one)

DEFINE_TIMER(_name, _function, _expires, _data);

/* Creates + initializes timer_list struct in one macro */

_name – name of the timer_list structure to be created
_function – Callback function to be called when the timer expires
_expires – the expiration time of the timer (in jiffies)
_data – data has to be given to the callback function

### Start the Timer

add_timer(&etx_timer);

/* Starts/queues the timer. Fires once at etx_timer.expires.

* Timer expires = jiffies value set via mod_timer or manually. */

### Modify Timer Timeout

mod_timer(&etx_timer, jiffies + msecs_to_jiffies(TIMEOUT));

/*

* Update expiry time — most commonly used function.

* Works on both ACTIVE and INACTIVE timers.

* If timer is inactive → activates it too.

* Equivalent to: del_timer + set expires + add_timer (in one atomic step)

*

* Returns: 0 if timer was inactive, 1 if timer was active (pending)

*/

### Stop the Timer

del_timer(&etx_timer);

/* Deactivates timer immediately.

* Returns: 0=was inactive, 1=was active.

* Does NOT wait for callback to finish if running. */

del_timer_sync(&etx_timer);

/* Deactivates timer AND waits for callback to complete.

* Safer — use in exit() to ensure callback is not running when module unloads.

* ❌ Cannot call from interrupt context.

* ❌ Do not call if timer callback re-arms itself (restarts timer). */

### Check Timer Status

timer_pending(&etx_timer);

/* Returns: 1 if timer is queued (pending), 0 if not pending */

## How to Make a Periodic Timer

Kernel timer fires only ONCE by default. To make it periodic → re-arm inside callback:

void timer_callback(struct timer_list *data)

{

/* do your work here */

mod_timer(&etx_timer, jiffies + msecs_to_jiffies(TIMEOUT)); /* re-arm  the timer*/

}

### Timer Callback Context Rules

Timer callback runs in interrupt context (softirq). Same rules as ISR:

❌ NO sleep / yield CPU

❌ NO mutex

❌ NO time-consuming tasks

❌ NO user space memory access

✅ CAN use spinlock

✅ CAN call schedule_work() to defer heavy work

### Use del_timer_sync() vs del_timer() in exit

|  | del_timer() | del_timer_sync() |
| --- | --- | --- |
| Stops timer | ✅ | ✅ |
| Waits for running callback | ❌ | ✅ |
| Use in interrupt context | ✅ | ❌ |
| Safer for module exit | ❌ | ✅ |

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Device Driver Insert...Done!!!

Timer Callback function Called [0]    ← after 5 seconds

Timer Callback function Called [1]    ← after 10 seconds

Timer Callback function Called [2]    ← after 15 seconds

...                                   ← every 5 seconds

sudo rmmod driver   # → del_timer() called → timer stops

### All Timer APIs — Quick Reference

/* Initialize */

timer_setup(&timer, callback, 0);       /* newer kernel >= 4.15 ✅ */

setup_timer(&timer, callback, data);    /* older kernel < 4.15    */

/* Start */

add_timer(&timer);                      /* starts timer at timer.expires */

mod_timer(&timer, jiffies + delay);     /* arm/rearm with new expiry ✅  */

/* Stop */

del_timer(&timer);                      /* stop immediately              */

del_timer_sync(&timer);                 /* stop + wait for callback ✅   */

/* Check */

timer_pending(&timer);                  /* 1=pending, 0=not pending      */

/* Time conversion */

msecs_to_jiffies(ms);                   /* ms → jiffies                  */

jiffies_to_msecs(jiffies);             /* jiffies → ms                  */

Key takeaway: Kernel timers fire once only by default — re-arm in callback for periodic behavior. Timer callback runs in interrupt context — no sleeping, no mutex. Always call del_timer() or del_timer_sync() in exit to prevent callback from firing after module unloads.

# Device Driver 27 -High Resolution Timer Tutorial

### Why High Resolution Timer (hrtimer)?

Kernel Timer (Part 26) uses jiffies — resolution limited by HZ (e.g., 1ms to 10ms granularity).

hrtimer uses nanoseconds resolution — much more precise for time-critical drivers.

They cannot be merged because the kernel timer uses a Cascading Timer Wheel (CTW) structure, while hrtimer uses a red-black tree (ordered by time for fast insertion and lookup). These two data structures are fundamentally incompatible.

### hrtimer vs Kernel Timer

|  | Kernel Timer (Part 26) | hrtimer (Part 27) |
| --- | --- | --- |
| Resolution | Jiffies (ms range) | Nanoseconds |
| Available since | Always | Kernel 2.6.21+ |
| Data structure | Cascading Timer Wheel | Red-Black Tree |
| API | timer_setup, mod_timer | hrtimer_init, hrtimer_start |
| Use when | General deferred work | Precise timing needed |

### Check if hrtimer is Available

# Check kernel config

cat /boot/config-$(uname -r) | grep CONFIG_HIGH_RES_TIMERS

# Should show: CONFIG_HIGH_RES_TIMERS=y

# Check proc timer list

cat /proc/timer_list | grep resolution

# Should show: .resolution: 1 nsecs

### Required Headers

#include <linux/hrtimer.h>   /* all hrtimer APIs         */

#include <linux/ktime.h>     /* ktime_t, ktime_set()     */

### hrtimer Structure

struct hrtimer {

struct rb_node       node;      /* red-black tree node — ordered by time    */

ktime_t              expires;   	/* expiry time in nanoseconds (absolute)     */

enum hrtimer_restart (*function)(struct hrtimer *); /* callback function     */

struct hrtimer_base  *base;     /* per-CPU, per-clock timer base             */

};

### ktime_t — Nanosecond Time Type

/* 64-bit systems: ktime_t = 64-bit integer in nanoseconds

32-bit systems: ktime_t = struct { 32-bit seconds, 32-bit nanoseconds } */

ktime_t ktime_set(long secs, long nanosecs);

/* Converts seconds + nanoseconds → ktime_t value

* Example: ktime_set(4, 1000000000) = 4 sec + 1 sec = 5 seconds total */

### 2 Clock Types for hrtimer

| Clock | Description |
| --- | --- |
| CLOCK_MONOTONIC | Always moves forward from system boot. Does NOT reflect wall clock. Like jiffies but in nanoseconds. ✅ Most commonly used |
| CLOCK_REALTIME | Matches real-world wall clock time. Can jump backward (NTP adjustments). |

### 2 Timer Modes

| Mode | Meaning |
| --- | --- |
| HRTIMER_MODE_ABS | Expires at an absolute time |
| HRTIMER_MODE_REL | Expires after a relative delay from NOW |

### Initialize

void hrtimer_init(struct hrtimer *timer, clockid_t clock_id, enum hrtimer_mode mode);

/* Sets up hrtimer but does NOT start it.

* Set timer.function manually AFTER calling this. */

timer – the timer to be initialized

clock_id – the clock to be used (CLOCK_MONOTONIC or CLOCK_REALTIME)

mode – timer mode absolute (HRTIMER_MODE_ABS) or relative (HRTIMER_MODE_REL)

hrtimer_init(&etx_hr_timer, CLOCK_MONOTONIC, HRTIMER_MODE_REL);

etx_hr_timer.function = &timer_callback;   /* set callback after init */

### Start

int hrtimer_start(struct hrtimer *timer, ktime_t time, const enum hrtimer_mode mode);

/* Arms and starts the timer.

* Returns: 0=success, 1=timer was already active */

timer – the timer to be added

time   – expiry time

mode – expiry mode: absolute (HRTIMER_MODE_ABS) or relative (HRTIMER_MODE_REL)

hrtimer_start(&etx_hr_timer, ktime, HRTIMER_MODE_REL);

### Stop

int hrtimer_cancel(struct hrtimer *timer);

/* Cancels timer AND waits for callback to finish.

* Returns: 0=was not active, 1=was active.

* Use in exit() — safest way to stop timer. */

int hrtimer_try_to_cancel(struct hrtimer *timer);

/* Tries to cancel — does NOT wait.

* Returns: 0=not active, 1=active+cancelled, -1=callback running (cannot cancel) */

### Callback Return Values — Key Difference from Kernel Timer

/* Kernel timer callback: void — no return value, re-arm via mod_timer()

hrtimer callback:       enum hrtimer_restart — return value controls behavior */

enum hrtimer_restart timer_callback(struct hrtimer *timer)

{

/* one-shot: just return */

return HRTIMER_NORESTART;

/* periodic: forward expiry time, then restart */

hrtimer_forward_now(timer, ktime_set(SECS, NSECS));

return HRTIMER_RESTART;

}

### Forward Timer (for periodic behavior)

/* Forward from specific time */

u64 hrtimer_forward(struct hrtimer *timer, ktime_t now, ktime_t interval);

timer – hrtimer to forward

now – forward past this time

interval – the interval to forward

/* Forward from NOW (most common) */

u64 hrtimer_forward_now(struct hrtimer *timer, ktime_t interval);

/* Returns: number of overruns (missed intervals) */

timer – hrtimer to forward

interval – the interval to forward

### Check Status

ktime_t hrtimer_get_remaining(const struct hrtimer *timer);

/* Returns remaining time before expiry */

int hrtimer_callback_running(struct hrtimer *timer);

/* Returns: 1=callback currently running, 0=not running */

ktime_t hrtimer_cb_get_time(struct hrtimer *timer);

/* Returns current time of the timer's clock */

### Callback Context Rules (Same as Kernel Timer)

Runs in interrupt context (softirq):

❌ NO sleep   ❌ NO mutex   ❌ NO user space access

✅ CAN use spinlock

✅ CAN call schedule_work() to defer heavy work

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Device Driver Insert...Done!!!

Timer Callback function Called [0]    ← after 5 seconds

Timer Callback function Called [1]    ← after 10 seconds

Timer Callback function Called [2]    ← after 15 seconds

...                                   ← every 5 seconds precisely

sudo rmmod driver   # → hrtimer_cancel() called → timer stops

### Kernel Timer vs hrtimer — All APIs Side-by-Side

/* ── Kernel Timer (jiffies-based) ── */

struct timer_list timer;

timer_setup(&timer, callback, 0);        /* init              */

mod_timer(&timer, jiffies + delay);      /* start/rearm       */

del_timer_sync(&timer);                  /* stop (safe)       */

/* callback: void fn(struct timer_list*) → mod_timer() to repeat */

/* ── hrtimer (nanosecond-based) ── */

struct hrtimer timer;

hrtimer_init(&timer, CLOCK_MONOTONIC, HRTIMER_MODE_REL);

timer.function = callback;              /* set callback after init */

hrtimer_start(&timer, ktime, HRTIMER_MODE_REL);  /* start    */

hrtimer_cancel(&timer);                           /* stop (safe) */

/* callback: enum hrtimer_restart fn(struct hrtimer*)

→ hrtimer_forward_now() + HRTIMER_RESTART to repeat */

Key takeaway: hrtimer = nanosecond precision, uses red-black tree, callback must return HRTIMER_RESTART or HRTIMER_NORESTART. Always call hrtimer_forward_now() before returning HRTIMER_RESTART to avoid immediate re-firing if callback ran late.

# Device Driver 28 - Completion Tutorial

### What is Completion?

Completion is a synchronization mechanism used to notify/wake up a sleeping thread when a specific task finishes. It's like a "done signal" — one part of the code waits, another part signals when ready.

Built on top of waitqueue and wakeup infrastructure of the Linux scheduler.— but with a cleaner, focused API specifically for "wait for event to complete" use cases.

Real analogy: Thread A starts a job and sleeps. Thread B finishes the job and rings a bell (complete). Thread A wakes up and continues.

### Why Completion over Waitqueue / Mutex?

| Mechanism | Issue |
| --- | --- |
| Mutex | Designed for mutual exclusion, not event notification |
| Busy-loop | Wastes CPU |
| Waitqueue | Works but requires manual flag management and is error-prone |
| Completion | ✅ Clean, focused, race-condition-free event notification |

### Required Header -> #include <linux/completion.h>

### Completion Structure

struct completion {

unsigned int      done;    /* 0 = not completed, non-zero = completed     */

wait_queue_head_t wait;    /* waitqueue — threads sleeping here           */

};

### Step 1 — Initialize

#### Static Method (compile time — simpler)

DECLARE_COMPLETION(data_read_done);

/* Creates + initializes completion struct named 'data_read_done'.

* done = 0 (not completed), waitqueue initialized. */

#### Dynamic Method (runtime — for per-object use)

struct completion data_read_done;   /* declare struct */

init_completion(&data_read_done);   /* initialize in init() */

/* done = 0, waitqueue initialized. Must call BEFORE thread starts. */

### Step 2 — Re-initialize (if reusing)

reinit_completion(&data_read_done);

/* Resets done = 0 ("not completed") WITHOUT touching the waitqueue.

* Use after complete_all() to reuse the completion.

* ⚠️ No racing wait_for_completion() calls should be active when called. */

### Step 3 — Wait Functions (Thread Sleeps Here)

void wait_for_completion (struct completion * x);

x – holds the state of this particular completion

unsigned long wait_for_completion_timeout (struct completion * x, unsigned long timeout);

x – holds the state of this particular completion
timeout – timeout value in jiffies
returns 0 if timed out, and positive for remaining jiffies left till timeout

int wait_for_completion_interruptible (struct completion * x)

return -ERESTARTSYS if interrupted, 0 if completed.

long wait_for_completion_interruptible_timeout (struct completion * x, unsigned long timeout);

x – holds the state of this particular completion
timeout – timeout value in jiffies
returns -ERESTARTSYS if interrupted, 0 if timed out, positive if remaining

int wait_for_completion_killable (struct completion * x);

returns -ERESTARTSYS if interrupted, 0 if completed.

long wait_for_completion_killable_timeout (struct completion * x, unsigned long timeout);

returns -ERESTARTSYS if interrupted, 0 if timed out, positive if remaining

bool try_wait_for_completion (struct completion * x);

returns 0 if completion is not available 1 if a got it succeeded.

| Function | Interruptible? | Timeout? | Returns |
| --- | --- | --- | --- |
| wait_for_completion(&c) | ❌ No | ❌ No | void |
| wait_for_completion_timeout(&c, timeout) | ❌ No | ✅ Yes | 0=timeout, positive=remaining jiffies |
| wait_for_completion_interruptible(&c) | ✅ Yes (signal) | ❌ No | 0=ok, -ERESTARTSYS=interrupted |
| wait_for_completion_interruptible_timeout(&c, timeout) | ✅ Yes | ✅ Yes | 0=timeout, positive=remaining, -ERESTARTSYS=interrupted |
| wait_for_completion_killable(&c) | ✅ Kill signal only | ❌ No | 0=ok, -ERESTARTSYS=killed |
| wait_for_completion_killable_timeout(&c, timeout) | ✅ Kill only | ✅ Yes | 0=timeout, positive=remaining, -ERESTARTSYS=killed |
| try_wait_for_completion(&c) | — | — | 0=not ready (would block), 1=got it (no blocking) |

try_wait_for_completion() is safe in IRQ/atomic context — others are NOT.

### Step 4 — Wake Up Functions

void complete (struct completion * x);
This will wake up a single thread waiting on this completion. Threads will be awakened in the same order in which they were queued.

complete(&data_read_done);

/* Wakes ONE waiting thread (FIFO order).

* Increments done counter — if no waiter, next wait_for_completion returns immediately. */

void complete_all (struct completion * x);

This will wake up all threads waiting on this particular completion event.

complete_all(&data_read_done);

/* Wakes ALL waiting threads simultaneously.

* Use reinit_completion() to reuse after this. */

### Step 5 — Check Status

bool completion_done (struct completion * x);

bool completion_done(&data_read_done);

/* Returns: 0 = waiters are present (completion is in progress)

1 = no waiters

* Safe to call in IRQ/atomic context. */

### Completion vs Waitqueue — Key Difference

|  | Waitqueue | Completion |
| --- | --- | --- |
| Purpose | Generic sleeping | Specific event notification |
| Race-free? | Needs careful flag management | ✅ Built-in race-free design |
| API complexity | More code needed | Focused, simple API |
| Built on | Scheduler | Waitqueue |

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Thread Created successfully

Device Driver Insert...Done!!!

Waiting For Event...         ← thread sleeping

sudo cat /dev/etx_device     # send complete from read

dmesg

Device File Opened...!!!

Read Function

Event Came From Read Function - 1   ← thread woke from read

Waiting For Event...                ← thread sleeping again

Device File Closed...!!!

sudo rmmod driver            # send complete from exit

dmesg

Event Came From Exit Function       ← thread woke from exit, returns

Device Driver Remove...Done!!!

## Dynamic Method (Only Differences Shown)

/* ── ONLY DIFFERENCES FROM STATIC METHOD───────────────── */

/* STATIC:  DECLARE_COMPLETION(data_read_done);   ← compile time */

/* DYNAMIC: declare struct only — NOT initialized yet */

struct completion data_read_done;

cd ../

/* In etx_driver_init() — BEFORE kthread_create(): */

/*

* init_completion(&data_read_done):

*   Sets done = 0 ("not completed")

*   Initializes the internal waitqueue

*   MUST be called before thread starts — thread uses it in wait_for_completion()

*

* ⚠️ Original tutorial places this AFTER kthread_create() — that is a bug.

*    Always init BEFORE thread creation.

*/

init_completion(&data_read_done);   /* ← place BEFORE kthread_create() */

wait_thread = kthread_create(wait_function, NULL, "WaitThread");

/* ... rest identical to static method ... */

### Static vs Dynamic — Side-by-Side

/* ── STATIC ── */

DECLARE_COMPLETION(data_read_done);    /* one line, compile time, global */

/* No init step in init() needed */

/* ── DYNAMIC ── */

struct completion data_read_done;      /* declare struct */

init_completion(&data_read_done);      /* init at runtime — BEFORE thread starts */

### Completion vs Waitqueue

/* WAITQUEUE (Part 10) — manual condition management */

wait_queue_flag = 1;                        /* set condition */

wake_up_interruptible(&wait_queue_etx);     /* wake */

/* in thread: wait_event_interruptible(wq, wait_queue_flag != 0) */

/* COMPLETION (Part 28) — cleaner, race-free */

completion_flag = 1;                        /* why we woke */

complete(&data_read_done);                  /* wake */

/* in thread: wait_for_completion(&data_read_done) */

/* KEY advantage of completion:

If complete() fires BEFORE wait_for_completion() → event is NOT lost!

Waitqueue: if wake_up fires before wait_event → event IS lost! */

Key takeaway: Completion is a cleaner, race-free alternative to waitqueue for "wait for event" use cases. The main advantage is that complete() called before wait_for_completion() is never lost — the next wait_for_completion() returns immediately. Always initialize (init_completion) before thread starts. Use complete() to wake one waiter, complete_all() to wake all.

# Device Driver 29 – EXPORT_SYMBOL

### What is a Symbol?

In programming, a symbol is either a variable or a function — a name that represents a memory location storing data or executable instructions.

### Why EXPORT_SYMBOL?

When you have multiple kernel modules and want Module B to use a function or variable defined in Module A, you cannot simply use extern. You must explicitly export the symbol from Module A so the kernel knows to make it available to other modules.

Example: printk() is defined in kernel/printk/printk.c. It is available in YOUR driver because it was exported using EXPORT_SYMBOL(printk)

### History

| Kernel Version | Behavior |
| --- | --- |
| Linux 2.4 | ALL non-static symbols automatically exported |
| Linux 2.6+ | ONLY symbols marked with EXPORT_SYMBOL() are exported |

### 2 Export Macros

EXPORT_SYMBOL(symbol_name);

/* Exports symbol to ANY loadable kernel module — no license restriction */

EXPORT_SYMBOL_GPL(symbol_name);

/* Exports symbol ONLY to GPL-licensed modules — restricted sharing */

### 3 Steps to Use EXPORT_SYMBOL

#### Step 1 — In the exporting module (Module A / driver1.c)

/* Define the variable and function */

int etx_count = 0;

void etx_shared_func(void) {

pr_info("Shared function called!\n");

etx_count++;

}

/* Export both — makes them visible to all other modules */

EXPORT_SYMBOL(etx_shared_func);

EXPORT_SYMBOL(etx_count);

#### Step 2 — In the consuming module (Module B / driver2.c)

/* Declare them with extern — tells compiler they're defined elsewhere */

extern int etx_count;

void etx_shared_func(void);   /* function declarations are extern by default */

/* Then use them directly */

etx_shared_func();

pr_info("Count = %d\n", etx_count);

#### Step 3 — Load Order (CRITICAL)

sudo insmod driver1.ko   # FIRST — defines the exported symbols

sudo insmod driver2.ko   # SECOND — uses the exported symbols

❌ Loading driver2 first → insmod: ERROR: Unknown symbol in module

### Limitations

- ❌ Symbol must NOT be static or inline — static means module-local only

- ❌ Load order matters — exporting module MUST be loaded first

- ❌ Unload order matters — consuming module MUST be unloaded first

### Verifying Exported Symbols

#### After sudo make — check >cat Module.symvers:

0x1db7034a   etx_shared_func   /path/driver1   EXPORT_SYMBOL

0x6dcb135c   etx_count         /path/driver1   EXPORT_SYMBOL

#### After loading — check kernel symbol table:

cat /proc/kallsyms | grep etx_shared_func   # should appear

cat /proc/kallsyms | grep etx_count          # should appear

### Test Commands & Expected Output

sudo make

sudo insmod driver1.ko   # load exporter first

sudo insmod driver2.ko   # load consumer second

dmesg

Major = 246 Minor = 0

Device Driver 1 Insert...Done!!!

Major = 245 Minor = 0

Device Driver 2 Insert...Done!!!

sudo cat /dev/etx_device2   # trigger read in driver2

dmesg

Device File Opened...!!!

Shared function been called!!!       ← driver1's function called from driver2

1 time(s) shared function called!    ← driver1's variable read from driver2

Data Read : Done!

Device File Closed...!!!

sudo rmmod driver2   # unload consumer FIRST

sudo rmmod driver1   # then unload exporter

❌ Removing driver1 first → rmmod: ERROR: Module driver1 is in use llby driver2

### Makefile

# Build BOTH modules from same Makefile

obj-m += driver1.o

obj-m += driver2.o

KDIR = /lib/modules/$(shell uname -r)/build

all:

make -C $(KDIR) M=$(shell pwd) modules

clean:

make -C $(KDIR) M=$(shell pwd) clean

### Complete Flow Summary

sudo make

└── Generates Module.symvers:

etx_shared_func  driver1  EXPORT_SYMBOL

etx_count        driver1  EXPORT_SYMBOL

sudo insmod driver1.ko

└── etx_count, etx_shared_func added to /proc/kallsyms

sudo insmod driver2.ko

└── Kernel resolves extern symbols from driver1's exports

└── /dev/etx_device2 created

sudo cat /dev/etx_device2

└── etx_read() in driver2

├── etx_shared_func()      → runs function FROM driver1

│     └── prints "Shared function been called!!!"

│     └── etx_count++ → etx_count = 1

└── pr_info("1 time(s) shared function called!")  → reads etx_count FROM driver1

sudo rmmod driver2   # consumer FIRST

sudo rmmod driver1   # exporter LAST

### EXPORT_SYMBOL vs extern — What Each Does

EXPORT_SYMBOL(func) — in driver1.c:

→ Puts function into kernel's global symbol table

→ Makes it discoverable by other modules at insmod time

extern func         — in driver2.c:

→ Tells compiler "defined elsewhere, don't error"

→ At insmod time, kernel links it to driver1's actual address

Key takeaway: EXPORT_SYMBOL in the provider + extern in the consumer. Load order: provider first, consumer second. Unload order: consumer first, provider last. Symbol must never be static or inline. Use EXPORT_SYMBOL_GPL when you want to restrict usage to GPL-only modules.

# Device Driver 30 - Atomic Variables Tutorial

### Why Atomic?

When two threads share a plain integer and both increment it:

Thread1: reads value(0) → increments(1) → writes back(1)

Thread2: reads value(0) → increments(1) → writes back(1)

Result = 1  ← WRONG! Expected 2 (race condition!)

Normal fix: Use mutex or spinlock. But for a single integer variable, this adds unnecessary overhead.

Better fix: Use atomic variables — operations happen in ONE machine instruction, no interruption possible, no lock needed.

Thread1: get+increment+store(0→1) in ONE step

Thread2: get+increment+store(1→2) in ONE step

Result = 2  ← CORRECT!

### What is an Atomic Variable?

A special kernel integer type where read, write, and arithmetic all happen in one uninterruptible CPU instruction — no mutex or spinlock needed. . They ensure thread safety and data consistency in multi-threaded applications without using explicit locks or synchronization, offering higher performance via non-blocking, lock-free mechanisms

an atomic variable is a special data type designed for thread-safe operations without the overhead of heavy locking mechanisms like mutexes or spinlocks.

### Required Headers

#include <asm/atomic.h>     /* atomic integer operations  */

#include <asm/bitops.h>     /* atomic bit operations      */

### 2 Types of Atomic Variables

#### Type 1 — Atomic Integer (atomic_t / atomic64_t)

atomic_t    → holds int   (32-bit)

atomic64_t  → holds long  (64-bit, only on 64-bit systems)

Internal structure (modern kernel):

typedef struct { int counter; } atomic_t;

typedef struct { long counter; } atomic64_t;

/* Note: volatile was removed from counter in newer kernels */

### Declare & Initialize

atomic_t etx_var;                        /* declare only — uninitialized    */

atomic_t etx_var = ATOMIC_INIT(0);       /* declare + initialize to 0       */

atomic64_t etx_var64 = ATOMIC64_INIT(0); /* 64-bit version, declare + initialize to 0*/

### All Atomic Integer Operations

#### Read & Set

int  atomic_read(atomic_t *v);           /* read value                       */

void atomic_set(atomic_t *v, int i);     /* set value to i                   */

#### Add & Subtract

void atomic_add(int i, atomic_t *v);     /* v += i                           */

void atomic_sub(int i, atomic_t *v);     /* v -= i                           */

#### Increment & Decrement

void atomic_inc(atomic_t *v);            /* v++                              */

void atomic_dec(atomic_t *v);            /* v--                              */

#### Test-and-Operate (returns bool)

int atomic_sub_and_test(int i, atomic_t *v);  /* v-=i; return true if (v==0)         */

int atomic_dec_and_test(atomic_t *v);           /* v--; return true if  (v==0)         */

int atomic_inc_and_test(atomic_t *v);            /* v++; return true if (v==0)         */

int atomic_add_negative(int i, atomic_t *v);   /* v+=i; return true if (v<0)         */

#### Operate and Return Value

int atomic_add_return(int i, atomic_t *v);   /* v+=i; return new v           */

int atomic_sub_return(int i, atomic_t *v);   /* v-=i; return new v           */

int atomic_inc_return(atomic_t *v);             /* v++; return new v            */

int atomic_dec_return(atomic_t *v);            /* v--; return new v            */

#### Conditional Add

atomic_add_unless(atomic_t *v, int a, int u);

/* if (v != u) then v += a

* Returns: non-zero if v was NOT u (add happened), 0 if v was u (skipped) */

v – the pointer of type atomic_t

a – the amount to add to v…

u – …unless v is equal to u.

### Type 2 — Atomic Bit Operations

Works on a generic pointer (not atomic_t) — operates on individual bits of any variable. Architecture-dependent, declared in <asm/bitops.h>.

#### Atomic Bit Functions

| Function | Description |
| --- | --- |
| void set_bit(int nr, void *addr) | Atomically set the nr-th bit starting from addr |
| void clear_bit(int nr, void *addr) | Atomically clear the nr-th bit starting from addr |
| void change_bit(int nr, void *addr) | Atomically flip the value of the nr-th bit starting from addr |
| int test_and_set_bit(int nr, void *addr) | Atomically set the nr-th bit starting from addr and return the previous value |
| int test_and_clear_bit(int nr, void *addr) | Atomically clear the nr-th bit starting from addr and return the previous value |
| int test_and_change_bit(int nr, void *addr) | Atomically flip the nr-th bit starting from addr and return the previous value |
| int test_bit(int nr, void *addr) | Atomically return the value of the nr-th bit starting from addr |
| int find_first_zero_bit(unsigned long *addr, unsigned int size) | Atomically returns the bit-number of the first zero bit, not the number of the byte containing a bit |
| int find_first_bit(unsigned long *addr, unsigned int size) | Atomically returns the bit-number of the first set bit, not the number of the byte containing a bit |

| Function | Description |
| --- | --- |
| set_bit(nr, addr) | Atomically set bit nr |
| clear_bit(nr, addr) | Atomically clear bit nr |
| change_bit(nr, addr) | Atomically flip bit nr |
| test_and_set_bit(nr, addr) | Set bit, return old value |
| test_and_clear_bit(nr, addr) | Clear bit, return old value |
| test_and_change_bit(nr, addr) | Flip bit, return old value |
| test_bit(nr, addr) | Return current value of bit |
| find_first_zero_bit(addr, size) | Find first 0 bit position |
| find_first_bit(addr, size) | Find first 1 bit position |

#### Non-Atomic Bit Functions (faster inside locked sections)

Same as above but with _ prefix: _set_bit(), _clear_bit(), _change_bit(), etc.

| Function | Description |
| --- | --- |
| void _set_bit(int nr, void *addr) | Non-atomically set the nr-th bit starting from addr |
| void _clear_bit(int nr, void *addr) | Non-atomically clear the nr-th bit starting from addr |
| void _change_bit(int nr, void *addr) | Non-atomically flip the value of the nr-th bit starting from addr |
| int _test_and_set_bit(int nr, void *addr) | Non-atomically set the nr-th bit starting from addr and return the previous value |
| int _test_and_clear_bit(int nr, void *addr) | Non-atomically clear the nr-th bit starting from addr and return the previous value |
| int _test_and_change_bit(int nr, void *addr) | Non-atomically flip the nr-th bit starting from addr and return the previous value |
| int _test_bit(int nr, void *addr) | Non-atomically return the value of the nr-th bit starting from addr |

Use non-atomic versions when already inside a mutex/spinlock — they're faster since the lock already ensures safety.

### When to Use What

| Situation | Use |
| --- | --- |
| Single integer shared between threads | atomic_t |
| Single bit flag shared between threads | test_and_set_bit() etc. |
| Multiple variables shared between threads | mutex or spinlock |
| Integer operations inside a locked section | regular int (lock provides safety) |

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Kthread1 Created Successfully...

Kthread2 Created Successfully...

Device Driver Insert...Done!!!

Function1 [value : 1] [bit:0]    ← Thread1: inc, bit was 0, now 1

Function2 [value : 2] [bit:1]    ← Thread2: inc, bit was 1, now 0

Function1 [value : 3] [bit:0]    ← Thread1 again...

Function2 [value : 4] [bit:1]

...

sudo rmmod driver

### Atomic vs Mutex/Spinlock — Decision Guide

Single integer variable shared between contexts?

→ Use atomic_t + atomic_inc/dec/read/set

→ FASTEST, no lock overhead

Single bit flag shared between contexts?

→ Use test_and_set_bit / test_and_clear_bit / test_and_change_bit

→ Atomic, no lock needed

Multiple variables / complex data structures?

→ Use spinlock or mutex (atomic won't help here)

Inside an already-locked section, need to flip bits?

→ Use non-atomic _set_bit / _clear_bit (faster inside lock)

Key takeaway: Atomic variables are the lightest synchronization mechanism — use them when sharing a single integer or bit between threads. They need NO lock, NO mutex, NO spinlock. Operations happen in ONE CPU instruction, making races impossible. Use atomic_read() to read, never access .counter directly.

# Device Driver 31 – Seqlock

### What is Seqlock?

Seqlock (Sequential Lock) — added in kernel 2.5.60 — is a reader-writer lock that gives priority to the WRITER. The reader never blocks the writer. The reader simply retries if a write is in progress.

vs Read-Write Spinlock (Part 24): rwlock gives priority to readers — writer can starve. Seqlock gives priority to writers — reader retries but never blocks writer.

### How Seqlock Works — The Sequence Number

The seqlock maintains an internal sequence counter:

seq = 0   → even → nothing happening → readers can proceed

seq = 1   → odd  → writer is currently writing → readers must WAIT/RETRY

seq = 2   → even → write done → readers check if seq changed since they started

#### Writer flow:

write_seqlock()   → lock spinlock + seq++ (0→1, odd = writing)

/* write data */

write_sequnlock() → seq++ (1→2, even = done) + unlock spinlock

#### Reader flow:

seq_no = read_seqbegin()   → save current seq (must be even)

/* read data */

read_seqretry()             → check if seq changed or is still odd

→ returns 1 (true)  = write happened during read → RETRY

→ returns 0 (false) = read was clean → done

### When to Use Seqlock

✅ Use seqlock when:

- Reads happen much more often than writes

- Write access is rare but must be fast (cannot be blocked by readers)

- Data is simple — no pointers (reader may follow stale pointer during concurrent write → crash)

❌ Do NOT use when data contains pointers — seqlock is unsafe for pointer-based data structures.

### Required Header -> #include <linux/seqlock.h>

### Initialize

seqlock_t etx_seq_lock;

seqlock_init(&etx_seq_lock);   /* initialize seqlock at runtime */

### Write API

| Function | Use Case |
| --- | --- |
| write_seqlock(&lock) | User/thread context |
| write_tryseqlock(&lock) | Try without waiting — returns non-zero if lock acquired, 0 if busy |
| write_seqlock_irqsave(&lock, flags) | IRQ context — saves IRQ state |
| write_seqlock_irq(&lock) | IRQ context — disables IRQ |
| write_seqlock_bh(&lock) | Bottom half context |

| Unlock Function | Paired With |
| --- | --- |
| write_sequnlock(&lock) | write_seqlock |
| write_sequnlock_irqrestore(&lock, flags) | write_seqlock_irqsave |
| write_sequnlock_irq(&lock) | write_seqlock_irq |
| write_sequnlock_bh(&lock) | write_seqlock_bh |

#### Write Example:

write_seqlock(&lock);

/* write data here */

write_sequnlock(&lock);

### Read API

| Function | Description |
| --- | --- |
| read_seqbegin(&lock) | Start read — returns current sequence number |
| read_seqbegin_irqsave(&lock, flags) | Start read + save IRQ state |
| read_seqretry(&lock, seq_no) | Check if read was valid — returns 1=retry, 0=ok |
| read_seqretry_irqrestore(&lock, seq_no, flags) | Check validity + restore IRQ state |

#### Read Example:

unsigned int seq_no;

do {

seq_no = read_seqbegin(&lock);  /* step 1: get sequence number */

/* step 2: read the data */

} while (read_seqretry(&lock, seq_no));  /* step 3: retry if write happened */

read_seqretry() returns 1 (retry) if:

- Current seq is ODD (write in progress), OR

- Current seq != saved seq_no (write happened during our read)

### Key Differences — All Locking Mechanisms

| Mechanism | Writer Blocked By | Reader Blocked By | Priority |
| --- | --- | --- | --- |
| Spinlock | Other writers/readers | Writers/readers | Equal |
| RW Spinlock (Part 24) | Readers + writers | Other writers | Readers |
| Seqlock (Part 31) | Other writers only | Nobody (retries) | Writers |
| Mutex (Part 22) | Other holders | Other holders | Equal |

### ⚠️ Bug in Tutorial Code

The original tutorial calls seqlock_init() AFTER kthread_run() — wrong. If a thread uses the seqlock before it's initialized → undefined behavior.

Correct order: seqlock_init() BEFORE kthread_run()

### Test Commands & Expected Output

sudo insmod driver.ko

dmesg

Kthread1 Created Successfully...   ← Writer thread

Kthread2 Created Successfully...   ← Reader thread

Device Driver Insert...Done!!!

In EmbeTronicX Thread Function2 : Read value 1   ← reader reads

In EmbeTronicX Thread Function2 : Read value 2   ← after next write

...

sudo rmmod driver

### All Synchronization Mechanisms — Final Comparison

Scenario                         		→ Use

────────────────────────────────────────────────────────

Single integer, shared threads  	→ atomic_t

Single bit flag                 		→ test_and_set_bit()

Multiple vars, equal priority   	→ spinlock / mutex

Multiple vars, reader priority  	→ read-write spinlock

Multiple vars, writer priority  	→ seqlock ← THIS TUTORIAL

Event notification              	→ completion

Long wait, process context      	→ mutex

Short wait, any context         	→ spinlock

ISR/atomic context              	→ spinlock / tasklet

Deferred work, can sleep        	→ workqueue

Key takeaway: Seqlock = writer-priority lock. Writers never wait for readers. Readers detect concurrent writes and retry. Use ONLY for simple (non-pointer) data that is read frequently but written rarely. Always call seqlock_init() BEFORE threads that use it.

# Device Driver 32 – Misc Device Driver

### What is a Misc Driver?

Misc (Miscellaneous) Driver is a special, simplified type of character driver used for devices that don't fit into any standard category. It is built on top of the character driver framework but with much less boilerplate code.

Simple rule: If your device is simple and you don't want the complexity of a full character driver — use a misc driver.

### Key Properties

- Fixed Major Number = 10 — always. All misc devices share this.

- Minor Number = 1 to 255 — you choose (or let kernel assign dynamically).

- Device file auto-created at /dev/<name> — no manual class/device creation.

- Supports all file operations: open, read, write, close, ioctl.

### Misc Driver vs Character Driver

|  | Character Driver | Misc Driver |
| --- | --- | --- |
| Major number | You allocate (dynamic or static) | Fixed = 10 |
| Minor number | You manage | You choose or use MISC_DYNAMIC_MINOR |
| Device file creation | Manual: cdev_init + cdev_add + class_create + device_create | Automatic via misc_register() |
| Code complexity | More steps | Just 2 API calls |
| RAM usage | Each driver reserves a major number slot | All misc devices share major 10 → less RAM |
| When to use | Complex, classifiable devices | Simple, unclassifiable devices |

### Required Header ->  #include <linux/miscdevice.h>

### Misc Device Structure

struct miscdevice {

int minor;                      /* minor number (1-255 or MISC_DYNAMIC_MINOR) */

const char *name;               /* device name → creates /dev/<name>          */

struct file_operations *fops;   /* pointer to file operations table            */

struct miscdevice *next, *prev; /* internal linked list management             */

};

minor field options:

- MISC_DYNAMIC_MINOR — let kernel auto-assign minor number ✅ recommended

- A specific number (1-255) — hardcode it, but check ls -l /dev/ first to avoid conflicts

### Only 2 APIs Needed

#### Register (replaces 5 char driver calls)

int misc_register(struct miscdevice *misc);

/*

* Replaces ALL of these character driver calls:

*   alloc_chrdev_region()  → handles major/minor

*   cdev_init()            → initializes cdev

*   cdev_add()             → adds to system

*   class_create()         → creates /sys/class/

*   device_create()        → creates /dev/name

*

* Returns: 0 on success, negative errno on failure.

* Call in __init function.

*/

#### Unregister (replaces 4 char driver calls)

void misc_deregister(struct miscdevice *misc);

/*

* Replaces ALL of these:

*   device_destroy()

*   class_destroy()

*   cdev_del()

*   unregister_chrdev_region()

*

* Call in __exit function.

*/

### Verify After Loading

sudo insmod misc_driver.ko

ls -l /dev/simple_etx_misc

# crw------- 1 root root 10, 53 May  9 20:42 /dev/simple_etx_misc

#                          ↑↑   ↑↑

#                          10 = major (always 10 for misc)

#                               53 = minor (auto-assigned)

### Test Commands & Expected Output

sudo insmod misc_driver.ko

dmesg   # → misc_register init done!!!

# Write test

sudo su

echo 1 > /dev/simple_etx_misc

dmesg

# → EtX misc device open

# → EtX misc device write

# → EtX misc device close

# Read test

cat /dev/simple_etx_misc

dmesg

# → EtX misc device open

# → EtX misc device read

# → EtX misc device close

sudo rmmod misc_driver

dmesg   # → misc_register exit done!!!

### Character Driver vs Misc Driver — Code Comparison

/* ── CHARACTER DRIVER (5 steps in init, 4 in exit) ── */

/* INIT */

alloc_chrdev_region(&dev, 0, 1, "name");   /* step 1 */

cdev_init(&etx_cdev, &fops);               /* step 2 */

cdev_add(&etx_cdev, dev, 1);               /* step 3 */

class_create(THIS_MODULE, "class");        /* step 4 */

device_create(dev_class, NULL, dev, NULL, "device"); /* step 5 */

/* EXIT */

device_destroy(dev_class, dev);            /* step 1 */

class_destroy(dev_class);                  /* step 2 */

cdev_del(&etx_cdev);                       /* step 3 */

unregister_chrdev_region(dev, 1);          /* step 4 */

/* ── MISC DRIVER (1 step each!) ── */

/* INIT */

misc_register(&etx_misc_device);           /* just 1 call → does all 5 above */

/* EXIT */

misc_deregister(&etx_misc_device);         /* just 1 call → does all 4 above */

### Complete Flow Summary

insmod misc_driver.ko

└── misc_register(&etx_misc_device)

├── assigns major=10, minor=53 (auto)

├── registers internally with kernel

└── udev creates /dev/simple_etx_misc

ls -l /dev/simple_etx_misc

→ crw------- 1 root root 10, 53 → major=10, minor=53

echo 1 > /dev/simple_etx_misc

├── etx_misc_open()   → "EtX misc device open"

├── etx_misc_write()  → "EtX misc device write"

└── etx_misc_close()  → "EtX misc device close"

cat /dev/simple_etx_misc

├── etx_misc_open()   → "EtX misc device open"

├── etx_misc_read()   → "EtX misc device read"

└── etx_misc_close()  → "EtX misc device close"

rmmod misc_driver

└── misc_deregister(&etx_misc_device)

└── /dev/simple_etx_misc removed

Key takeaway: Misc driver = simplified character driver. Use it when your device is simple and doesn't fit any standard class. Just fill struct miscdevice (minor + name + fops), call misc_register() in init and misc_deregister() in exit. Major number is always 10. Device file created automatically.

# Device Driver 33 – USB Device Driver Basics

## USB Subsystem Overview

When a USB device is plugged into a Linux system, three components work together:

USB Device  ←→  USB Host Controller  ←→  USB Core  ←→  USB Driver (your code)

The USB subsystem consists of the USB core, the USB Host Controller, and the USB host controller’s driver.

## 1. USB Device

Any physical USB peripheral — pendrive, mouse, keyboard, USB-to-serial converter, MP3 player, etc.

## 2. USB Host Controller

The USB host controller is used to control the communications between the host system and USB devices. Has both hardware and software parts.

Hardware part does:

- Detect connected/disconnected USB devices

- Provide power to connected USB devices

Software part (Host Controller Driver) does:

- Performs initial transaction on detection: reads Vendor ID, Product ID, device type

- Loads the correct USB device driver based on these IDs

- Manages data transfer between driver and hardware

### Host Controller Interface Types (by USB version):

| Interface | USB Version |
| --- | --- |
| OHCI (Open Host Controller Interface) | USB 1.x |
| UHCI (Universal Host Controller Interface) | USB 1.x |
| EHCI (Enhanced Host Controller Interface) | USB 2.0 |
| XHCI (Extended Host Controller Interface) | USB 3.0 |

## 3. USB Core

A codebase of routines and structures shared by both the Host Controller Driver (HCD) and USB drivers. It is the glue between hardware and drivers.

## 4. USB Driver

The driver YOU write to handle a specific USB device — this is what these tutorials teach.

## USB Descriptors (Device Identity)

When a USB device connects, it sends descriptors to tell the host what it is and what it can do. There are 4 layers of descriptors, nested inside each other:

Device Descriptor          → 1 per device (Vendor ID, Product ID, USB version)

└── Configuration Descriptor  → 1 or more per device (power info, interfaces count)

└── Interface Descriptor   → 1 or more per config (functional unit, class)

└── Endpoint Descriptor  → 1 or more per interface (transfer type, direction)

### 1. Device Descriptor

- Only ONE per USB device

- Contains: USB revision, Vendor ID (VID), Product ID (PID), number of configurations

- VID + PID → used by kernel to match and load the correct driver

### 2. Configuration Descriptor

- A device can have more than one configuration

- But only ONE can be active at a time

- Contains: power consumption, bus/self-powered info, number of interfaces

### 3. Interface Descriptor

- A device can have one or more interfaces per configuration

- Multiple interfaces can be active simultaneously (unlike configurations)

- Each interface = one functional unit of the device

- Example: Multi-function printer has 3 interfaces (fax, scanner, printer)

- Each interface has its own set of endpoints

### 4. Endpoint Descriptor

- Each endpoint = one source or sink of data

- Contains: transfer type, direction (IN/OUT), polling interval, max packet size

- Direction: IN = device to host, OUT = host to device

## Data Flow Types (Transfer Types)

Set in the endpoint descriptor. There are 4 types:

### 1. Control Transfers

- Used for command and status operations

- All enumeration (device setup when first connected) uses control transfers

- Bi-directional — uses both IN and OUT endpoints

- Example: setting device address, reading descriptors

### 2. Interrupt Transfers

- NOT related to hardware interrupts — name is historical

- Regularly scheduled IN or OUT transactions (polled periodically)

- IN direction is more common

- Used for devices that need periodic small data (mouse movements, keyboard keys)

- Example: Mouse, Keyboard, Joystick

### 3. Bulk Transfers

- For large amounts of data with guaranteed error-free delivery

- No guaranteed bandwidth — scheduled after other transfers get their allocation

- If bandwidth is available → fast. If busy → waits.

- OUT endpoint = host sends data TO device

- IN endpoint = host receives data FROM device

- Example: USB pen drive, USB printer

### 4. Isochronous Transfers

- Guaranteed bandwidth but error-free delivery is NOT guaranteed

- Some data loss/corruption is acceptable

- Used for real-time streaming where continuous flow matters more than accuracy

- Example: USB microphone, USB webcam, USB speaker

- Uses either IN or OUT depending on endpoint type

## All 4 Transfer Types — Quick Reference

| Transfer Type | Bandwidth Guaranteed? | Error-Free? | Direction | Use Case |
| --- | --- | --- | --- | --- |
| Control | ✅ Yes (reserved) | ✅ Yes | Both (IN + OUT) | Enumeration, setup |
| Interrupt | ✅ Yes (periodic) | ✅ Yes | Mostly IN | Mouse, keyboard |
| Bulk | ❌ No | ✅ Yes | IN or OUT | Pen drive, printer |
| Isochronous | ✅ Yes | ❌ No | IN or OUT | Audio, video, webcam |

## USB Enumeration Flow (What Happens When You Plug in a USB Device)

USB Device plugged in

│

▼

USB Host Controller Hardware detects connection

│

▼

Host Controller Driver reads Device Descriptor

→ gets Vendor ID (VID) + Product ID (PID)

│

▼

USB Core looks up matching driver in kernel

using VID:PID table

│

▼

Matching USB Driver loaded

→ reads Configuration + Interface + Endpoint descriptors

→ sets up endpoints for data transfer

│

▼

Device is ready to use

## Descriptor Hierarchy Visualized

USB Device

├── Device Descriptor (ONE only)

│     VID=0x0781, PID=0x5567, USB 2.0

│

└── Configuration 1 (only this one active)

Bus-powered, 100mA

│

├── Interface 0 (Mass Storage)

│     ├── Endpoint 1 IN  (Bulk — read data from device)

│     └── Endpoint 2 OUT (Bulk — send data to device)

│

└── Interface 1 (HID / Control Panel)

└── Endpoint 3 IN (Interrupt — button state changes)

## Summary Table — All Concepts

USB Subsystem Components:

USB Device          → physical hardware (pendrive, mouse, etc.)

Host Controller     → hardware+software managing USB bus

USB Core            → shared kernel codebase (glue layer)

USB Driver          → your code that talks to the device

Descriptors (4 layers):

Device Descriptor      → ONE: VID, PID, USB version

Configuration Desc     → power, interface count, one active at a time

Interface Descriptor   → functional unit, multiple can be active

Endpoint Descriptor    → data source/sink: type + direction

Transfer Types (4 types):

Control        → bidirectional, enumeration, setup commands

Interrupt      → periodic, small data, mostly IN (mouse/keyboard)

Bulk           → large data, error-free, no bandwidth guarantee (pendrive)

Isochronous    → real-time streaming, bandwidth guaranteed, error OK (audio/video)

Key takeaway: Every USB device announces what it is through descriptors (VID+PID for driver matching, configurations for power management, interfaces for functional units, endpoints for actual data transfer). The type of transfer used by an endpoint determines the speed vs reliability trade-off. Programming a USB driver means matching VID:PID and setting up communication through the right endpoints — which is covered in Part 34.

# Linux Device Driver 34 – USB Device Driver Example Program

### How USB Driver Differs from Character Driver

|  | Character Driver | USB Driver |
| --- | --- | --- |
| Register with | Kernel (major/minor) | USB Core/Subsystem |
| Device match | Major number | Vendor ID + Product ID |
| Entry function | module_init | probe (called on USB insert) |
| Exit function | module_exit | disconnect (called on USB remove) |
| Register API | register_chrdev | usb_register |
| Deregister API | unregister_chrdev | usb_deregister |

### Required Header-> #include <linux/usb.h>   /* all USB driver APIs */

### struct usb_driver — Core Structure

This is the main structure you fill to register your USB driver with the USB core.

static struct usb_driver etx_usb_driver = {

.name       = "EmbeTronicX USB Driver",  	/* unique driver name          */

.probe      = etx_usb_probe,             		/* called when device plugged  */

.disconnect = etx_usb_disconnect,       	/* called when device removed  */

.id_table   = etx_usb_table,            		/* devices this driver handles */

};

#### Key fields:

| Field | Mandatory? | Purpose |
| --- | --- | --- |
| name | ✅ Yes | Unique driver name |
| probe | ✅ Yes | Called when matching device is connected |
| disconnect | ✅ Yes | Called when device is disconnected |
| id_table | ✅ Yes | VID+PID table — MUST be set or probe never called |
| ioctl | ❌ Optional | Talk to user space via usbfs filesystem |
| suspend | ❌ Optional | Called before system suspend |
| resume | ❌ Optional | Called after system resume |
| reset_resume | ❌ Optional | Called if device was reset during suspend |
| pre_reset | ❌ Optional | Called before USB device reset |
| post_reset | ❌ Optional | Called after USB device reset |

### Step 1 — Define the Device ID Table

#define USB_VENDOR_ID   0x22d9   /* your device's Vendor ID  */

#define USB_PRODUCT_ID  0x2764   /* your device's Product ID */

const struct usb_device_id etx_usb_table[] = {

{ USB_DEVICE(USB_VENDOR_ID, USB_PRODUCT_ID) },  /* match this device */

{ }   /* terminating entry — MUST be last  */

};

MODULE_DEVICE_TABLE(usb, etx_usb_table);

/*

* MODULE_DEVICE_TABLE:

*   Exposes id_table to the kernel's hotplug system.

*   Allows kernel to automatically load this driver when device is plugged in.

*   Without this: driver won't auto-load (manual insmod still works).

*/

How to find VID and PID: Plug in device → run lsusb → find your device line → ID xxxx:yyyy (xxxx=VID, yyyy=PID)

### Step 2 — Write probe() Function

static int etx_usb_probe(struct usb_interface *interface,

const struct usb_device_id *id)

{

/* Returns 0 = we accept/manage this device

Returns error code = we reject it */

}

Called when a matching USB device is inserted. Use it to:

- Verify the device is acceptable

- Allocate resources

- Read and store interface/endpoint descriptors

- Return 0 on success, negative error on failure

### Step 3 — Write disconnect() Function

static void etx_usb_disconnect(struct usb_interface *interface)

{

/* Called when device is physically removed or driver unloaded */

/* Free all resources allocated in probe() */

}

### Step 4 — Register/Deregister with USB Core

#### Old Method (kernel < 3.3)

static int __init etx_usb_init(void)

{

return usb_register(&etx_usb_driver);

}

static void __exit etx_usb_exit(void)

{

usb_deregister(&etx_usb_driver);

}

module_init(etx_usb_init);

module_exit(etx_usb_exit);

#### New Method (kernel >= 3.3) — One line! ✅

module_usb_driver(etx_usb_driver);

/* Replaces ALL of: usb_register, usb_deregister, __init, __exit, module_init, module_exit */

### dev_info vs pr_info

dev_info(&interface->dev, "message\n");   /* USB driver style */

pr_info("message\n");                     /* general kernel style */

dev_info prints the message PLUS device-specific info (device name, bus, etc.) — helps filter dmesg for a specific device. Preferred in USB drivers.

### How to Read Interface and Endpoint Descriptors in probe()

struct usb_host_interface *iface_desc = interface->cur_altsetting;

/* Interface descriptor fields */

iface_desc->desc.bNumEndpoints         /* number of endpoints     */

iface_desc->desc.bInterfaceNumber     /* interface number        */

iface_desc->desc.bInterfaceClass      	/* device class            */

/* Loop through all endpoints */

for (i = 0; i < iface_desc->desc.bNumEndpoints; i++) {

iface_desc->endpoint[i].desc.bEndpointAddress  /* IN or OUT? */

iface_desc->endpoint[i].desc.bmAttributes      	    /* transfer type */

iface_desc->endpoint[i].desc.wMaxPacketSize    /* max packet size */

}

### Test Commands

sudo make                        		# build driver

sudo insmod usb_driver.ko        	# load driver

dmesg                            # → "registered new interface driver EmbeTronicX USB Driver"

# Plug in USB device with matching VID:PID

lsusb                            # find your device VID:PID

dmesg                         # → probe() called → prints interface + endpoint info

# Unplug USB device

dmesg                        # → "USB Driver Disconnected"

sudo rmmod usb_driver            # unload

dmesg                            # → "deregistering interface driver EmbeTronicX USB Driver"

### Complete Flow Summary

sudo insmod usb_driver.ko

└── usb_register() → registered with USB core

→ dmesg: "registered new interface driver EmbeTronicX USB Driver"

Plug in USB device (VID=0x22d9, PID=0x2764)

├── USB core detects connection

├── reads device descriptor → gets VID+PID

├── checks all registered drivers' id_tables

├── finds match in etx_usb_table

└── calls etx_usb_probe(interface, id)

├── dev_info("USB Driver Probed: VID=0x22d9, PID=0x2764")

├── PRINT_USB_INTERFACE_DESCRIPTOR → prints all interface fields

└── for each endpoint → PRINT_USB_ENDPOINT_DESCRIPTOR

Unplug USB device

└── calls etx_usb_disconnect(interface)

└── dev_info("USB Driver Disconnected")

sudo rmmod usb_driver

└── usb_deregister() → removed from USB core

→ dmesg: "deregistering interface driver EmbeTronicX USB Driver"

### USB Driver vs Character Driver — Code Structure Comparison

/* CHARACTER DRIVER */

alloc_chrdev_region();         	/* get major/minor         */

cdev_init(); cdev_add();       	/* register with kernel    */

class_create(); device_create();	/* create /dev/ entry     */

/* file_operations: open, read, write, release, ioctl */

/* USB DRIVER */

usb_register(&usb_driver);    /* register with USB core  */

/* usb_driver: probe (on insert), disconnect (on remove)  */

/* id_table:   VID+PID → tells kernel which device        */

MODULE_DEVICE_TABLE(usb, table); /* enable hotplug auto-load */

Key takeaway: USB driver = probe + disconnect + id_table + usb_register. probe is called on device insert, disconnect on removal. id_table matches devices by VID+PID. Use module_usb_driver() macro on kernel >= 3.3 to replace all boilerplate init/exit code. Always end id_table[] with an empty {} sentinel entry -(a special "dummy" element placed at the end of a struct array to signal its termination).

# Device Driver 35 – GPIO Driver Basic

First tutorial using real hardware (Raspberry Pi). Controls an LED connected to GPIO 21 using a Linux character driver. Write "1" → LED ON. Write "0" → LED OFF.

### Hardware Required

- Raspberry Pi 4 Model B

- Breadboard

- LED + Resistor

- Jumper wires

Setup:

# Install kernel headers (mandatory for building modules on RPi)

sudo apt install raspberrypi-kernel-headers

### Required Header-> #include <linux/gpio.h>   /* all GPIO kernel APIs */

### 7 Steps to Use GPIO in Kernel

1. gpio_is_valid()        → check if GPIO number is usable

2. gpio_request()         → claim/allocate the GPIO

3. gpio_export()          → [OPTIONAL] expose to sysfs for debugging

4. gpio_direction_input() or gpio_direction_output() → set direction

5. gpio_set_value()       → drive high/low (output only)

6. gpio_get_value()       → read current state (input or output)

7. gpio_free()            → release the GPIO when done

### All GPIO APIs — Quick Reference

#### 1. Validate

bool gpio_is_valid(int gpio_number);

/* Returns true=valid, false=invalid or temporarily unavailable */

#### 2. Request (Allocate)

int gpio_request(unsigned gpio, const char *label);

/* label = string visible in /sys/kernel/debug/gpio

* Returns: 0=success, negative=failure

* MUST call before any other GPIO operation */

/* Variants: */

int gpio_request_one(unsigned gpio, unsigned long flags, const char *label);

int gpio_request_array(struct gpio *array, size_t num);

#### 3. Export to sysfs (Optional — for debugging)

int gpio_export(unsigned int gpio, bool direction_may_change);

/* gpio                  = GPIO to export

* direction_may_change  = true: user can change direction

*                         false: direction locked (read-only from userspace)

* Creates: /sys/class/gpio/gpio21/ with value, direction files

* Returns: 0=success, negative=failure */

After export, control from shell:

echo 1 > /sys/class/gpio/gpio21/value   # turn ON

echo 0 > /sys/class/gpio/gpio21/value   # turn OFF

cat /sys/class/gpio/gpio21/value        # read state

#### 4. Unexport (Remove from sysfs)

void gpio_unexport(unsigned int gpio);

/* Reverses gpio_export() — removes /sys/class/gpio/gpio21/ */

#### 5. Set Direction

int gpio_direction_input(unsigned gpio);

/* Set as input — for reading button/sensor state

* Returns: 0=success, negative=error */

int gpio_direction_output(unsigned gpio, int value);

/* Set as output AND set initial value (0=low, 1=high)

* Returns: 0=success, negative=error */

#### 6. Set/Get Value

void gpio_set_value(unsigned int gpio, int value);

/* 0=LOW (LED OFF), 1=HIGH (LED ON) — output only */

int gpio_get_value(unsigned gpio);

/* Returns: 0 or 1 — works for input AND output */

#### 7. Release

void gpio_free(unsigned int gpio);

/* Release GPIO back to system — MUST call in exit() */

void gpio_free_array(struct gpio *array, size_t num);  /* release multiple */

### GPIO Interrupt (IRQ)

Covered in Part 36. For now, know that GPIOs can trigger interrupts on edge (rising/falling) or level changes.

### Connection-> Raspberry Pi GPIO 21 → Resistor → LED → GND

### Test Commands

sudo make                          		# build

sudo insmod driver.ko              	# load

sudo su                            		# get root

echo 1 > /dev/etx_device           	# LED ON

echo 0 > /dev/etx_device           	# LED OFF

cat /dev/etx_device                	# read GPIO state

# OR using sysfs (since gpio_export was called):

echo 1 > /sys/class/gpio/gpio21/value   # LED ON

echo 0 > /sys/class/gpio/gpio21/value   # LED OFF

cat /sys/class/gpio/gpio21/value        	# read state

sudo rmmod driver                  # unload

### Complete Flow Summary

insmod driver.ko

├── Standard char device setup (/dev/etx_device created)

├── gpio_is_valid(21)        → check GPIO 21 is available

├── gpio_request(21)         → claim GPIO 21 for this driver

├── gpio_direction_output(21, 0) → configure as output, LED=OFF

└── gpio_export(21, false)   → /sys/class/gpio/gpio21/ created

echo 1 > /dev/etx_device

├── copy_from_user → rec_buf[0]='1'

└── gpio_set_value(21, 1) → GPIO 21 HIGH → LED ON ✅

echo 0 > /dev/etx_device

└── gpio_set_value(21, 0) → GPIO 21 LOW → LED OFF ✅

cat /dev/etx_device

├── gpio_get_value(21) → 0 or 1

└── copy_to_user → sends raw byte to user

rmmod driver

├── gpio_unexport(21) → /sys/class/gpio/gpio21/ removed

└── gpio_free(21)     → GPIO 21 released

### GPIO API Usage Order (Always Follow This)

gpio_is_valid()           ← 1st: check before anything

gpio_request()            ← 2nd: claim it

gpio_direction_output()   ← 3rd: set direction

gpio_set_value()          ← use to drive output

gpio_get_value()          ← use to read state

gpio_unexport()           ← before free (if exported)

gpio_free()               ← LAST: release in exit()

Key takeaway: GPIO driver = character driver + GPIO APIs. Always validate → request → configure direction → use → free. Call gpio_free() in exit — forgetting it means next insmod will fail with "GPIO request error". gpio_export() is optional but very useful for debugging directly from shell without needing the device file.

# Device Driver 36 – GPIO Interrupt

Part 35 = GPIO as OUTPUT only (LED control). Part 36 = GPIO as INPUT with INTERRUPT — a button/sensor triggers an IRQ which toggles the LED.

GPIO 21 → OUTPUT → LED

GPIO 25 → INPUT  → Button/Vibration Sensor → triggers IRQ → toggles LED

### New APIs Introduced (Not in Part 35)

#### 1. Set Debounce Interval

int gpiod_set_debounce(unsigned gpio, unsigned debounce);

/*

* gpio     = GPIO pin number

* debounce = delay in milliseconds

* Returns: 0=success, negative=error

*

* ⚠️ Raspberry Pi does NOT support hardware debounce.

* Use software debounce workaround instead (jiffies trick).

*/

#### 2. Get IRQ Number from GPIO

int gpio_to_irq(unsigned gpio);

/*

* Converts a GPIO number → Linux IRQ number.

* The returned IRQ number is then used with request_irq().

* Example: GPIO 25 → IRQ number 200 (kernel assigns it)

*/

#### 3. Interrupt Trigger Flags (new for GPIO)

Used in request_irq() as the flags argument:

| Flag | Trigger Condition |
| --- | --- |
| IRQF_TRIGGER_RISING | Fires when signal goes LOW→HIGH |
| IRQF_TRIGGER_FALLING | Fires when signal goes HIGH→LOW |
| IRQF_TRIGGER_HIGH | Fires while signal is HIGH (level) |
| IRQF_TRIGGER_LOW | Fires while signal is LOW (level) |

### Steps to Use GPIO as Input with IRQ

1. gpio_is_valid(GPIO_25_IN)          	→ validate

2. gpio_request(GPIO_25_IN, "label") 	→ claim

3. gpio_direction_input(GPIO_25_IN)  	→ set as input

4. gpio_set_debounce() OR software   	→ handle bounce

5. GPIO_irqNumber = gpio_to_irq(GPIO_25_IN) 	→ get IRQ number

6. request_irq(GPIO_irqNumber, handler, flags, name, dev_id)

7. free_irq(GPIO_irqNumber, NULL)    → in exit()

8. gpio_free(GPIO_25_IN)             → in exit()

### Debounce Problem & Software Workaround

Problem: When a button is pressed, the signal bounces between HIGH and LOW multiple times in microseconds → triggers ISR multiple times for ONE press.

Hardware fix: gpio_set_debounce() — NOT supported on Raspberry Pi.

Software fix (jiffies trick):

unsigned long diff = jiffies - old_jiffie;

if (diff < 20) {

return IRQ_HANDLED;  /* too soon — ignore this bounce */

}

old_jiffie = jiffies;   /* record time of valid IRQ */

jiffies = kernel tick counter. If less than 20 jiffies have passed since last valid IRQ → ignore (it's a bounce). 20 jiffies ≈ 80ms at HZ=250.

### local_irq_save / local_irq_restore in ISR

local_irq_save(flags);    /* save current IRQ state + disable all IRQs */

/* critical section */

local_irq_restore(flags); /* restore IRQ state (re-enable if they were on) */

Used inside the ISR to protect the LED toggle operation from being interrupted by other IRQs while in progress. Creates a brief atomic section.

### Test Commands

sudo make                      	   # build

sudo insmod driver.ko          # load

# Press button or vibrate sensor connected to GPIO 25

# → LED connected to GPIO 21 toggles ON/OFF each press

dmesg                          # see: "Interrupt Occurred : GPIO_21_OUT : 1/0"

sudo rmmod driver              # unload

### Complete Flow Summary

insmod driver.ko

├── GPIO 21 → OUTPUT, initial LOW (LED OFF)

├── GPIO 25 → INPUT (button/sensor)

├── GPIO_irqNumber = gpio_to_irq(25) → get IRQ number

└── request_irq(irqNumber, gpio_irq_handler, RISING) → ISR registered

Button pressed (GPIO 25 goes LOW→HIGH = rising edge)

└── kernel fires IRQ → gpio_irq_handler() called

├── [debounce check: if < 20 jiffies → return, it's a bounce]

├── local_irq_save() → protect critical section

├── led_toggle = 0x01 ^ led_toggle  → flip 0↔1

├── gpio_set_value(21, led_toggle)  → LED toggles

├── pr_info("Interrupt Occurred : GPIO_21_OUT : X")

└── local_irq_restore() → done

rmmod driver

├── free_irq()    → ISR unregistered (no more GPIO 25 interrupts)

├── gpio_free(25) → release input GPIO

└── gpio_free(21) → release output GPIO

### Part 35 vs Part 36 — What's New

Part 35 (Output only):

gpio_request(21)

gpio_direction_output(21, 0)

→ write() calls gpio_set_value()

Part 36 (Input + Interrupt) adds:

gpio_request(25)

gpio_direction_input(25)

GPIO_irqNumber = gpio_to_irq(25)    ← NEW: get IRQ from GPIO

request_irq(irqNumber, handler, IRQF_TRIGGER_RISING, ...)  ← NEW

→ ISR auto-called on button press → toggles LED

free_irq()  ← NEW: in exit()

Key takeaway: GPIO interrupt = gpio_to_irq() to get IRQ number + request_irq() with the right trigger flag. Always call free_irq() before gpio_free() in exit. Raspberry Pi needs software debounce (jiffies trick) because gpio_set_debounce() is not supported. local_irq_save/restore protects the critical section inside the ISR.

# Device Driver 37 – I2C Linux Device Driver

Writing an I2C client driver that communicates with an SSD1306 OLED display connected to Raspberry Pi via I2C. Loads driver → fills display with 0xFF (all pixels ON). Unloads driver → clears display with 0x00.

### Prerequisites

- Enable I2C on Raspberry Pi: raspi-config → Interface Options → I2C → Enable

- Check I2C bus: ls -al /sys/bus/i2c/devices/ → should show i2c-1

- Scan for devices: i2cdetect -y 1 → should show address 0x3C (SSD1306)

### I2C Subsystem — 4 Components

I2C Subsystem

├── Buses

│     ├── Algorithm  → a general code that can be used for a whole class of I2C adapters.

│     └── Adapter    → represents a physical I2C bus (ties algorithm + bus number)

└── Devices

├── Client     → represents the I2C slave chip (e.g., SSD1306)

└── Driver     → the driver YOU write for the client (this tutorial)

### 6 Steps to Write I2C Driver

1. i2c_get_adapter()        	→ get the I2C bus adapter structure

2. I2C_BOARD_INFO() + i2c_new_device() → create client for slave

3. i2c_device_id[]          	→ define slave device IDs

4. i2c_driver{}             	→ define probe/remove + id_table

5. i2c_add_driver()         	→ register driver with I2C subsystem

6. i2c_master_send/recv() → transfer data to/from slave

### Step 1 — Get Adapter

struct i2c_adapter *i2c_get_adapter(int nr);

/* nr = I2C bus number (1 for Raspberry Pi 4)

* Returns: i2c_adapter pointer — handle to the I2C bus */

etx_i2c_adapter = i2c_get_adapter(1);

### Step 2 — Create Board Info + Device

#### Create Board info - Just create the i2c_board_info  structure and assign required members of that.

struct i2c_board_info {

char type[I2C_NAME_SIZE];

unsigned short flags;

unsigned short addr;

void * platform_data;

struct dev_archdata * archdata;

struct device_node * of_node;

struct fwnode_handle * fwnode;

int irq;

};

where,

type[I2C_NAME_SIZE] – chip type, to initialize i2c_client.name
flags                             – to initialize i2c_client.flags
addr                               – stored in i2c_client.addr
platform_data             – stored in i2c_client.dev.platform_data
archdata                       – copied into i2c_client.dev.archdata
of_node                         – pointer to OpenFirmware device node
fwnode                           – device node supplied by the platform firmware
irq                                 – stored in i2c_client.irq

use I2C_BOARD_INFO macro to initialize the essential fields of struct i2c_board_info.

I2C_BOARD_INFO ( dev_type, dev_addr);

where,

dev_type – identifies the device type

dev_addr – the device’s address on the bus

#### Create Device

struct i2c_client * i2c_new_device ( struct i2c_adapter * adap, struct i2c_board_info const * info);

where,

*adap – Adapter structure that we got from i2c_get_adapter()

*info – Board info structure that we have created

/* struct i2c_board_info key fields:

*   type[I2C_NAME_SIZE] → chip type (matches driver name)

*   addr                → slave 7-bit I2C address (e.g., 0x3C)

*/

/* Use I2C_BOARD_INFO macro: */

static struct i2c_board_info oled_i2c_board_info = {

I2C_BOARD_INFO("ETX_OLED", 0x3C)  /* name + slave address */

};

/* Create the client (slave device instance): */

struct i2c_client *i2c_new_device(struct i2c_adapter *adap,

struct i2c_board_info const *info);

/* kernel >= 5.2: use i2c_new_client_device() instead */

### Step 3 — Device ID Table

struct i2c_device_id {

char name[I2C_NAME_SIZE];

kernel_ulong_t driver_data;

};

name               –  Slave name
driver_data – Data private to the driver (This data will be passed to the respective driver)

After this, call MODULE_DEVICE_TABLE(i2c, my_id_table) in order to expose the driver along with its I2C device table IDs to userspace.

static const struct i2c_device_id etx_oled_id[] = {

{ "ETX_OLED", 0 },   /* name must match board_info type */

{ }                   /* terminating entry — MUST be last */

};

MODULE_DEVICE_TABLE(i2c, etx_oled_id);

/* Exposes driver to userspace for auto-loading */

### Step 4 — i2c_driver Structure

struct i2c_driver {

unsigned int class;

int (* attach_adapter) (struct i2c_adapter *);

int (* probe) (struct i2c_client *, const struct i2c_device_id *);

int (* remove) (struct i2c_client *);

void (* shutdown) (struct i2c_client *);

void (* alert) (struct i2c_client *, unsigned int data);

int (* command) (struct i2c_client *client, unsigned int cmd, void *arg);

struct device_driver driver;

const struct i2c_device_id * id_table;

int (* detect) (struct i2c_client *, struct i2c_board_info *);

const unsigned short * address_list;

struct list_head clients;

};

Where,

class                   – What kind of i2c device we instantiate (for detect)
attach_adapter – Callback for bus addition (deprecated)
probe                 – Callback for device binding
remove              – Callback for device unbinding
shutdown          – Callback for device shutdown
alert                   – Alert callback, for example for the SMBus alert protocol
command          – Callback for bus-wide signaling (optional)
driver                 – Device driver model driver
id_table              – List of I2C devices supported by this driver
detect                – Callback for device detection
address_list     – The I2C addresses to probe (for detect)
clients             – List of detected clients we created (for i2c-core use only)

static struct i2c_driver etx_oled_driver = {

.driver = {

.name  = "ETX_OLED",     /* must match device name */

.owner = THIS_MODULE,

},

.probe   = etx_oled_probe,   	/* called when slave found */

.remove  = etx_oled_remove,  	/* called when slave removed */

.id_table = etx_oled_id,    	 	/* supported devices */

};

### Step 5 — Add/Remove Driver

i2c_add_driver(struct i2c_driver *i2c_drive);

Where,

i2c_drive – The i2c_driver structure that we have created.

i2c_add_driver(&etx_oled_driver);   /* register with I2C subsystem → probe() 												called */

i2c_del_driver(&etx_oled_driver);    /* remove from I2C subsystem */

### Step 6 — Data Transfer APIs

#### Write (Send to Slave)

int i2c_master_send ( const struct i2c_client * client, const char * buf, int count);

s32 i2c_smbus_write_byte ( const struct i2c_client * client, u8 value);

s32 i2c_smbus_write_byte_data ( const struct i2c_client * client, u8 command, u8 value);

s32 i2c_smbus_write_word_data ( const struct i2c_client * client, u8 command, u16 value);

s32 i2c_smbus_write_block_data ( const struct i2c_client * client, u8 command, u8 length, const u8 * values);

| Function | Use |
| --- | --- |
| i2c_master_send(client, buf, count) | Send count bytes buffer to slave |
| i2c_smbus_write_byte(client, value) | Send single byte |
| i2c_smbus_write_byte_data(client, cmd, val) | Send command + byte |
| i2c_smbus_write_word_data(client, cmd, val) | Send command + 16-bit word |
| i2c_smbus_write_block_data(client, cmd, len, vals) | Send block (max 32 bytes) |

#### Read (Receive from Slave)

int i2c_master_recv ( const struct i2c_client * client, const char * buf, int count);

s32 i2c_smbus_read_byte ( const struct i2c_client * client);

s32 i2c_smbus_read_byte_data ( const struct i2c_client * client, u8 command);

s32 i2c_smbus_read_word_data ( const struct i2c_client * client, u8 command);

s32 i2c_smbus_read_block_data ( const struct i2c_client * client, u8 command, u8 * values);

| Function | Use |
| --- | --- |
| i2c_master_recv(client, buf, count) | Read count bytes into buffer |
| i2c_smbus_read_byte(client) | Read single byte |
| i2c_smbus_read_byte_data(client, cmd) | Send command, read byte |
| i2c_smbus_read_word_data(client, cmd) | Send command, read 16-bit word |
| i2c_smbus_read_block_data(client, cmd, vals) | Send command, read block (max 32 bytes) |

#### Multiple Messages

int i2c_transfer(struct i2c_adapter *adap, struct i2c_msg *msgs, int num);

/* Send multiple START-data-STOP sequences in one call */

⚠️ i2c_master_send and i2c_transfer CANNOT be used inside an ISR — they sleep internally between bus conditions.

### How I2C Transfer Works Internally

i2c_master_send() called by client driver

↓

master_xfer() in bus driver

↓

Bus driver generates: START → ADDRESS → DATA → STOP

(one by one, sleeps on waitqueue between each condition)

↓

Hardware interrupt on each condition completion

↓

master_xfer wakes up, sends next condition

### SSD1306 OLED — Control Byte

Before sending command or data to OLED, always send a control byte:

Control byte:

Bit 7 (Co) = 0    → following bytes contain data only

Bit 6 (D/C) = 0   → next byte is COMMAND

Bit 6 (D/C) = 1   → next byte is DATA to display RAM

Bits 5-0 = 0

| Send | Control Byte | Data Byte |
| --- | --- | --- |
| Command | 0x00 | command (e.g., 0xAE) |
| Display Data | 0x40 | data (e.g., 0xFF) |

### SSD1306 Memory Map

1KB GDDRAM = 8 pages × 128 segments × 8 bits = 8192 bits

Fill all 1024 bytes to fill entire display:

for (i = 0; i < 128*8; i++) SSD1306_Write(false, 0xFF);

ssd1306 pixels

Width  = 128 pixels
Height = 64 pixels

Because each pixel is only:

ON = 1

OFF = 0

the display needs:

128 × 64 = 8192 bits

8192 / 8 = 1024 bytes

So the SSD1306 contains 1024 bytes of display RAM.

Why 8 pages?

Because:

display height = 64 pixels

one page stores 8 vertical pixels

So:

64 / 8 = 8 pages

Memory Structure

The memory is arranged like this:

PAGE 0 → top 8 rows

PAGE 1 → rows 8–15

PAGE 2 → rows 16–23

...

PAGE 7 → bottom 8 rows

Each page contains:128 columns (segments)

So:

8 pages × 128 bytes = 1024 bytes

Important Concept — One Byte = 8 Vertical Pixels - A single byte does NOT represent 8 horizontal pixels.

Instead:   1 byte = 8 VERTICAL pixels

Bit mapping inside one byte:

bit0 → top pixel

bit1

bit2

bit3

bit4

bit5

bit6

bit7 → bottom pixel

Complete Display Layout

Here is the full RAM layout:

COLUMN →

0   1   2   3  ...127

PAGE0  [ ] [ ] [ ] [ ] ... [ ]

PAGE1  [ ] [ ] [ ] [ ] ... [ ]

PAGE2  [ ] [ ] [ ] [ ] ... [ ]

PAGE3  [ ] [ ] [ ] [ ] ... [ ]

PAGE4  [ ] [ ] [ ] [ ] ... [ ]

PAGE5  [ ] [ ] [ ] [ ] ... [ ]

PAGE6  [ ] [ ] [ ] [ ] ... [ ]

PAGE7  [ ] [ ] [ ] [ ] ... [ ]

Each [ ] is one byte controlling 8 vertical pixels.

### Test Commands

sudo make

sudo insmod driver.ko     	# loads → probe() → fills OLED with 0xFF (all ON)

dmesg                     		# → "OLED Probed!!!"

sudo rmmod driver         	# unloads → remove() → fills OLED with 0x00 (all OFF)

dmesg                     		# → "OLED Removed!!!"

### Complete Flow Summary

insmod driver.ko

├── i2c_get_adapter(1)          → get I2C bus 1 handle

├── i2c_new_device(adapter, &oled_info) → create i2c_client at 0x3C

└── i2c_add_driver(&etx_oled_driver)

└── etx_oled_probe() called

├── SSD1306_DisplayInit() → 27 init commands via I2C

└── SSD1306_Fill(0xFF)   → 1024 data bytes → all pixels ON

rmmod driver

└── i2c_del_driver()

└── etx_oled_remove() called

└── SSD1306_Fill(0x00) → 1024 data bytes → all pixels OFF

└── i2c_unregister_device() → client removed

### SSD1306 Write Sequence (for any command or data)

I2C_Write([0x00, 0xAE]):

START → slave_addr(0x3C)+W → ACK

→ send 0x00 (command control byte) → ACK

→ send 0xAE (display OFF command) → ACK

STOP

I2C_Write([0x40, 0xFF]):

START → slave_addr(0x3C)+W → ACK

→ send 0x40 (data control byte) → ACK

→ send 0xFF (fill pixel data) → ACK

STOP

Key takeaway: I2C client driver = i2c_get_adapter + i2c_new_device + i2c_add_driver. The probe function is automatically called when the driver matches the device. Use i2c_master_send/recv for raw byte transfers. For SSD1306: always precede each byte with the correct control byte (0x00=command, 0x40=data).

# Device Driver 38 – Dummy I2C Bus Driver

Part 37 = I2C Client Driver (talks to SSD1306 OLED using existing bus). Part 38 = I2C Bus Driver — who actually generates START/STOP/ACK on the wire?

This tutorial creates a dummy I2C bus driver that intercepts all transfers and just prints them — no real hardware communication. Purpose: understand HOW the bus driver works and what data flows through it.

### Why a Bus Driver?

In Part 37, we used i2c_master_send() and it "just worked". But someone has to generate START, STOP, ADDRESS, ACK/NACK, DATA signals on the physical wire. That is the job of the I2C bus driver.

Client Driver (Part 37)               		Bus Driver (Part 38)

──────────────────────                ───────────────────────

i2c_master_send()  →→→→→→→→→→→→→→→  master_xfer() called

i2c_smbus_read_byte() →→→→→→→→→→→→  smbus_xfer() called

↓

Generates START/STOP/ACK on I2C wire

(or in our dummy: just prints data)

### 2 Core Structures for I2C Bus Driver

#### 1. struct i2c_algorithm — The Transfer Method

struct i2c_algorithm {

/* Called by i2c_master_send(), i2c_master_recv(), i2c_transfer() */

int (*master_xfer)(struct i2c_adapter *adap, struct i2c_msg *msgs, int num);

/* Called by i2c_smbus_read_byte(), i2c_smbus_write_byte(), etc. */

s32 (*smbus_xfer)(struct i2c_adapter *adap, u16 addr,

unsigned short flags, char read_write,

u8 command, int size, union i2c_smbus_data *data);

/* Returns bitmask of what this bus/algorithm supports */

u32 (*functionality)(struct i2c_adapter *);

/* Optional: for I2C slave mode (if CONFIG_I2C_SLAVE enabled) */

int (*reg_slave)(struct i2c_client *client);

int (*unreg_slave)(struct i2c_client *client);

};

#### 2. struct i2c_adapter — The Physical Bus Representation

struct i2c_adapter {

struct module	*owner;    	/* THIS_MODULE                        */

unsigned int	class;    	/* I2C_CLASS_HWMON, I2C_CLASS_SPD... */

const struct i2c_algorithm *algo; 	/* pointer to algorithm structure     */

void	*algo_data;

struct rt_mutex	bus_lock; 	/* mutex protecting bus access        */

int                     	timeout;  	/* timeout in jiffies                 */

int                     	retries;  	/* retry count on failure             */

struct device       dev;      	/* adapter device                     */

int                     	nr;       	/* bus number (used by numbered APIs) */

char                    name[48]; 	/* bus name shown in sysfs            */

/* ... other internal fields ... */

};

### 2 Register APIs

#### Dynamic bus number (kernel assigns any free number)

int i2c_add_adapter(struct i2c_adapter *adapter);

/* Returns 0=success, negative=failure

* Bus number assigned dynamically — check with: tree /sys/bus/i2c/ */

#### Specific bus number (you request a number)

int i2c_add_numbered_adapter(struct i2c_adapter *adap);

/* Set adap->nr BEFORE calling this

* Returns 0 if requested number was free, negative if taken */

#### Remove

void i2c_del_adapter(struct i2c_adapter *adap);

/* Unregisters adapter from I2C subsystem */

### functionality() Flags — Common Values

I2C_FUNC_I2C                 		/* basic I2C support                    */

I2C_FUNC_SMBUS_QUICK         	/* SMBus quick command                  */

I2C_FUNC_SMBUS_BYTE          	/* SMBus send/receive byte              */

I2C_FUNC_SMBUS_BYTE_DATA     	/* SMBus read/write byte data           */

I2C_FUNC_SMBUS_WORD_DATA     	/* SMBus read/write word data           */

I2C_FUNC_SMBUS_BLOCK_DATA    	/* SMBus read/write block data          */

### Load Order (CRITICAL)

sudo insmod driver_bus.ko      	# FIRST: bus must exist before client

tree /sys/bus/i2c/             		# find assigned bus number (e.g., i2c-11)

# Update I2C_BUS_AVAILABLE macro in client driver to match (e.g., 11)

sudo insmod driver_client.ko   	# SECOND: client uses the bus

sudo rmmod driver_client       	# unload client FIRST

sudo rmmod driver_bus          	# then unload bus

### Test Commands & Expected Output

sudo insmod driver_bus.ko

# dmesg: "Bus Driver Added!!!"

tree /sys/bus/i2c/

# Shows: i2c-1 (real), i2c-11 (our dummy — number varies!)

sudo insmod driver_client.ko

# dmesg shows ALL bytes the client tried to send:

# [Count: 0] [etx_i2c_xfer]: [Addr = 0x3c] [Len = 2] [Data] = [0x00] [0xae]

# [Count: 0] [etx_i2c_xfer]: [Addr = 0x3c] [Len = 2] [Data] = [0x00] [0xd5]

# ... (one line per i2c_master_send call from the client driver)

# OLED Probed!!!

sudo rmmod driver_client

sudo rmmod driver_bus

## Part 2 — Client Driver (Modified from Part 37) with Comments

/* ── KEY CHANGE FROM PART 37 ─────────────────────────────────────────────── */

/* Part 37: used real Raspberry Pi I2C bus 1 (physical hardware) */

#define I2C_BUS_AVAILABLE  ( 1 )

/* Part 38: use OUR dummy bus driver's assigned number */

#define I2C_BUS_AVAILABLE  ( 11 )   /* ← get this from: tree /sys/bus/i2c/ */

/* Also: SSD1306_Fill() calls are commented out in probe/remove:

* probe:  SSD1306_DisplayInit() still called (sends 27 init commands)

*         → each command flows through etx_i2c_xfer() → printed in dmesg

*

* remove: nothing sent → "OLED Removed!!!" just logs

*

* Why comment out Fill? To reduce dmesg output (Fill = 1024 transfer calls)

* Init alone gives 27 clean calls to verify the bus driver intercepts correctly.

*/

### Complete Flow Summary

sudo insmod driver_bus.ko

└── i2c_add_adapter() → /sys/bus/i2c/devices/i2c-11/ created

tree /sys/bus/i2c/ → find "i2c-11" (number varies!)

Update client driver: #define I2C_BUS_AVAILABLE (11)

sudo insmod driver_client.ko

├── i2c_get_adapter(11)      → gets our dummy bus adapter

├── i2c_new_device(adapter, &oled_board_info) → create client at 0x3C

└── i2c_add_driver() → etx_oled_probe() called

└── SSD1306_DisplayInit() → 27 × SSD1306_Write()

→ 27 × I2C_Write(buf, 2)

→ 27 × i2c_master_send()

→ 27 × etx_i2c_xfer() in BUS DRIVER!

→ prints each [Addr=0x3c][Len=2][0x00][0xAE] etc.

dmesg shows:

[Count:0][etx_i2c_xfer]: [Addr=0x3c][Len=2][Data]=[0x00][0xae]  ← display OFF cmd

[Count:0][etx_i2c_xfer]: [Addr=0x3c][Len=2][Data]=[0x00][0xd5]  ← clock config

... 27 lines total ...

OLED Probed!!!

Client Driver Added!!!

sudo rmmod driver_client  → FIRST (client depends on bus)

sudo rmmod driver_bus     → THEN  (bus can now be removed)

### Client Driver vs Bus Driver — Roles

CLIENT DRIVER (Part 37/38):

"I want to send 0xAE to slave 0x3C on bus 11"

Calls: i2c_master_send(client, buf, 2)

→ kernel routes to bus driver

BUS DRIVER (Part 38):

"Okay, I'll put it on the wire"

Real bus driver: generates START→0x3C+W→0x00→0xAE→STOP on I2C pins

Dummy bus driver: just prints [Addr=0x3c][Len=2][0x00][0xae]

Key takeaway: Bus driver = i2c_algorithm (master_xfer + smbus_xfer + functionality) + i2c_adapter + i2c_add_adapter(). The master_xfer function is called for every i2c_master_send/recv call. Load bus driver FIRST, then client driver. Unload in reverse order. The dummy bus driver proves that all I2C client API calls flow through the bus driver's master_xfer — which in real hardware would generate physical I2C signals.

# Linux Device Driver Part 39 – Real I2C Bus Driver

Part 38 = Dummy I2C bus driver — just printed data, no real hardware communication. Part 39 = Real I2C bus driver — actually generates START/STOP/ACK/NACK on the I2C wire using bit-banging via GPIO.

One New API Used:

/* Part 38 used: */

i2c_add_adapter(&adapter)           /* dynamic bus number */

/* Part 39 uses: */

i2c_add_numbered_adapter(&adapter)  /* specific bus number (e.g., 5) */

/* Set .nr = 5 in adapter struct first */

### What is Bit-Banging?

Bit-banging = manually controlling GPIO pins to generate I2C protocol signals in software — instead of using a dedicated hardware I2C peripheral.

GPIO 20 = SCL (clock line)

GPIO 21 = SDA (data line)

You control HIGH/LOW on each pin manually to create:

START condition → send address → send data bytes → STOP condition

Limitations of this bit-bang implementation:

- ❌ No I2C READ implemented (TODO)

- ❌ No clock stretching handled

- ❌ No arbitration handling

- ✅ Write-only — enough for SSD1306 OLED initialization

### Hardware Connection

Raspberry Pi GPIO 20 → SCL (I2C clock)

Raspberry Pi GPIO 21 → SDA (I2C data)

Both connected to SSD1306 OLED I2C pins

### I2C Bit-Bang Protocol — Step by Step

#### START Condition

SDA goes HIGH → then SCL goes HIGH → then SDA goes LOW → then SCL goes LOW

#### STOP Condition

SDA goes LOW → SCL goes HIGH → SDA goes HIGH → SCL goes LOW

#### Send Address (7-bit)

Send each bit MSB first: set SDA HIGH/LOW, pulse SCL HIGH then LOW

8th bit = R/W: 0=write, 1=read

Read ACK/NACK: SCL HIGH, read SDA (LOW=ACK, HIGH=NACK)

#### Send Data Byte (8-bit)

Same as address: send 8 bits MSB first, pulse SCL for each bit

Then read ACK/NACK from slave

### usleep_range() — Fine-Grained Delay

#define I2C_DELAY usleep_range(5, 10)

/* Sleeps 5-10 microseconds between I2C signal transitions.

* Needed for proper I2C timing — signals must be stable for enough time.

* Preferred over udelay() in modern kernel (allows scheduler to run). */

### GPIO Direction Switching (Key for I2C bit-bang)

In I2C, the SDA line is bidirectional — master drives it for address/data, slave drives it for ACK. So the GPIO direction switches dynamically:

/* Set pin HIGH — configure as output first, then set value */

gpio_direction_output(pin, 1);

gpio_set_value(pin, 1);

/* Read pin — configure as input first, then read */

gpio_direction_input(pin);

value = gpio_get_value(pin);

### Key Difference in etx_i2c_xfer() (Part 38 vs Part 39)

/* Part 38 (Dummy): just print data */

pr_info("[Addr=0x%x][Len=%d][Data]...", msg->addr, msg->len);

/* Part 39 (Real): actually send on wire */

ETX_I2C_Init();     	/* request GPIOs, set as outputs */

ETX_I2C_Start();    	/* generate START on wire        */

ETX_I2C_Send(addr, buf, len); 	/* send address+data  */

ETX_I2C_Stop();     /* generate STOP on wire         */

ETX_I2C_DeInit();   /* release GPIOs                 */

### Load Order

sudo insmod driver_bus.ko     # FIRST — creates /sys/bus/i2c/devices/i2c-5

sudo insmod driver_client.ko  # SECOND — uses bus 5, probe() called → OLED fills

# OLED shows all pixels ON (0xFF)

sudo rmmod driver_client      # remove() called → OLED clears (0x00)

sudo rmmod driver_bus

### Complete Flow Summary

insmod driver_bus.ko

└── i2c_add_numbered_adapter(nr=5)

→ creates /sys/bus/i2c/devices/i2c-5

insmod driver_client.ko (I2C_BUS_AVAILABLE=5)

└── probe() → SSD1306_DisplayInit() → SSD1306_Fill(0xFF)

→ each i2c_master_send() triggers:

→ etx_i2c_xfer()

├── ETX_I2C_Init()    → gpio_request(20,21), set as outputs

├── ETX_I2C_Start()   → SDA↓ while SCL HIGH on wire

├── ETX_I2C_Send(0x3C, [0x00,0xAE], 2)

│     ├── ETX_I2C_Send_Addr(0x3C, write)

│     │     → 7 bits + W bit sent bit-by-bit, read ACK

│     └── ETX_I2C_Send_Byte(0x00)  → control byte

│           → ETX_I2C_Send_Byte(0xAE) → command byte

├── ETX_I2C_Stop()   → SDA↑ while SCL HIGH on wire

└── ETX_I2C_DeInit() → gpio_free(20,21)

→ OLED displays all pixels ON!

rmmod driver_client → SSD1306_Fill(0x00) → OLED clears

rmmod driver_bus

### Dummy (Part 38) vs Real (Part 39) Comparison

Part 38 (Dummy):                    		Part 39 (Real):

──────────────────                  ──────────────────────────

i2c_add_adapter()                   		i2c_add_numbered_adapter()

(dynamic bus number)               		(.nr=5 → always i2c-5)

master_xfer: just prints data       	master_xfer: bit-bangs GPIO

No GPIO used                        		GPIO 20=SCL, GPIO 21=SDA

No real I2C signals                 		Actual START/STOP/ACK on wire

Client: I2C_BUS_AVAILABLE=11        	Client: I2C_BUS_AVAILABLE=5

Key takeaway: Real I2C bus driver implements master_xfer() using GPIO bit-banging to generate actual I2C protocol signals. Use i2c_add_numbered_adapter() with .nr set when you need a predictable bus number. The bit-bang approach: Init GPIOs → START → send address → send data bytes (each bit manually) → check ACK → STOP → release GPIOs.

# Device Driver 40 – I2C Bus Driver using I2C-GPIO

Part 39 = Real I2C bus driver — bit-banging manually written from scratch (START/STOP/ACK — all custom code, no clock stretching, no arbitration).

Part 40 = Uses kernel's built-in i2c-gpio driver — provides fully-tested, complete bit-bang I2C protocol including clock stretching, arbitration, etc. You just provide GPIO callbacks — kernel handles the rest.

### Why Use i2c-gpio Instead of Writing Your Own?

| Part 39 (Custom Bit-Bang) | Part 40 (i2c-gpio) |
| --- | --- |
| You implement START, STOP, ACK, NACK, data bits manually | Kernel's i2c-gpio implements all of that |
| No clock stretching | ✅ Clock stretching supported |
| No arbitration | ✅ Arbitration supported |
| 300+ lines of GPIO bit-bang code | Just 4 GPIO callback functions |
| Error-prone, untested | Fully tested, production quality |

The idea: You provide 4 simple GPIO callbacks (setsda, setscl, getsda, getscl). The i2c-gpio kernel subsystem handles everything else.

### New Header Required

#include <linux/i2c-algo-bit.h>

/* struct i2c_algo_bit_data and i2c_bit_add_bus/i2c_bit_add_numbered_bus */

### New Structure — struct i2c_algo_bit_data

This is the bridge between YOUR GPIO callbacks and the kernel's bit-bang I2C algorithm.

struct i2c_algo_bit_data {

void *data;       /* optional private data passed to all callbacks         */

/* Mandatory GPIO callbacks — you implement these 4 functions */

void (*setsda)(void *data, int state);  	/* drive SDA HIGH(1) or LOW(0)    */

void (*setscl)(void *data, int state);  		/* drive SCL HIGH(1) or LOW(0)    */

int  (*getsda)(void *data);             		/* read SDA — returns 0 or 1      */

int  (*getscl)(void *data);             		/* read SCL — returns 0 or 1      */

/* Optional callbacks */

int  (*pre_xfer)(struct i2c_adapter *);  	/* called BEFORE each transfer   */

void (*post_xfer)(struct i2c_adapter *); 	/* called AFTER each transfer    */

/* Timing and behavior */

int  udelay;        /* half clock cycle time in microseconds:

min 2us for fast mode (400kHz)

min 5us for standard mode (100kHz) and SMBus

max 50us for SMBus */

int  timeout;       /* timeout in jiffies (e.g., 100 = 100ms)             */

bool can_do_atomic; /* true if callbacks don't sleep (atomic context ok)  */

};

Key insight: i2c-gpio calls setsda/setscl to drive the wire and getsda/getscl to read it. It internally handles START, STOP, ACK/NACK, clock stretching, arbitration — just like a full I2C implementation.

### Key Change to i2c_adapter Structure

/* Part 39: uses .algo pointing to our own i2c_algorithm */

static struct i2c_adapter etx_i2c_adapter = {

.algo   = &etx_i2c_algorithm,   /* points to OUR master_xfer, smbus_xfer */

.nr     = 5,

};

/* Part 40: uses .algo_data pointing to i2c_algo_bit_data callbacks */

static struct i2c_adapter etx_i2c_adapter = {

.algo_data = &etx_bit_data,     /* points to GPIO callbacks for i2c-gpio */

.nr        = 5,

};

/* NO .algo field — i2c-gpio fills .algo automatically when registered */

### 2 New Registration APIs

/* Dynamic bus number (i2c-gpio assigns any free number) */

int i2c_bit_add_bus(struct i2c_adapter *adapter);

/* Specific bus number (uses adap->nr — must be free) */

int i2c_bit_add_numbered_bus(struct i2c_adapter *adap);

/* Returns: 0=success (bus number adap->nr is now i2c-5), negative=error */

Both return 0 on success. Use i2c_bit_add_numbered_bus when client driver needs to know the exact bus number.

### Must Load i2c-gpio Kernel Module First

sudo modprobe i2c-gpio     # load BEFORE your bus driver

# OR add to /etc/modules for auto-loading

# Then load your bus driver

sudo insmod driver_bus.ko

### Load/Unload Order

sudo modprobe i2c-gpio                # 1st: kernel's i2c-gpio subsystem

sudo insmod driver_bus.ko          # 2nd: our bus driver (registers adapter)

sudo insmod driver_client.ko      # 3rd: client driver (uses bus 5)

sudo rmmod driver_client          # reverse: client first

sudo rmmod driver_bus             # then bus

sudo modprobe -r i2c-gpio         # then unload i2c-gpio

### Complete Flow Summary

sudo modprobe i2c-gpio               # load kernel's i2c-gpio module first

sudo insmod driver_bus.ko

├── ETX_I2C_Init()

│     ├── gpio_request(20, "SCL_GPIO") → claim SCL

│     └── gpio_request(21, "SDA_GPIO") → claim SDA

└── i2c_bit_add_numbered_bus()

├── reads etx_bit_data (our GPIO callbacks)

├── sets adapter .algo = i2c-algo-bit built-in algorithm

└── creates /sys/bus/i2c/devices/i2c-5

sudo insmod driver_client.ko (I2C_BUS_AVAILABLE=5)

└── probe() → SSD1306_DisplayInit() → SSD1306_Fill(0xFF)

Each i2c_master_send() flows:

→ i2c-algo-bit master_xfer()

→ calls ETX_I2C_Set_SCL / ETX_I2C_Set_SDA (our GPIO callbacks)

→ generates real I2C START/DATA/ACK/STOP on GPIO 20/21 pins

→ OLED receives commands and lights up!

sudo rmmod driver_client

sudo rmmod driver_bus

├── ETX_I2C_DeInit() → gpio_free(20, 21)

└── i2c_del_adapter() → removes i2c-5

sudo modprobe -r i2c-gpio

### All 3 Bus Driver Approaches — Final Comparison

Part 38 (Dummy):

i2c_add_adapter()

master_xfer() → just prints data (no wire)

No GPIO used

Part 39 (Manual bit-bang):

i2c_add_numbered_adapter()

master_xfer() → ETX_I2C_Init + START + send bits manually + STOP + DeInit

300+ lines, no clock stretch, no arbitration

Part 40 (i2c-gpio):

i2c_bit_add_numbered_bus()

Just 4 GPIO callbacks: setsda, setscl, getsda, getscl

i2c-gpio handles: START, STOP, ACK/NACK, clock stretching, arbitration

~40 lines total — full production-quality I2C

Key takeaway: Part 40 = cleanest, most complete approach. Provide 4 GPIO callbacks in struct i2c_algo_bit_data, set .algo_data in adapter, call i2c_bit_add_numbered_bus(). The kernel's i2c-gpio subsystem handles all bit-banging details — clock stretching, arbitration, timing — fully tested and production-ready.

# Device Driver 41 – SSD1306 I2C Linux Device Driver

A complete, practical SSD1306 OLED driver that:

- Uses the Raspberry Pi's existing I2C bus (bus 1, no custom bus driver needed)

- Provides a /dev/etx_oled character device

- User writes a string → prints on OLED display

- Supports full ASCII character rendering using a 5×8 pixel font table

### SSD1306 Display Structure

128 columns (segments) × 64 rows = 8192 pixels total

64 rows ÷ 8 = 8 pages (PAGE0 to PAGE7)

Each page = 1 row of 128 bytes × 8 bits tall

Column 0 ........ Column 127

PAGE 0  [8 rows tall × 128 bytes wide]

PAGE 1  [8 rows tall × 128 bytes wide]

...

PAGE 7  [8 rows tall × 128 bytes wide]

GDDRAM (Graphics Display Data RAM): 128 × 64 bits = 1024 bytes — each bit controls ONE pixel.

### Character Rendering — 5×8 Font

Each character = 5 columns × 8 rows. Each column = 1 byte (8 bits = 8 pixel rows).

Character 'A' = 5 bytes: [0x7C, 0x12, 0x11, 0x12, 0x7C]

Column 0: 0x7C = 0111 1100 → pixels on rows 1-6 lit

Column 1: 0x12 = 0001 0010 → pixels on rows 1 and 4 lit

Column 2: 0x11 = 0001 0001 → pixels on rows 1 and 7 lit

Column 3: 0x12 = same as column 1

Column 4: 0x7C = same as column 0

### Key SSD1306 Commands

| Command | Command | Description |
| --- | --- | --- |
| 0x00 | COMMAND | This command is used to indicate the next data byte is acted as a command. |
| 0x40~0x7F | Display Start Line | This command sets the Display Start Line register to determine the starting address of display RAM |
| 0xAE | Display OFF | This command is used to turn OFF the OLED display panel. |
| 0xAF | Display ON | This command is used to turn ON the OLED display panel. |
| 0x20 | Memory Addressing Mode | If horizontal address increment mode is enabled by command 20h, after finishing read/write one column data, it is incremented automatically to the next column address. |
| 0x26/0x27 | Continuous Horizontal Scroll | 0x26 – Right horizontal scroll 0x27 – Left horizontal scroll |
| 0x29/0x2A | Continuous Vertical and Horizontal Scroll | 0x29 – Vertical and Right Horizontal Scroll 0x2A – Vertical and Left Horizontal Scroll |
| 0x2E | Deactivate Scroll | This command deactivates the scroll. |
| 0x2F | Activate Scroll | This command activates the scroll if it is configured before. |
| 0x21 | Column Address | This command is used to define the current read/write column address in graphic display data RAM. |
| 0x22 | Page Address | This command is used to define the current read/write Line(Page as per data sheet) address in graphic display data RAM. |
| 0x81 | Contrast Control | This command sets the Contrast Setting of the display. The chip has 256 contrast steps from 00h to FFh. The segment output current increases as the contrast step value increases. |
| 0xA0 | Segment Re-map | This command sets the column address 0 is mapped to SEG0. |
| 0xA1 | Segment Re-map | This command sets the column address 127 is mapped to SEG0. |
| 0xA6 | Normal Display | This command sets the display to normal mode. In the normal display, a RAM data of 1 indicates an “ON” pixel. |
| 0xA7 | Inverse Display | This command sets the display to inverse mode. In the normal display, a RAM data of 0 indicates an “ON” pixel. |
| 0xA8 | Multiplex Ratio | This command sets the display multiplex ratio. |
| 0xC0/0xC8 | COM Output Scan direction | 0xC0 – Normal mode (RESET) Scan from COM0 to COM[N –1] 0xC8- Remapped mode. Scan from COM[N-1] to COM0 Where N is the Multiplex ratio. |
| 0xD3 | Display Offset | This command sets vertical shift by COM from 0d~63d. |
| 0xD5 | Display Clock Divide Ratio/Oscillator Frequency | This command sets the display Clock Divide Ratio and Oscillator Frequency. |

### Setting Cursor Position

To write at a specific page and column:

SSD1306_SetCursor(page, column):

SSD1306_Write(true, 0x21);    /* Set Column Address command */

SSD1306_Write(true, column);  /* start column */

SSD1306_Write(true, 0x7F);    /* end column = 127 */

SSD1306_Write(true, 0x22);    /* Set Page Address command */

SSD1306_Write(true, page);    /* start page */

SSD1306_Write(true, 0x07);    /* end page = 7 */

### Writing a Character

SSD1306_PrintChar('A'):

look up SSD1306_font['A' - 0x20]  → [0x7C, 0x12, 0x11, 0x12, 0x7C]

send each of 5 bytes as display data to GDDRAM

→ OLED draws 'A' at current cursor position

Characters start from ASCII 0x20 (space), so offset = char - 0x20.

### Writing a String

SSD1306_String("Hello"):

for each character: SSD1306_PrintChar(ch)

→ writes each character in sequence left to right

→ display auto-advances column after each byte (horizontal addressing)

### User Interaction — /dev/etx_oled

echo "EmbeTronicX" > /dev/etx_oled

→ write() in driver → SSD1306_String("EmbeTronicX") → displays on OLED

### Test Commands

sudo make                           	# build

sudo insmod driver.ko               	# load → OLED initialized, prints "EmbeTronicX"

sudo su

echo "Hello World" > /dev/etx_oled  	# new text on OLED

cat /dev/etx_oled                   	# read current display content

sudo rmmod driver                   # unload → OLED cleared

### Complete Flow Summary

insmod driver.ko

├── /dev/etx_oled created (char device)

└── I2C probe → SSD1306_DisplayInit() → "EmbeTronicX" shown on OLED

echo "Hello" > /dev/etx_oled

├── etx_write()

│     ├── copy_from_user() → "Hello" in kernel buffer

│     ├── SSD1306_Fill(0x00) → clear display

│     ├── SSD1306_SetCursor(0,0) → cursor to top-left

│     └── SSD1306_String("Hello")

│           └── for each char → SSD1306_PrintChar()

│                 → look up 5-byte font bitmap

│                 → send 5 data bytes to GDDRAM

│                 → send 1 blank byte (spacing)

│           → "Hello" appears on OLED!

rmmod driver

└── etx_oled_remove() → SSD1306_Fill(0x00) → OLED cleared

### SSD1306 Display Writing — How Characters Are Rendered

To display 'A' at page 0, col 0:

SSD1306_SetCursor(0, 0)  → cursor = (page 0, col 0)

Send data bytes: [0x7C, 0x12, 0x11, 0x12, 0x7C, 0x00]

col0   col1  col2  col3  col4  space

Each byte = 8 vertical pixels in that column of page 0

0x7C = 0111 1100 → rows 1-6 are ON in col 0

Display auto-advances column after each byte (horizontal mode).

Next character 'B' goes to col 6 (5+1 space).

Key takeaway: SSD1306 display = 8 pages × 128 columns. Each character = 5 bytes from font table + 1 blank byte for spacing. SSD1306_SetCursor positions write location. SSD1306_String renders any text. The char device /dev/etx_oled lets user write text directly from shell via echo.

# Device Driver 42 - Poll Linux Example

When an app needs to monitor multiple file descriptors simultaneously:

- Blocking I/O: App waits on one fd at a time — misses events from others

- Separate threads: Works but complex, wastes resources for many fds

- Solution: Use IO Multiplexing — monitor multiple fds with ONE call

### 3 IO Multiplexing Options in Linux

|  | select | poll | epoll |
| --- | --- | --- | --- |
| FD limit | 1024 | Unlimited | Unlimited |
| Scales with many fds | ❌ Poor | ❌ Moderate | ✅ Best |
| API | Older | Middle | Newer |

This tutorial covers poll.

### What is poll?

poll() allows a process to monitor one or more file descriptors and determine if they are readable or writable without blocking. When any monitored fd becomes ready, poll() returns.

### Poll Events (Flags)

| Event Flag | Meaning |
| --- | --- |
| POLLIN | Data available for reading (without blocking) |
| POLLRDNORM | Normal data available for reading |
| POLLOUT | Space available for writing (without blocking) |
| POLLWRNORM | Normal write space available |
| POLLHUP | Device disconnected (hang-up) |
| POLLERR | Error condition on device |
| POLLPRI | High-priority (out-of-band) data available |
| POLLRDBAND | Out-of-band data available for reading |
| POLLWRBAND | Out-of-band data can be written |

Common combinations:

Readable device → return (POLLIN | POLLRDNORM)

Writable device → return (POLLOUT | POLLWRNORM)

### User Space — How to Use poll()

#### Step 1 — Fill struct pollfd

struct pollfd {

int   fd;       /* file descriptor to monitor                */

short events;    /* requested events - events you want to WATCH for              */

short revents;  /* returned events - events that ACTUALLY occurred (kernel fills this) */

};

struct pollfd pfd;

pfd.fd     = fd;		// File Descriptor that you want to monitor

pfd.events = (POLLIN | POLLRDNORM | POLLOUT | POLLWRNORM);  /* Event that you 										want to monitor for this fd */

Note: You can add multiple file descriptors by creating struct pollfd pfd[num_of_fd];

#### Step 2 — Call poll()

int poll(struct pollfd *fds, nfds_t nfds, int timeout);

/*

* fds     = array of pollfd structs

* nfds    = number of fds to monitor

* timeout = milliseconds to wait:

*           -1 = block forever (no timeout)

*            0 = return immediately (non-blocking check)

*           5000 = wait up to 5 seconds

*

* Returns: >0 = number of ready fds

*           0 = timeout expired, nothing ready

*          -1 = error

*/

ret = poll(&pfd, 1, 5000);   /* monitor 1 fd, 5 second timeout */

#### Step 3 — Check revents

if (pfd.revents & POLLIN)  { read(fd, buf, size); }   	/* data ready to read  */

if (pfd.revents & POLLOUT) { write(fd, buf, size); }  /* space ready to write */

### Kernel Driver — How to Support poll

#### Step 1 — Declare a Waitqueue - In the Linux device driver, the poll is internally using the wait queue. So we have to declare the wait queue and initialize that. You can use static or dynamic method.

#include <linux/wait.h>

#include <linux/poll.h>

/* Static method: */

DECLARE_WAIT_QUEUE_HEAD(wait_queue_etx_data);

/* Dynamic method: init_waitqueue_head(&wait_queue_etx_data); */

#### Step 2 — Implement poll Function

static unsigned int etx_poll(struct file *filp, struct poll_table_struct *wait)

{

__poll_t mask = 0;

/* Register our waitqueue with the poll table.

* poll_wait() does NOT block here — it just adds the waitqueue

* to the poll_table so kernel knows to call us again when woken up. */

poll_wait(filp, &wait_queue_etx_data, wait);

/* Check conditions and return appropriate event flags */

if (can_read)  { mask |= (POLLIN  | POLLRDNORM); }

if (can_write) { mask |= (POLLOUT | POLLWRNORM); }

return mask;   /* 0 = not ready, non-zero = ready for read/write */

}

#### Step 3 — Register in file_operations

static struct file_operations fops = {

.poll = etx_poll,   /* links poll() syscall to our driver function */

/* ... other operations ... */

};

#### Step 4 — Wake Up Poll from Elsewhere (e.g., sysfs write)

can_read = true;

wake_up(&wait_queue_etx_data);   /* triggers poll_wait to re-evaluate */

### How poll_wait Works Internally

User calls poll()

↓

Kernel calls driver's etx_poll()

↓

poll_wait(filp, &waitqueue, wait_table)

→ adds waitqueue to poll_table

→ returns immediately (does NOT sleep here)

↓

etx_poll() checks conditions → returns 0 (not ready)

↓

Kernel puts process to SLEEP, waiting for wakup

↓

Sysfs write/read → wake_up(&waitqueue)

↓

Kernel wakes process → calls etx_poll() AGAIN

↓

etx_poll() checks conditions → returns POLLIN or POLLOUT

↓

Kernel writes result to pfd.revents

↓

User's poll() returns positive number → user checks revents

### This Tutorial's Concept

SYSFS READ  (cat etx_value)   → driver wakes poll with POLLOUT  → app WRITES data

SYSFS WRITE (echo > etx_value) → driver wakes poll with POLLIN   → app READS data

Two control points (sysfs) drive the user app to read or write via poll.

### Complete Flow Summary

3 terminals: Driver | App | Sysfs

sudo insmod poll_driver.ko

sudo ./poll_app

└── open("/dev/etx_device", O_RDWR|O_NONBLOCK)

└── poll() → etx_poll() → poll_wait() → returns 0 → app blocks/waits

Prints: "Starting poll..." every 5 seconds (timeout)

[Sysfs terminal]: cat /sys/kernel/etx_sysfs/etx_value

└── sysfs_show() called

├── can_write = true

└── wake_up(&wait_queue_etx_data)

→ etx_poll() called again

→ sees can_write=true → returns POLLOUT

→ poll() returns > 0, revents = POLLOUT

App sees POLLOUT → write("User Space") to /dev/etx_device

Prints: "POLLOUT : Kernel_val = User Space"

[Sysfs terminal]: echo "EmbeTronicX" > /sys/kernel/etx_sysfs/etx_value

└── sysfs_store() called

├── strcpy(etx_value, "EmbeTronicX")

├── can_read = true

└── wake_up(&wait_queue_etx_data)

→ etx_poll() called again → returns POLLIN

App sees POLLIN → read() from /dev/etx_device

Prints: "POLLIN : Kernel_val = EmbeTronicX"

Key takeaway: Poll requires: DECLARE_WAIT_QUEUE_HEAD + poll_wait() in .poll file operation + wake_up() when state changes. poll_wait() never blocks — it just registers the waitqueue. Blocking happens in kernel's poll infrastructure between etx_poll() calls. Always return event bitmask from .poll: 0=not ready, POLLIN/POLLOUT=ready.

# Device Driver 43 - Select Linux Example

### What is select?

select() is an older IO multiplexing system call that monitors multiple file descriptors to see if any are ready for reading, writing, or have exceptions — without blocking on any single one.

Key fact: select() and poll() provide the same functionality. Internally, select() also calls poll_wait() in the driver — so the SAME driver works for both select and poll. No driver changes needed!

### select vs poll — Key Differences

|  | select | poll |
| --- | --- | --- |
| Origin | BSD Unix (1983) | SVR3 Unix (1986) |
| Max FD limit | 1024 (FD_SETSIZE) | Unlimited |
| FD set type | fd_set bitmask (static array) | struct pollfd array (dynamic) |
| nfds argument | Must specify highest fd + 1 | Not needed |
| After return | Sets CLEARED and must be rebuilt | Array reused (events vs revents) |
| Timeout | Undefined after return — must reinit | Unchanged after return |
| Scanning | Linear O(n) from 0 to nfds | Checks only requested fds |
| Stack safety | ❌ FD ≥ 1024 corrupts stack | ✅ No such issue |
| Portability | ✅ Every UNIX-like OS | Some Unix systems lack poll |

Warning: Using fd ≥ 1024 with select corrupts the stack — fd_set is a fixed 128-byte (1024-bit) array!

### User Space — How to Use select()

#### Step 1 — Declare fd_set and timeval

fd_set read_fd;    /* set of fds to watch for reading */

fd_set write_fd;   /* set of fds to watch for writing */

struct timeval timeout;   /* how long to wait */

#### Step 2 — Initialize fd_set Macros

FD_ZERO(&read_fd);         /* clear ALL fds from the set — MUST do first */

FD_SET(fd, &read_fd);      /* add fd to read set — watch for read events */

FD_ZERO(&write_fd);        /* clear write set */

FD_SET(fd, &write_fd);     /* add fd to write set */

FD_CLR(fd, &read_fd);      /* remove fd from set — not used here */

FD_ISSET(fd, &read_fd);    /* check if fd is STILL in set after select() returns */

#### Step 3 — Set Timeout

timeout.tv_sec  = 5;   /* 5 seconds */

timeout.tv_usec = 0;   /* 0 microseconds */

/* NULL timeout = block forever */

/* {0,0} = return immediately (non-blocking check) */

#### Step 4 — Call select()

int select(int nfds, fd_set *readfds, fd_set *writefds,

fd_set *exceptfds, struct timeval *timeout);

/*

* nfds      = HIGHEST fd number in ANY set + 1 (e.g., fd=26 → nfds=27)

*             NOT the count of fds! This is a common mistake.

*             Can use FD_SETSIZE (1024) as safe upper bound.

* readfds   = set of fds to monitor for read readiness (NULL = don't watch)

* writefds  = set of fds to monitor for write readiness

* exceptfds = set of fds to monitor for exceptions (NULL = don't watch)

* timeout   = max wait time (NULL = block forever)

*

* Returns: >0 = number of ready fds (check FD_ISSET for which ones)

*           0 = timeout expired, nothing ready

*          -1 = error (check errno)

*

* ⚠️ After return: readfds and writefds are MODIFIED by kernel.

*    Only ready fds remain in the sets. Others are REMOVED.

*    Must call FD_ZERO + FD_SET AGAIN before next select() call!

*    timeout is also UNDEFINED after return — must reinitialize!

*/

ret = select(FD_SETSIZE, &read_fd, &write_fd, NULL, &timeout);

#### Step 5 — Check Results with FD_ISSET

if (FD_ISSET(fd, &read_fd))   { read(fd, buf, len);  }   /* fd is readable */

if (FD_ISSET(fd, &write_fd))  { write(fd, buf, len); }   /* fd is writable */

### Kernel Driver — No Changes Needed!

select() internally uses the same poll_wait() mechanism as poll(). So the exact same driver from Part 42 works here. You only change the user space app.

The driver's .poll file operation handles BOTH poll() and select() system calls.

### Same Driver Concept (from Part 42)

Sysfs READ  (cat etx_value)    → can_write=true → wake_up → POLLOUT → app WRITES

Sysfs WRITE (echo > etx_value) → can_read=true  → wake_up → POLLIN  → app READS

### Complete Flow Summary

sudo insmod poll_driver.ko   ← SAME driver as Part 42, no changes

sudo ./select_app

└── open("/dev/etx_device", O_RDWR|O_NONBLOCK)

└── loop:

FD_ZERO + FD_SET + timeout reinit ← EVERY iteration (required!)

select() → etx_poll() → poll_wait() → returns 0 → blocks 5s

Prints: "Starting Select..." every 5 seconds

[Sysfs]: cat /sys/kernel/etx_sysfs/etx_value

→ sysfs_show() → can_write=true → wake_up()

→ select() wakes → etx_poll() returns POLLOUT

→ FD_ISSET(fd, &write_fd) = true

→ app writes "User Space" to driver

Prints: "WRITE : Kernel_val = User Space"

[Sysfs]: echo "EmbeTronicX" > /sys/kernel/etx_sysfs/etx_value

→ sysfs_store() → can_read=true → wake_up()

→ select() wakes → etx_poll() returns POLLIN

→ FD_ISSET(fd, &read_fd) = true

→ app reads from driver

Prints: "READ : Kernel_val = EmbeTronicX"

### poll vs select — User Space Code Comparison

/* ── poll (Part 42) ── */

struct pollfd pfd;

pfd.fd = fd;

pfd.events = (POLLIN | POLLOUT);          /* set ONCE — not reset each iteration */

ret = poll(&pfd, 1, 5000);               	    /* 5 second timeout in milliseconds     */

if (pfd.revents & POLLIN)  { read();  }      /* check revents (separate from events) */

if (pfd.revents & POLLOUT) { write(); }

/* ── select (Part 43) ── */

fd_set read_fd, write_fd;

timeout.tv_sec = 5;                      			/* MUST reinit EVERY iteration          */

FD_ZERO(&read_fd); FD_SET(fd, &read_fd); 	/* MUST reinit EVERY iteration          */

FD_ZERO(&write_fd); FD_SET(fd, &write_fd);

ret = select(FD_SETSIZE, &read_fd, &write_fd, NULL, &timeout);

if (FD_ISSET(fd, &read_fd))  { read();  }  	/* FD_ISSET checks remaining fds      */

if (FD_ISSET(fd, &write_fd)) { write(); }

Key takeaway: select() uses fd_set bitmasks instead of struct pollfd. The kernel driver is identical for both — both call poll_wait(). The critical difference: with select() you MUST reinitialize fd_set and timeout before every call. select() is limited to 1024 fds and is older but more portable. For new code, prefer poll() or epoll().

# Device Driver 44 – E-Poll Linux Example

### What is epoll?

epoll (event poll) is an enhanced version of poll — specifically designed for large numbers of file descriptors. It is Linux-specific (not portable to other UNIX systems).

Like select and poll, epoll internally calls poll_wait() in the driver. So the exact same kernel driver works for all three (poll, select, epoll). Only the user space app changes.

### Level Triggered vs Edge Triggered

|  | Level Triggered (default) | Edge Triggered (EPOLLET) |
| --- | --- | --- |
| epoll_wait returns when | Condition is true (data exists) | NEW event arrives |
| If unread data remains | ✅ Returns again | ❌ Does NOT return until new data |
| Analogy | Light switch — returns while ON | Door bell — rings only when pressed |

Example: 5KB written to a pipe, you can only read 1KB at a time.

- Level triggered: epoll_wait returns 5 times (once per 1KB read)

- Edge triggered: epoll_wait returns ONCE only. You must read ALL available data immediately before returning, or you miss it.

### Required Header - >   #include <sys/epoll.h>

### 3 epoll System Calls

#### 1. epoll_create() — Create epoll instance

int epoll_create(int size);

/*

* size = hint for kernel (ignored since Linux 2.6.8, but must be > 0)

* Returns: epoll file descriptor (epfd) on success, -1 on error

* Close with close(epoll_fd) when done — frees all resources

*

* Alternative: epoll_create1(flags) — use flags=0 for same behavior

*/

epoll_fd = epoll_create(256);

#### 2. epoll_ctl() — Add/Modify/Remove fd from epoll

int epoll_ctl(int epfd, int op, int fd, struct epoll_event *event);

/*

* epfd  = epoll instance from epoll_create()

* op    = operation to perform:

*           EPOLL_CTL_ADD → add fd to epoll (register for events)

*           EPOLL_CTL_MOD → modify events for fd already in epoll

*           EPOLL_CTL_DEL → remove fd from epoll

* fd    = the file descriptor to watch

* event = events to watch and data to associate with fd

* Returns: 0=success, -1=error

*/

struct epoll_event ev;

ev.data.fd = fd;                 /* store fd in event's data union */

ev.events  = (EPOLLIN | EPOLLOUT); /* watch for read + write readiness */

epoll_ctl(epoll_fd, EPOLL_CTL_ADD, fd, &ev);

#### 3. epoll_wait() — Wait for events

int epoll_wait(int epfd, struct epoll_event *events, int maxevents, int timeout);

/*

* epfd      = epoll instance

* events    = array where kernel writes ready events (returned to caller)

* maxevents = max number of events to return (array size)

* timeout   = milliseconds to wait:

*               -1 = block forever

*                0 = return immediately (non-blocking)

*             5000 = wait up to 5 seconds

*

* Returns: > 0 = number of ready fds (events filled)

*            0 = timeout expired

*           -1 = error

*

* KEY ADVANTAGE vs poll/select:

*   Returns ONLY the ready fds — no scanning all fds!

*   Performance is O(1) for ready events regardless of total fd count.

*/

ret = epoll_wait(epoll_fd, events, MAX_EVENTS, 5000);

### epoll_event Event Flags

| Flag | Meaning |
| --- | --- |
| EPOLLIN | fd ready for reading |
| EPOLLOUT | fd ready for writing |
| EPOLLPRI | Urgent/out-of-band data available |
| EPOLLERR | Error condition on fd |
| EPOLLHUP | Hang-up (disconnect) on fd |
| EPOLLET | Use Edge Triggered mode (default = Level Triggered) |
| EPOLLONESHOT | Disable fd in epoll after first event (must re-arm with EPOLL_CTL_MOD) |
| EPOLLRDHUP | Peer closed connection (useful with edge-triggered) |
| EPOLLWAKEUP | Prevent system suspend while event pending |
| EPOLLEXCLUSIVE | Avoid thundering herd — only one waiter woken |

### epoll vs poll vs select — Performance Comparison

| FDs Monitored | poll (ms) | select (ms) | epoll (ms) |
| --- | --- | --- | --- |
| 10 | 0.61 | 0.73 | 0.41 |
| 100 | 2.9 | 3.0 | 0.42 |
| 1000 | 35 | 35 | 0.53 |
| 10000 | 990 | 930 | 0.66 |

epoll is nearly O(1) — performance barely changes as fd count grows. poll and select are O(n) — they scan all registered fds every call.

### epoll vs poll vs select — Feature Comparison

|  | select | poll | epoll |
| --- | --- | --- | --- |
| Max FDs | 1024 | Unlimited | Unlimited |
| Performance at scale | ❌ O(n) | ❌ O(n) | ✅ O(1) |
| Returns only ready fds | ❌ No (scan all) | ❌ No | ✅ Yes |
| Reinit before each call | ❌ Yes | ✅ No | ✅ No |
| Trigger mode | Level only | Level only | Level + Edge |
| Portability | ✅ POSIX | ✅ POSIX | ❌ Linux only |
| Kernel driver change needed | ❌ None | ❌ None | ❌ None |

### Kernel Driver — Same as Part 42 (No Changes)

epoll() calls the same poll_wait() as poll() and select(). The driver's .poll = etx_poll handles all three.

## Driver Code — Same as Part 42/43

### Complete Flow Summary

sudo insmod poll_driver.ko   ← SAME driver as Parts 42, 43

sudo ./epoll_app

├── open("/dev/etx_device", O_RDWR|O_NONBLOCK)

├── epoll_create(256)     → epoll_fd created

├── epoll_ctl(ADD, fd)    → fd registered ONCE (no reinit needed!)

└── loop:

epoll_wait(5000ms) → etx_poll() → poll_wait() → blocks 5s → timeout

Prints: "Starting epoll..." every 5 seconds

[Sysfs]: cat /sys/kernel/etx_sysfs/etx_value

→ sysfs_show() → can_write=true → wake_up()

→ epoll_wait returns ret=1, events[0].events=EPOLLOUT

→ (events[0].events & EPOLLOUT) == EPOLLOUT → write("User Space")

Prints: "EPOLLOUT : Kernel_val = User Space"

[Sysfs]: echo "EmbeTronicX" > /sys/kernel/etx_sysfs/etx_value

→ sysfs_store() → can_read=true → wake_up()

→ epoll_wait returns ret=1, events[0].events=EPOLLIN

→ (events[0].events & EPOLLIN) == EPOLLIN → read() from driver

Prints: "EPOLLIN : Kernel_val = EmbeTronicX"

## All 3 IO Multiplexing APIs — User Space Code Comparison

/* ── poll (Part 42) ── */

struct pollfd pfd = { fd, POLLIN|POLLOUT, 0 };

// loop:

poll(&pfd, 1, 5000);

if (pfd.revents & POLLIN)  read();

if (pfd.revents & POLLOUT) write();

/* ── select (Part 43) ── */

// loop: (REINIT EVERY ITERATION!)

FD_ZERO(&rfd); FD_SET(fd, &rfd);   // ← must rebuild each time

FD_ZERO(&wfd); FD_SET(fd, &wfd);

timeout = {5,0};                    // ← must reset each time

select(FD_SETSIZE, &rfd, &wfd, NULL, &timeout);

if (FD_ISSET(fd, &rfd))  read();

if (FD_ISSET(fd, &wfd))  write();

/* ── epoll (Part 44) ── */

epoll_fd = epoll_create(256);       // once: before loop

ev.data.fd = fd; ev.events = EPOLLIN|EPOLLOUT;

epoll_ctl(epoll_fd, EPOLL_CTL_ADD, fd, &ev); // once: before loop

// loop: (NO reinit needed!)

ret = epoll_wait(epoll_fd, events, MAX_EVENTS, 5000);

for (n=0; n<ret; n++) {

if (events[n].events & EPOLLIN)  read(events[n].data.fd, ...);

if (events[n].events & EPOLLOUT) write(events[n].data.fd, ...);

}

Key takeaway: epoll = best IO multiplexing for large-scale applications. Three steps: epoll_create + epoll_ctl (once, before loop) + epoll_wait (in loop). Returns ONLY ready fds — O(1) performance. Supports Level and Edge trigger. Linux-specific (not portable). Same kernel driver as poll and select — all use poll_wait() internally.

# Device Driver 45 – Softirq Linux Example

### What is Softirq?

Softirq (Software Interrupt Request) is the lowest-level, fastest bottom-half mechanism in Linux. It runs in atomic context (not process context) — meaning it cannot sleep. Tasklets are built ON TOP of softirqs.

Use softirq when: You need absolute maximum performance for time-critical deferred work. For most drivers, tasklets are sufficient and easier to use.

### Bottom Half Mechanisms — All 4

| Mechanism | Context | Can Sleep? | Use When |
| --- | --- | --- | --- |
| Workqueue | Process | ✅ Yes | Heavy work, needs sleep |
| Threaded IRQ | Process | ✅ Yes | Cleaner threaded approach |
| Softirq | Atomic | ❌ No | Highest performance, timing-critical |
| Tasklet | Atomic | ❌ No | General deferred work (built on softirq) |

### Why Softirq Requires Kernel Source Changes

Unlike tasklets (dynamically created) and workqueues, softirqs are statically allocated at compile time. They live in a fixed array:

static struct softirq_action softirq_vec[NR_SOFTIRQS];

/* Max 32 entries — currently 11 used by kernel itself */

Currently registered softirqs (in <linux/interrupt.h>):

enum {

HI_SOFTIRQ=0,             /* highest priority — used by tasklets */

TIMER_SOFTIRQ,           /* timer subsystem                     */

NET_TX_SOFTIRQ,         /* network transmit                    */

NET_RX_SOFTIRQ,         /* network receive                     */

BLOCK_SOFTIRQ,           /* block I/O                           */

IRQ_POLL_SOFTIRQ,      /* IRQ polling                         */

TASKLET_SOFTIRQ,         /* tasklets (normal priority)          */

SCHED_SOFTIRQ,            /* scheduler                           */

HRTIMER_SOFTIRQ,       /* high-res timers                     */

RCU_SOFTIRQ,                /* RCU — always last                   */

NR_SOFTIRQS                 /* count — add yours BEFORE this */

};

To add your own, you MUST edit the kernel source and recompile.

### Kernel Changes Required (2 files)

#### 1. include/linux/interrupt.h — Add your entry

enum {

/* ... existing entries ... */

RCU_SOFTIRQ,

EMBETRONICX_SOFT_IRQ,   /* ← add yours BEFORE NR_SOFTIRQS */

NR_SOFTIRQS

};

#### 2. kernel/softirq.c — Export the 3 APIs

EXPORT_SYMBOL(raise_softirq_irqoff);  /* ← add after each function */

EXPORT_SYMBOL(raise_softirq);

EXPORT_SYMBOL(open_softirq);

Without exporting, your loadable module cannot call these functions → insmod fails.

### 4 Steps to Use Softirq

#### Step 1 — Declare (in kernel enum — already done above)

#### Step 2 — Create Handler

void softirq_handler(struct softirq_action *action)

{

/* bottom half work — NO sleep, NO mutex */

}

#### Step 3 — Register Handler

open_softirq(EMBETRONICX_SOFT_IRQ, gpio_interrupt_softirq_handler);

/*

* Links your handler to the softirq entry.

* Call once in driver init() — before any raise_softirq() calls.

* Fills softirq_vec[EMBETRONICX_SOFT_IRQ].action = handler

*/

#### Step 4 — Trigger from ISR

raise_softirq(EMBETRONICX_SOFT_IRQ);

/* Marks softirq as PENDING.

* The handler runs when do_softirq() is called next.

* Alternative: raise_softirq_irqoff() — if IRQs are already disabled. */

### When Does Softirq Handler Run?

After raise_softirq() marks it pending, it executes in:

- Return from hardware interrupt (most common — right after ISR)

- ksoftirqd kernel thread — dedicated per-CPU thread for softirqs

- Any code explicitly calling do_softirq() (e.g., networking subsystem)

### Key Rules for Softirq

- ❌ A softirq CANNOT preempt another softirq

- ✅ Same softirq CAN run on multiple CPUs simultaneously (unlike tasklets!)

- ✅ Only hardware IRQs can preempt a running softirq

- ❌ CANNOT sleep, use mutex, or do time-consuming work

- ✅ Must be thread-safe — may run on multiple CPUs at same time → use spinlock

### Workaround — No Kernel Modification

If you cannot modify the kernel, hijack an existing softirq (educational purpose only — don't do in production):

/* Use HRTIMER_SOFTIRQ instead of creating your own */

open_softirq(HRTIMER_SOFTIRQ, gpio_interrupt_softirq_handler);

raise_softirq(HRTIMER_SOFTIRQ);

/* ⚠️ May break hrtimer functionality! Educational use ONLY */

### Test Commands

sudo make                      # build (with custom kernel that has EMBETRONICX_SOFT_IRQ)

sudo insmod driver.ko          # load

# Press button on GPIO 25

dmesg

# → "Interrupt Occurred : GPIO_21_OUT : 1"   ← softirq handler toggled LED

sudo rmmod driver              # unload

### Complete Flow Summary

Custom kernel compilation required:

interrupt.h → add EMBETRONICX_SOFT_IRQ to enum

softirq.c   → EXPORT_SYMBOL for 3 APIs

insmod driver.ko

├── GPIO 21 configured as OUTPUT (LED)

├── GPIO 25 configured as INPUT (button)

├── request_irq(GPIO_25_IN, gpio_irq_handler, RISING)

└── open_softirq(EMBETRONICX_SOFT_IRQ, gpio_interrupt_softirq_handler)

Button pressed (GPIO 25 → RISING edge)

└── gpio_irq_handler() [TOP HALF — runs immediately]

├── debounce check — ignore if < 20 jiffies

└── raise_softirq(EMBETRONICX_SOFT_IRQ)  → mark pending, return fast

└── [ISR returns]

└── do_softirq() called automatically

└── gpio_interrupt_softirq_handler() [BOTTOM HALF]

├── led_toggle ^= 1

└── gpio_set_value(21, led_toggle) → LED toggles!

rmmod driver

└── free_irq() → no more ISR → no more raise_softirq() → cleanup

### All Bottom Half Mechanisms Compared (Final)

ISR fires → raise bottom half

Workqueue:   schedule_work()     → process context → CAN  sleep → most flexible

Tasklet:     tasklet_schedule()  → atomic context  → CANNOT sleep → runs on 1 CPU

Softirq:     raise_softirq()    → atomic context  → CANNOT sleep → runs on N CPUs

FASTEST: Softirq (runs on multiple CPUs, lowest overhead)

SAFEST:  Workqueue (process context, can sleep, use mutex)

COMMON:  Tasklet (simple, built on softirq, can't run in parallel with itself)

Key takeaway: Softirq is the lowest-level, fastest bottom-half mechanism — tasklets are built on it. Requires kernel source modification (add enum entry + EXPORT_SYMBOL). Use open_softirq() to register, raise_softirq() to trigger from ISR. Unlike tasklets, same softirq CAN run on multiple CPUs simultaneously — must use spinlock for shared data. For most drivers, use tasklets instead — only use softirq when maximum performance is required.

# Device Driver 46 – Threaded IRQ in Linux

### What is Threaded IRQ?

Threaded IRQ is the cleanest and simplest bottom-half mechanism. Instead of managing workqueues or tasklets yourself, you just give the kernel TWO functions:

- Top Half (handler) — fast ISR, called immediately on interrupt

- Bottom Half (thread_fn) — slow work, kernel creates a dedicated thread for it

The kernel handles everything else — no manual workqueue setup, no schedule_work(), no tasklet_schedule().

### Why Threaded IRQ?

Main goal: Reduce the time interrupts are disabled.

Traditional ISR problem:

Interrupt fires → disable ALL interrupts → do ALL work → re-enable interrupts

→ Other interrupts MISSED during processing!

Threaded IRQ solution:

Interrupt fires → ISR does MINIMAL work (<100μs) → returns IRQ_WAKE_THREAD

→ interrupts RE-ENABLED immediately

→ kernel thread runs bottom half with interrupts enabled

→ Other interrupts NOT missed!

### One New API — request_threaded_irq()

Replaces request_irq(). Just one extra parameter “irq_handler_t thread_fn”.

int request_threaded_irq(

unsigned int irq,                    /* IRQ number from gpio_to_irq()        */

irq_handler_t handler,          /* Top Half (ISR) — fast, atomic context */

irq_handler_t thread_fn,       /* Bottom Half — process context, CAN sleep */

unsigned long irqflags,         /* same flags as request_irq()          */

const char *devname,            /* device name in /proc/interrupts       */

void *dev_id                    /* unique cookie for shared IRQ          */

);

/* Returns: 0=success, negative=error */

#### Key Parameter Notes

| Parameter | Notes |
| --- | --- |
| handler | If NULL + thread_fn != NULL → kernel uses default primary handler. Must return IRQ_WAKE_THREAD to invoke thread_fn. |
| thread_fn | If NULL → no thread created (same as request_irq). Must return IRQ_HANDLED when done. |

### 3 Return Values from Top Half (handler)

| Return Value | Meaning |
| --- | --- |
| IRQ_HANDLED | ISR handled everything — thread_fn NOT called |
| IRQ_NONE | This interrupt wasn't mine |
| IRQ_WAKE_THREAD | ISR done — wake the thread to run thread_fn |

IRQ_WAKE_THREAD is the KEY — it's what triggers the bottom half thread.

### Thread Handler Rules

The thread_fn runs in PROCESS CONTEXT (a dedicated kernel thread):

✅ CAN sleep        ✅ CAN use mutex      ✅ CAN do heavy work

✅ CAN access user space via copy_to/from_user (if needed)

Must return IRQ_HANDLED when done.

### What If thread_fn Takes Too Long?

If thread_fn is still running when the next IRQ fires:

- The NEW interrupt fires → handler() (top half) is called normally

- thread_fn for the new IRQ is queued but not started until old one finishes

- Other interrupts (GPIO, UART, etc.) are NOT missed — still processed normally

- Only the SAME thread_fn may be delayed

### Threaded IRQ vs Other Bottom Halves

|  | Workqueue | Tasklet | Softirq | Threaded IRQ |
| --- | --- | --- | --- | --- |
| Context | Process | Atomic | Atomic | Process |
| Can sleep | ✅ | ❌ | ❌ | ✅ |
| Setup | Manual | Manual | Manual+kernel mod | Automatic |
| Kernel thread | Created by you | No | No | Auto-created |
| Scheduling | You call schedule_work | You call tasklet_schedule | You call raise_softirq | Kernel handles it |
| Simplicity | Moderate | Simple | Complex | Simplest |

### Test Commands & Expected Output

sudo make

sudo insmod driver.ko

# Press button on GPIO 25

dmesg

GPIO_irqNumber = 67

Device Driver Insert...Done!!!

Interrupt(IRQ Handler)                    ← Top half ran first (fast)

Interrupt(Threaded Handler) : GPIO_21_OUT : 1  ← Bottom half ran after (thread)

sudo rmmod driver

### Complete Flow Summary

insmod driver.ko

├── GPIO 21 = OUTPUT (LED), GPIO 25 = INPUT (button)

└── request_threaded_irq()

→ IRQ registered

→ kernel creates [irq/67-etx_device] kernel thread (sleeping)

Button pressed (GPIO 25 RISING)

└── gpio_irq_handler() [TOP HALF — atomic, fast]

├── debounce check

├── pr_info("Interrupt(IRQ Handler)")

└── return IRQ_WAKE_THREAD → kernel wakes [irq/67-etx_device] thread

└── gpio_interrupt_thread_fn() [BOTTOM HALF — process context]

├── led_toggle ^= 1

├── gpio_set_value(21, led_toggle) → LED toggles!

├── pr_info("Interrupt(Threaded Handler) : GPIO_21_OUT : 1")

└── return IRQ_HANDLED → thread goes back to sleep

rmmod driver

└── free_irq() → ISR unregistered + [irq/67-etx_device] thread killed

### All Bottom Half Mechanisms — Final Comparison

/* Workqueue (manual setup) */

DECLARE_WORK(work, workqueue_fn);              /* init once */

schedule_work(&work);                          		  /* in ISR    */

workqueue_fn(struct work_struct *work) { ... } /* can sleep */

/* Tasklet (manual setup) */

DECLARE_TASKLET(tasklet, tasklet_fn, data);    /* init once */

tasklet_schedule(&tasklet);                    		/* in ISR    */

tasklet_fn(unsigned long arg) { ... }          	/* NO sleep  */

/* Softirq (kernel modification required) */

open_softirq(MY_SOFTIRQ, softirq_fn);          /* init once */

raise_softirq(MY_SOFTIRQ);                     	/* in ISR    */

softirq_fn(struct softirq_action *) { ... }    /* NO sleep  */

/* Threaded IRQ — SIMPLEST */

request_threaded_irq(irq, top_half, bottom_half, ...); 	/* registers both */

top_half()    { return IRQ_WAKE_THREAD; }      		/* just wake thread       */

bottom_half() { /* heavy work */; return IRQ_HANDLED; } 	/* CAN sleep    */

Key takeaway: Threaded IRQ = simplest and cleanest bottom half. Just pass TWO functions to request_threaded_irq() — top half returns IRQ_WAKE_THREAD, bottom half does the work and returns IRQ_HANDLED. No manual workqueue creation, no tasklet setup, no kernel modifications. The kernel creates a dedicated thread automatically. Preferred over workqueue/tasklet when you want the simplest interrupt handler split.

# Device Driver 47 - SPI Protocol Driver

### What This Tutorial Covers

Writing an SPI Protocol Driver that communicates with an SSD1306 OLED display over SPI. Loads driver → displays text and logo. Unloads driver → clears display.

### SPI Subsystem — 3 Components

SPI Core          → kernel-provided, hardware-independent layer

APIs for registration, management

Location: kernel/drivers/spi/spi.c

SPI Controller Driver   → platform-specific (SoC-specific) driver

controls actual SPI hardware pins (MOSI, MISO, CLK, CS)

Similar to I2C adapter/bus driver

Written by SoC manufacturer

SPI Protocol Driver   → YOUR driver — communicates with a specific slave device

Similar to I2C client driver (this tutorial)

### Enable SPI on Raspberry Pi

# Add to /boot/config.txt:

dtoverlay=spi1-1cs,cs0_spidev=disabled

# Enables SPI1 and disables spidev app so driver can use bus

### SPI Connections (Hardware)

RPi GPIO 20 → SDA (MOSI)

RPi GPIO 21 → SCL (CLK)

RPi GPIO 24 → RES (Reset)

RPi GPIO 23 → DC  (Data/Command)

3.3V        → Vcc

GND         → GND

### Required Header- > #include <linux/spi/spi.h>   /* all SPI driver APIs */

### 5 Steps to Write SPI Protocol Driver

#### Step 1 — Get the SPI Controller

struct spi_master *master;

master = spi_busnum_to_master(1);  /* get SPI bus 1 controller */

/* Returns pointer to spi_controller, or NULL if not found */

/* Must be released when done */

#### Step 2 — Fill Board Info + Create Slave Device

struct spi_board_info {

char modalias[SPI_NAME_SIZE];

const void      *platform_data;

const struct property_entry *properties;

void *controller_data;

int irq;

u32 max_speed_hz;

u16 bus_num;

u16 chip_select;

u32 mode;

};

modalias – This string is used to identify the driver.

platform_data – the particular data stored there is driver-specific.

properties – Additional device properties for the device.

controller_data – some controllers need hints about hardware setup, e.g. for DMA.

irq – Interrupt number

max_speed_hz – Maximum speed that slave supports. Refer to the slave’s datasheet.

bus_num– SPI bus number that slave is going to connect.

chip_select – Chip select that you want to use.

mode – SPI mode. Refer to the slave’s datasheet.

struct spi_device * spi_new_device( struct spi_controller *ctlr, struct spi_board_info *chip )

where,

ctlr – Controller to which device is connected. In our case, it has been returned by spi_busnum_to_master().

chip – SPI slave device structure.

Return - It returns the new device, or NULL.

struct spi_board_info etx_spi_device_info = {

.modalias     = "etx-spi-ssd1306-driver",  	/* driver name */

.max_speed_hz = 4000000,    		/* 4MHz — check slave datasheet */

.bus_num      = 1,           			/* SPI bus 1 */

.chip_select  = 0,           			/* CS0 = GPIO 18 on RPi */

.mode         = SPI_MODE_0   		/* SPI mode — check datasheet */

};

struct spi_device *etx_spi_device;

etx_spi_device = spi_new_device(master, &etx_spi_device_info);

/* Returns: new spi_device pointer on success, NULL on failure */

/* Internally calls spi_alloc_device() + spi_add_device() */

#### Step 3 — Configure SPI Device  Till now, we have added the slave device to the controller driver. If we change any mode or clock frequency, then we have to call this below API to take effect on the bus.

int spi_setup(struct spi_device *spi)

where,

spi – the device whose settings are being modified.

etx_spi_device->bits_per_word = 8;   /* 8 bits per word */

ret = spi_setup(etx_spi_device);         /* apply configuration to bus */

/* Returns: 0=success, negative=error */

#### Step 4 — Transfer Data We have done with our initialization. Now we can transfer the data using the below APIs.

| Function | Type | Use |
| --- | --- | --- |
| spi_sync_transfer(spi, xfers, num) | Sync (blocking) | Normal transfers — waits until done |
| spi_async(spi, message) | Async (non-blocking) | Can use in interrupt context |
| spi_write_then_read(spi, tx, n_tx, rx, n_rx) | Sync | Write then read in one operation |

This function will be blocked until the master completes the transfer. int spi_sync_transfer(struct spi_device *spi, struct spi_transfer *xfers, unsigned int num_xfers) where, spi – a device with which data will be exchanged. xfers – An array of spi_transfers. num_xfers – Number of items in the xfer array. Return - It returns zero on success, or else a negative error code.

This API is used to transfer the data asynchronously. This call may be used in_irq and other contexts which can’t sleep, int spi_async(struct spi_device *spi, struct spi_message *message) where, spi – a device with which data will be exchanged. message – describes the data transfers, including completion callback. Return - It returns zero on success, or else a negative error code.

This API is used to write the data and is followed by a read. This is synchronous. int spi_write_then_read(struct spi_device * spi, const void * txbuf, unsigned n_tx, void * rxbuf, unsigned n_rx) where, spi – a device with which data will be exchanged. txbuf – data to be written. n_tx – size of txbuf (in bytes). rxbuf – the buffer into which data will be read. n_rx – size of rxbuf (in bytes).

/* Simple synchronous write example: */

struct spi_transfer tr = {

.tx_buf = &data,    	/* data to send */

.rx_buf = &rx,      	/* buffer for received data */

.len    = 1,        		/* number of bytes */

};

spi_sync_transfer(etx_spi_device, &tr, 1);  /* 1 = one transfer */

#### Step 5 — Remove Device

void spi_unregister_device(struct spi_device *spi)

where,

spi – a device that needs to be removed.

spi_unregister_device(etx_spi_device);   /* unregister slave from SPI controller */

https://elixir.bootlin.com/linux/v7.0.10/source/drivers/spi/spi.c

### SSD1306 via SPI — DC Pin Difference vs I2C

In I2C: control byte (0x00 or 0x40) sent before each byte to indicate command/data. In SPI: dedicated GPIO (DC pin) used to indicate command/data:

DC = LOW  → next byte is COMMAND

DC = HIGH → next byte is DATA (pixel data)

Also needs Reset (RST) GPIO:

RST = LOW → hardware reset active

RST = HIGH → normal operation

### Source Code Structure

spi_ssd1306_driver.c  → SPI protocol driver (Linux driver framework)

ssd1306.c             → SSD1306 display functions (display logic)

ssd1306.h             → macros and function declarations

### Test Commands

sudo make

sudo insmod spissd1306.ko

# OLED: shows text scrolling → then displays EmbeTronicX logo

sudo rmmod spissd1306

# OLED: cleared

### SPI vs I2C Protocol Driver Comparison

I2C Protocol Driver:             			SPI Protocol Driver:

─────────────────────            ─────────────────────

i2c_get_adapter(1)               			spi_busnum_to_master(1)

i2c_new_device(adapter, info)    			spi_new_device(master, info)

[no bits_per_word needed]        			etx_spi_device->bits_per_word = 8

[no spi_setup]                   				spi_setup(etx_spi_device)

i2c_add_driver(driver)           			[no driver registration needed]

i2c_master_send(client, buf, n)  			spi_sync_transfer(dev, xfers, num)

i2c_unregister_device(client)    			spi_unregister_device(dev)

Command/Data:

I2C: control_byte(0x00/0x40) + data_byte

SPI: DC_GPIO = LOW(cmd) or HIGH(data) + data_byte via SPI

### Complete Flow Summary

insmod spissd1306.ko

├── spi_busnum_to_master(1)     → get SPI1 controller

├── spi_new_device(info)        → create slave at CS0

├── bits_per_word=8 + spi_setup()

└── ETX_SSD1306_DisplayInit()   → RST pulse + 27 init commands via SPI

└── Display text (scrolling) for 9 seconds → shows logo

rmmod spissd1306

├── ETX_SSD1306_ClearDisplay()  → clears OLED

├── ETX_SSD1306_DisplayDeInit() → frees RST + DC GPIOs

└── spi_unregister_device()     → removes slave from SPI controller

Key takeaway: SPI protocol driver = spi_busnum_to_master + spi_new_device + spi_setup + spi_sync_transfer. The main difference from I2C: SPI uses a dedicated DC GPIO to distinguish commands from data (instead of a control byte). SPI also needs a Reset (RST) GPIO. The display functions are nearly identical to I2C — only the write primitive changes.

# Device Driver 48 - BMP280 I2C Pressure sensor Driver

### What This Tutorial Does

Integrates the Bosch BMP280 pressure sensor driver (from Bosch's GitHub) with a Linux I2C client driver. When user runs cat /dev/etx_device → reads atmospheric pressure from sensor → prints to kernel log.

### BMP280 Sensor — Key Facts

- Absolute barometric pressure sensor by Bosch

- Interfaces: I2C and SPI (tutorial uses I2C)

- I2C addresses: 0x76 (SDO → GND) or 0x77 (SDO → VDDIO)

- Power modes: Normal, Forced, Sleep

- Features: oversampling, IIR filter, configurable data rate

### Hardware Connection (RPi 3B → BMP280)

RPi PIN 1  (3.3V) → Vcc

RPi PIN 9  (GND)  → GND

RPi PIN 5  (SCL)  → SCL

RPi PIN 3  (SDA)  → SDA

RPi PIN 17 (3.3V) → CSB  (selects I2C mode — MUST tie to VDDIO)

RPi PIN 6  (GND)  → SDO  (sets I2C address = 0x76)

CSB to VDDIO = selects I2C mode (not SPI). SDO to GND = address 0x76.

### Approach — Bosch Driver + Linux I2C

Bosch provides a complete BMP280 API in C (bmp280.c, bmp280.h, bmp280_defs.h). This tutorial bridges the Bosch API to the Linux I2C subsystem via two callback functions:

Bosch API:  bmp280_init(), bmp280_get_config(), bmp280_get_uncomp_data()

↓ calls bmp.read / bmp.write

Custom bridge:  i2c_reg_read() + i2c_reg_write()

↓ calls Linux I2C

Linux I2C:  i2c_master_send() + i2c_master_recv()

↓ physical I2C bus

BMP280 Sensor

### Files Required

bmp280.c         ← Bosch BMP280 driver (from GitHub)

bmp280.h         ← Bosch BMP280 header

bmp280_defs.h    ← Bosch BMP280 definitions

driver.c         ← Your Linux I2C driver (this tutorial)

Download from: github.com/BoschSensortec/BMP280_driver

https://github.com/nitanshnagpal/BMP280_Linux_Driver

### Known Compilation Fix

On 32-bit RPi OS:

/* Add to driver.c to avoid "__aeabi_ldivmod undefined" error */

#define BMP280_DISABLE_64BIT_COMPENSATION 1

### Test Commands

sudo make all                    # build

sudo insmod driver.ko            # load → probe() → sensor initialized

sudo cat /dev/etx_device         # read → raw + compensated pressure value

dmesg                            # → "Pressure value = XXXXX"

sudo rmmod driver                # unload → sensor put to sleep

### Complete Flow Summary

insmod driver.ko

├── /dev/etx_device created (char device)

├── i2c_get_adapter(1) → I2C bus 1

├── i2c_new_client_device(0x76) → BMP280 client

└── i2c_add_driver → etx_bmp_probe()

├── bmp.read = i2c_reg_read   ← link Bosch API to Linux I2C

├── bmp.write = i2c_reg_write ← link Bosch API to Linux I2C

├── bmp280_init()             → reads chip ID + calibration

├── bmp280_set_config()       → filter=2, oversampling=4x, ODR=1Hz

├── bmp280_set_power_mode(NORMAL) → continuous measurement

└── bmp280_get_comp_pres_32bit() → first reading printed

sudo cat /dev/etx_device

└── etx_read()

├── bmp280_get_uncomp_data()   → raw ADC from sensor registers

├── bmp280_get_comp_pres_32bit() → calibrated Pa * 256

└── copy_to_user()             → sends uint32_t to user

rmmod driver

└── etx_bmp_remove() → bmp280_set_power_mode(SLEEP) → sensor sleeps

└── cleanup

### Key Architecture Insight

┌─────────────────────────────────────────────┐

│          Bosch BMP280 API (bmp280.c)        		              │

│   bmp280_init(), bmp280_get_uncomp_data()   		   │

│         calls bmp.read / bmp.write          		              │

└──────────────┬──────────────────────────────┘

│ function pointers

┌──────────────▼──────────────────────────────┐

│    Bridge Functions (driver.c)              			              │

│   i2c_reg_read()  i2c_reg_write()          			  │

│      calls i2c_master_send/recv             			  │

└──────────────┬──────────────────────────────┘

│ Linux I2C APIs

┌──────────────▼──────────────────────────────┐

│    Linux I2C Subsystem                      			              │

│   i2c_master_send() / i2c_master_recv()     		              │

│         physical I2C bus (SDA/SCL)          	 		  │

└─────────────────────────────────────────────┘

│

BMP280 Sensor (0x76)

Key takeaway: This tutorial shows how to integrate a vendor-provided sensor API with the Linux driver framework. The bridge functions (i2c_reg_read and i2c_reg_write) are the critical link — they translate between Bosch's API callback interface and Linux's i2c_master_send/recv calls. The pattern is reusable for any sensor that provides a C API with pluggable read/write callbacks.

https://www.bosch-sensortec.com/media/boschsensortec/downloads/datasheets/bst-bmp280-ds001.pdf

# Linux Johannes 4 gnu

# Tmux Commands — Most to Least Used

## Sessions

tmux                          start new session

tmux new -s name              start named session

tmux ls                       list sessions

tmux attach -t name           attach to session

tmux kill-session -t name     kill session

Ctrl+b d                      detach from session

Ctrl+b $                      rename session

Ctrl+b s                      switch between sessions

## Windows (Tabs)

Ctrl+b c          new window

Ctrl+b n          next window

Ctrl+b p          previous window

Ctrl+b number     go to window N (0-9)

Ctrl+b w          list all windows

Ctrl+b ,          rename window

Ctrl+b &          close window

## Panes (Splits)

Ctrl+b %          split vertical  (left|right)

Ctrl+b "          split horizontal (top/bottom)

Ctrl+b arrow      move between panes

Ctrl+b z          zoom/unzoom pane (fullscreen toggle)

Ctrl+b x          close pane

Ctrl+b q          show pane numbers → press number to jump

Ctrl+b {          move pane left

Ctrl+b }          move pane right

Ctrl+b spacebar   cycle through pane layouts

## Resize Panes

Ctrl+b :resize-pane -D 5     resize down 5

Ctrl+b :resize-pane -U 5     resize up 5

Ctrl+b :resize-pane -L 5     resize left 5

Ctrl+b :resize-pane -R 5     resize right 5

## Copy Mode (Scroll)

Ctrl+b [          enter copy mode (scroll with arrows/PageUp)

q                 exit copy mode

Ctrl+b PgUp       enter copy mode + scroll up

Space             start selection (in copy mode)

Enter             copy selection

Ctrl+b ]          paste

## Misc

Ctrl+b ?          list all keybindings

Ctrl+b :          enter command mode

Ctrl+b t          show clock

Ctrl+b ~          show messages

## Quick Reference Card

PREFIX = Ctrl+b   (must press before every command)

Most used daily:

new session   →  tmux new -s work

attach        →  tmux attach -t work

split vert    →  Ctrl+b %

split horiz   →  Ctrl+b "

move panes    →  Ctrl+b ←↑↓→

zoom pane     →  Ctrl+b z

new window    →  Ctrl+b c

switch window →  Ctrl+b n / p

detach        →  Ctrl+b d

Copy folder from VM(multipass) → Local

ezhil@ubuntu:~/Desktop/ldd$ multipass transfer -r primary:/home/ubuntu/ldd ~/Desktop/ldd

# Embedded linux driver developement Book notes

### Notes: https://github.com/dryruner/LDD3/tree/master

### example code: https://github.com/martinezjavier/ldd3

## Chapter 1: An Introduction to Device Drivers

#### The Role of the Device Driver

- Mechanism vs. Policy: Drivers provide mechanism (hardware access) without imposing policy (how to use it). This separation allows flexibility; users/applications decide usage.

- Driver as Black Box: Hides hardware details behind standardized interfaces (e.g., open, read, write). Enables modular, runtime loading.

- Flexibility Trade-offs: Balance hardware capabilities, concurrency, and simplicity. Avoid unnecessary policies (e.g., don't restrict access unless needed for security).

- Characteristics of Policy-Free Drivers: Support sync/async ops, multiple opens, full hardware features; no simplifying layers. Often bundled with user tools/libraries for config/policy.

- Why Write Drivers?: Access new hardware, expand markets for vendors, contribute to open source.

#### Splitting the Kernel

- Kernel Responsibilities (see Figure 1-1):

  - Process Management: Create/destroy processes, IPC (signals, pipes), scheduling (multitasking on few CPUs).

  - Memory Management: Virtual addressing, allocation (malloc/free-like calls).

  - Filesystems: VFS abstraction over hardware; supports multiple types (e.g., ext3, FAT).

  - Device Control: Maps ops to hardware via drivers (e.g., disks, keyboards).

  - Networking: Packet handling, routing, address resolution.

- Modularity: Kernel extensible via loadable modules (added/removed at runtime with insmod/rmmod).

#### Classes of Devices and Modules

- Device Types (orthogonal to other classifications like USB/SCSI):

  - Char Devices: Stream of bytes (e.g., /dev/ttyS0 console/serial ports). Sequential access; supports open/close/read/write. Can seek/mmap if like data area (e.g., frame grabbers).

  - Block Devices: Hosts filesystems (e.g., disks). Whole-block I/O (512B+); Linux allows byte-level reads/writes. Differs in kernel interface from char.

  - Network Interfaces: Packet-oriented (e.g., eth0). No /dev node; kernel-driven tx/rx. Handles connections without knowing streams (e.g., TCP).

- Modules: Object code dynamically linked; classes include drivers, filesystems (e.g., ext3 maps inodes/dirs/files). Decompose into small modules for scalability.

- Other Classes: USB/SCSI/FireWire/I2O—kernel exports common functions to avoid duplication.

#### Security Issues

- Kernel Enforces Security: Holes in kernel = system-wide holes. init_module restricted to privileged users (CAP_SYS_MODULE).

- Driver Best Practices:

  - Avoid policy in drivers; handle at higher levels (admin control).

  - Restrict dangerous ops (e.g., interrupt setup, firmware load, disk format) to root.

  - Prevent bugs: No buffer overruns; zero uninitialized memory; validate user input; avoid info leaks.

  - Treat user data suspiciously; limit ops that affect globals/other users.

- General Tips: Compile kernel without modules if paranoid; disable post-boot loading. Avoid untrusted precompiled kernels (backdoors possible).

#### Version Numbering

- Kernel Versions: Even (e.g., 2.6.x) = stable; odd (2.7.x) = development (ephemeral, no support).

- Book Coverage: Focuses on 2.6.10 (stable); no backward compat. API changes possible—check Documentation/Changes, lwn.net/Articles/2.6-kernel-api.

- Platform Independence: Code tested on x86/x86-64; works on 20+ arches. Hardware-specific samples noted.

- Tools: Use recent distro; kernel sources list exact reqs.

#### License Terms

- GPL v2: Allows redistribution/sale if source provided; derivatives must be GPL. In COPYING file.

- Modules: Binary distribution tolerated if following kernel ABI, but risky (copyrights vary; interfaces break). Prefer GPL for mainline inclusion/patches.

- Book Code: BSD/GPL dual-licensed; free at ftp.ora.com/pub/examples/linux/drivers (LICENSE file).

#### Joining the Kernel Development Community

- linux-kernel List: Central hub (200+ msgs/day); Linus et al. subscribe. Essential for devs/help.

- Tips: Read FAQ (tux.org/lkml); show homework done. Great for testers/ideas/critique.

#### Overview of the Book

- Structure: Ch. 2: Modules. Ch. 3: Char drivers (scull example). Builds gradually; software-first, then hardware.

- Part 1 (Ch. 1–11): Char drivers, concurrency, memory, interrupts, PCI/USB, etc. Testable without hardware (except minor parallel port wire).

- Part 2 (Ch. 12–18): Block/network drivers, device model, DMA, TTY.

- Approach: Device-independent; sample code in text/sources. Kernel internals overview.

===========================================================================

## Chapter 2: Building and Running Modules

#### Setting Up Your Test System

- Kernel Source Requirement: For 2.6.x modules, need full configured/built kernel tree (not just headers). Download from kernel.org or distro; build/install kernel. Run target kernel when building modules for ease (not required).

- Testing Safety: Use "sacrificial" system without critical data/services. Kernel bugs can crash system but rarely corrupt disk.

- Examples: Available at O'Reilly FTP (ftp.ora.com/pub/examples/linux/drivers). Build with make; load/unload as root.

#### The Hello World Module

- Basic Structure: Simple module with init/exit functions; uses module_init/module_exit macros to register them. MODULE_LICENSE("Dual BSD/GPL") declares free license (avoids taint/warnings).

- Code Example:

- #include <linux/init.h>  // For __init, __exit

- #include <linux/module.h>  // Core module headers

- MODULE_LICENSE("Dual BSD/GPL");

- static int __init hello_init(void) {

- printk(KERN_ALERT "Hello, world\n");

- return 0;  // Success

- }

- static void __exit hello_exit(void) {

- printk(KERN_ALERT "Goodbye, cruel world\n");

- }

- module_init(hello_init);

- module_exit(hello_exit);

- Build/Load/Test:

  - Compile: make (uses kernel tree's Makefile).

  - Load: insmod hello.ko (as root; prints to kernel log).

  - Unload: rmmod hello (prints exit message).

- printk: Kernel's printf equivalent; KERN_ALERT for high priority (logs to /var/log/messages or dmesg). No floating-point support.

#### Kernel Modules Versus Applications

- Execution Model: Modules register for future calls (event-driven); init() prepares, exit() cleans up. Unlike apps, no sequential run—must undo all init work to avoid leaks.

- Linking: No user libs (e.g., no <stdio.h>); only kernel exports (e.g., printk). Headers from kernel tree (include/linux, include/asm).

- Fault Handling: Kernel faults kill process/system (no safe segfaults); debug carefully (Ch. 4).

- Spaces: Kernel space (privileged, direct HW access) vs. user space (restricted). Syscalls/interrupts switch modes; modules run in kernel space.

- Concurrency: Always assume multi-tasking/SMP/preemption. Code must be reentrant; protect shared data (Ch. 5). Sources: processes, interrupts, timers, SMP.

- Current Process: Access via current (struct task_struct *; <linux/sched.h>). Points to invoking process during syscalls.

#### Compiling and Loading

- Build Process: Use kernel's build system. Create Makefile with obj-m += module.o; run make -C /path/to/kernel M=$PWD from module dir. Outputs .ko (compiled module).

- Kernel Config: Enable CONFIG_MODVERSIONS=y for symbol versioning (ABI checks). Modules must match running kernel version/arch.

- Loading: insmod links .ko to kernel (resolves symbols). modprobe auto-loads dependencies (uses /lib/modules/uname -r/modules.dep).

- Version Magic: Embedded string in .ko/module verifies compatibility (e.g., "2.6.10 SMP mod_unload...").

- Tainted Kernel: Loading proprietary modules taints (tracked via /proc/sys/kernel/tainted); affects bug reports.

#### The Kernel Symbol Table

- Exports: Kernel functions/variables accessible via EXPORT_SYMBOL(name) (e.g., printk). Listed in /proc/kallsyms (or System.map).

- Module Exports: Modules export via EXPORT_SYMBOL for other modules to use.

- Resolution: insmod resolves symbols at load (dynamic linking). Use kallsyms for debugging.

- Versioning: EXPORT_SYMBOL_GPL restricts to GPL modules. Module versions ensure ABI match.

#### Preliminaries

- Licensing: Use MODULE_LICENSE (e.g., "GPL") to avoid warnings/taint. Supports GPL/BSD variants.

- Author/Description: MODULE_AUTHOR/DESCRIPTION for metadata (visible in modinfo).

- Error Handling: Init returns non-zero on failure (unload attempted). Use ERR_PTR for pointer errors.

#### Initialization and Shutdown

- Init Function: Marked __init (discarded after load to save memory). Runs on insmod; returns 0 on success.

- Exit Function: Marked __exit (only used if unloadable). Runs on rmmod; must free all resources (e.g., unregister devices).

- Cleanup: Use list_del for lists, kfree for memory. Avoid if !CONFIG_MODULE (built-in).

- Unload Safety: Kernel checks try_module_get refs; refuse unload if in use. Use module_put in exit.

#### Module Parameters

- Declaring: Use module_param(name, type, perm) (e.g., module_param(myint, int, 0);). Types: bool, charp, int, etc. Perm: 000 (no /sys), 0644 (read/write).

- Access: Params in /sys/module/modname/parameters/. User-set via insmod (e.g., insmod mod.ko myint=5).

- Arrays: module_param_array(name, type, num, perm).

- Read-Only: Use module_param_ro for kernel-set values.

- Usage: Init reads/changes params; useful for tuning without rebuild.

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/init.h>

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Example");

MODULE_DESCRIPTION("Simple Module Parameters Example");

/* Module parameters */

int my_int = 10;

char *my_string = "default";

bool my_bool = 0;

/* Declare parameters */

module_param(my_int, int, 0644);

MODULE_PARM_DESC(my_int, "An integer parameter");

module_param(my_string, charp, 0644);

MODULE_PARM_DESC(my_string, "A string parameter");

module_param(my_bool, bool, 0644);

MODULE_PARM_DESC(my_bool, "A boolean parameter");

/* Module init function */

static int __init param_init(void){

printk(KERN_INFO "Module Loaded\n");

printk(KERN_INFO "Integer value: %d\n", my_int);

printk(KERN_INFO "String value: %s\n", my_string);

printk(KERN_INFO "Boolean value: %d\n", my_bool);

return 0;

}

/* Module exit function */

static void __exit param_exit(void){

printk(KERN_INFO "Module Removed\n");

}

module_init(param_init);

module_exit(param_exit);

//sudo insmod param_example.ko my_int=25 my_string="LDD3" my_bool=1

#### Doing It in User Space

- User-Space Alternatives: For testing, use FUSE (Filesystem in Userspace) or UML (User-Mode Linux) to simulate kernel without risks. Avoids full kernel recompiles.

- Limitations: Not all kernel APIs available; good for prototyping char/block FS.

- Tools: uml_net for networking; good for driver dev without HW.

#### Quick Reference

- Build: make -C $KDIR M=$PWD modules.

- Load/Unload: insmod/rmmod/modprobe.

- Info: modinfo mod.ko; lsmod lists loaded.

- Symbols: /proc/kallsyms; EXPORT_SYMBOL.

- Params: module_param; access via sysfs.

- Concurrency: Always protect shared data (mutexes, etc.; Ch. 5).

- Best Practice: Keep init/exit symmetric; zero-init structs; check return codes.

===========================================================================

## Chapter 3: Char Drivers

#### Major and Minor Numbers

- Device Files: Represented as /dev/<name>; ls -l shows major (driver ID) and minor (device instance) numbers, e.g., crw-rw-rw- 1 root root 1, 3 Apr 11 2002 null (major=1, minor=3).

- Major Role: Identifies driver; traditionally one-per-driver.

- Minor Role: Driver uses to distinguish instances (e.g., index into array); kernel ignores beyond allocation.

- dev_t Type: (defined in <linux/types.h>) is used to hold device number - both the major and minor parts.32-bit (12-bit major, 20-bit minor); use macros: MKDEV(major, minor)   // create device number
MAJOR(dev)            // get major number
MINOR(dev)            // get minor number

- In morden linux kernes range of major and minor numbers

| Type | Range |
| --- | --- |
| Major Number | 0 – 4095 (12 bits) |
| Minor Number | 0 – 1048575 (20 bits) |

Example:
	dev_t dev;

dev = MKDEV(240, 0);

printk("Major: %d\n", MAJOR(dev));

printk("Minor: %d\n", MINOR(dev));

#### Allocating and Freeing Device Numbers

- Static Allocation: use if major number is known (returns 0 on success, else -ve error). 
int register_chrdev_region(dev_t first, unsigned int count, char *name);

- Dynamic Allocation: kernel assigns major (firstminor usually 0); read from /proc/devices. 
int alloc_chrdev_region(dev_t *dev, unsigned int firstminor, unsigned int count, char *name);

- Free: void unregister_chrdev_region(dev_t first, unsigned int count); //in module exit.

- Script Example (scull_load): Loads module, parses /proc/devices for major, creates /dev/scull[0-3] with mknod, sets perms/group (e.g., staff/wheel).

- Best Practice: Default to dynamic (SCULL_MAJOR=0); override via macro, module param (scull_major=NN), or script.

#### Some Important Data Structures

- file_operations (fops): struct file_operations in Linux Kernel (defined in fs.h) is a table of function pointers used by the Virtual File System (VFS) to call the correct operations of a file or device driver.

file_operations is a structure that contains pointers to functions that perform 	operations on a file.file_operations is basically a table of function pointers that 	implements the file’s behavior

struct file_operations {

struct module *owner;

fop_flags_t fop_flags;

loff_t (*llseek) (struct file *, loff_t, int);

ssize_t (*read) (struct file *, char __user *, size_t, loff_t *);

ssize_t (*write) (struct file *, const char __user *, size_t, loff_t *);

ssize_t (*read_iter) (struct kiocb *, struct iov_iter *);

ssize_t (*write_iter) (struct kiocb *, struct iov_iter *);

int (*iopoll)(struct kiocb *kiocb, struct io_comp_batch *, unsigned int flags);

int (*iterate_shared) (struct file *, struct dir_context *);

__poll_t (*poll) (struct file *, struct poll_table_struct *);

long (*unlocked_ioctl) (struct file *, unsigned int, unsigned long);

long (*compat_ioctl) (struct file *, unsigned int, unsigned long);

int (*mmap) (struct file *, struct vm_area_struct *);

int (*open) (struct inode *, struct file *);

int (*flush) (struct file *, fl_owner_t id);

int (*release) (struct inode *, struct file *);

int (*fsync) (struct file *, loff_t, loff_t, int datasync);

int (*fasync) (int, struct file *, int);

int (*lock) (struct file *, int, struct file_lock *);

unsigned long (*get_unmapped_area)(struct file *, unsigned long, unsigned long, unsigned long, unsigned long);

int (*check_flags)(int);

int (*flock) (struct file *, int, struct file_lock *);

ssize_t (*splice_write)(struct pipe_inode_info *, struct file *, loff_t *, size_t, unsigned int);

ssize_t (*splice_read)(struct file *, loff_t *, struct pipe_inode_info *, size_t, unsigned int);

void (*splice_eof)(struct file *file);

int (*setlease)(struct file *, int, struct file_lease **, void **);

long (*fallocate)(struct file *file, int mode, loff_t offset,

loff_t len);

void (*show_fdinfo)(struct seq_file *m, struct file *f);

#ifndef CONFIG_MMU

unsigned (*mmap_capabilities)(struct file *);

#endif

ssize_t (*copy_file_range)(struct file *, loff_t, struct file *,

loff_t, size_t, unsigned int);

loff_t (*remap_file_range)(struct file *file_in, loff_t pos_in,

struct file *file_out, loff_t pos_out,

loff_t len, unsigned int remap_flags);

int (*fadvise)(struct file *, loff_t, loff_t, int);

int (*uring_cmd)(struct io_uring_cmd *ioucmd, unsigned int issue_flags);

int (*uring_cmd_iopoll)(struct io_uring_cmd *, struct io_comp_batch *,

unsigned int poll_flags);

int (*mmap_prepare)(struct vm_area_desc *);

} __randomize_layout;

Example:
static struct file_operations fops = {

.owner = THIS_MODULE,

.open = my_open,

.read = my_read,

.write = my_write,

.release = my_close,

};

- file Struct:  The file structure represents an open file instance in the kernel. Whenever a process opens a file using open(), the kernel creates a file object.

- Purpose: It keeps track of the state of an opened file.

struct file {

struct inode *f_inode;

struct file_operations *f_op;

loff_t f_pos;

unsigned int f_flags;

};

- inode Struct: An inode (index node) represents a file in the filesystem. It stores metadata about the file, but not the file name.

- Purpose: It contains all the information about the file itself.

struct inode {

umode_t i_mode;

unsigned long i_ino;

struct file_operations *i_fop;

};

#### Char Device Registration

- cdev Struct: Kernel's char device repr (include <linux/cdev.h>).

- Alloc/Init: struct cdev *cdev = cdev_alloc(); or embed in own struct + cdev_init(&dev->cdev, &scull_fops);; set .owner = THIS_MODULE.

- Add: cdev_add(&dev->cdev, devno, count) (count=1 usually; fails → -ve; device "live" post-call).

- Del: cdev_del(&dev->cdev) in cleanup (don't access after).

- scull Example: Embed in struct scull_dev { ... struct cdev cdev; };; init/add in scull_setup_cdev(dev, index) with MKDEV.

- Old Way (pre-2.6, avoid): register_chrdev(major, "name", &fops) (allocs 0-255 minors); unregister_chrdev(major, "name").

#### open and release

- open: Init (check errors, init device, alloc private_data, update f_op). Prototype: int open(struct inode *inode, struct file *filp);.

  - ID device: container_of(inode->i_cdev, struct scull_dev, cdev) → store in filp->private_data.

  - Or use iminor(inode) if register_chrdev.

  - scull: Trims to 0 on O_WRONLY; returns 0.

- release: Cleanup (free private_data, shutdown on last close). Prototype: int release(struct inode *inode, struct file *filp);.

  - Invoked only when usage count →0 (not per close; fork/dup increments count).

  - scull: Empty (no alloc/free needed).

- flush: Per-close (not release); rare (e.g., SCSI tape); NULL ignores.

#### scull’s Memory Usage

- Design: Dynamic RAM buffer (grows on write, trims on shorter write); no size limit (eat RAM for testing: cp /dev/zero /dev/scull0).

- kmalloc/kfree: <linux/slab.h>; alloc/free contiguous slabs (GFP_KERNEL flags); efficient for small/large, but not huge (Ch. 8).

- Layout: Linked list of scull_qset (array of 1000 ptrs to 4KB quanta; ~4MB per set); overhead ~4MB/4MB data.

  - Config: Macros (SCULL_QUANTUM=4KB, SCULL_QSET=1000), module params (scull_quantum, scull_qset), ioctl.

  - struct scull_dev { ... void **data; int quantum, qset, size; ... }; (data → first qset).

  - struct scull_qset { void **data; struct scull_qset *next; };.

- scull_trim(dev): Frees all (walk list, kfree quanta/sets); resets size/quantum/qset; called on write-open & module exit.

- scull_follow(dev, item): Gets/allocs qset at index (creates chain as needed).

#### read and write

- Prototypes: ssize_t read(struct file *filp, char __user *buf, size_t count, loff_t *f_pos); (similar for write; update *f_pos).

  - Return: Bytes transferred (>=0), 0=EOF/nothing, -ve=error (e.g., -EFAULT bad addr, -EINTR interrupt).

  - Partial OK (app retries); 0 on write=non-error (retry for blocking).

- User Pointers: Use copy_to_user(to, from, count)/copy_from_user(to, from, count) (<asm/uaccess.h>); returns uncopied bytes (0=success, else -EFAULT).

  - Safe: Handles paging/faults/sleep; reentrant (Ch. 5); avoid direct deref (security/oops).

- scull_read: Single quantum; calc item/s_pos/q_pos from *f_pos; scull_follow → copy_to_user; trim count if EOF/quantum end.

- scull_write: Single quantum; alloc if needed (GFP_KERNEL); copy_from_user; update size if grown.

- pread/pwrite: Ignore *f_pos changes (user-supplied pos).

- readv/writev: Vector I/O (scatter/gather); proto uses struct iovec { void __user *iov_base; __kernel_size_t iov_len; } iov[], unsigned long count.

  - NULL → kernel emulates via r/w; implement for efficiency (e.g., tape record).

#### Playing with the New Devices

- Test: Compile/load; use cp, dd, redirection (ls > /dev/scull0); free watches RAM; strace sees quantized calls.

- Debug: printk in r/w; trace syscalls/returns.

#### Quick Reference

- Headers: <linux/fs.h>, <linux/cdev.h>, <linux/kernel.h>, <asm/uaccess.h>, <linux/slab.h>.

- Macros/Funcs: dev_t/MAJOR/MINOR/MKDEV; register/alloc/unregister_chrdev_region; cdev_alloc/init/add/del; container_of; copy_to/from_user; kmalloc/kfree.

===========================================================================

LDD roadmap: https://www.linkedin.com/posts/pradeeptewani_embeddedlinux-linuxdrivers-devicedrivers-activity-7392778708253622272-iy-x/

https://embitude.in/roadmap-to-master-embedded-linux-device-drivers/